3GPP TSG-RAN4 Meeting #113	R4-2419858

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0698 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UE Capability for HST FR2 with multi-Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no UE Capability “simultaneousReceptionFR2HST-r18” in 38.306. The appropriate UE Capability for HST FR2 with multi-Rx as in 38.306 should be “simultaneousReceptionTwoQCL-r18” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replacing simultaneousReceptionFR2HST-r18 in Table 7.1.1.3-1 with simultaneousReceptionTwoQCL-r18 and removing the corresponding square brackets |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No appropriate UE capability for HST FR2 with multi-Rx |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a revision of R4-2419344 by adding the missing “Source to TSG” in the cover page |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2419344 |  |  |  |  |  |  |  |  |

# 7	Demodulation performance requirements (Radiated requirements)

## 7.1	General

### 7.1.1	Applicability of requirements

#### 7.1.1.1	General

The minimum performance requirements are applicable to the FR2 operating bands defined in TS 38.101-2 [7] . The minimum requirements for FR2-1 is applicable for FDL_high not exceeding 48200 MHz. Additional applicability rules for certain operating bands are specified in Clause 7.1.1.6 and 7.1.1.8.

The minimum performance requirements in Clause 7 are mandatary for UE supporting NR operation, except test cases listed in Clause 7.1.1.3, 7.1.1.4, 7.1.1.5, 7.1.1.7.

If same test is listed for different UE features/capabilities in Clauses 7.1.1.3 and 7.1.1.4, then this test shall apply for UEs which support all corresponding UE features/capabilities.

#### 7.1.1.2	Applicability of requirements for different number of RX antenna ports

UE shall support 2 RX ports for different RF operating bands. The UE requirements applicability is defined in Table 7.1.1.2-1.

Table 7.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports 2RX antenna ports | PDSCH | All tests in Clause 7.2.2 All tests in Clause 7.6.2 |
|  | PDCCH | All tests in Clause 7.3.2 |
|  | PBCH | All tests in Clause 7.4.2 |

#### 7.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 7.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 7.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 3-1) |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 7.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.2 |  |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR2 TDD | PDSCH | Clause 7.2.2.2.2 |  |  |
| DRX Adaptation (drx-Adaptation-r16) | FR2-1 TDD | PDCCH | Clause 7.3.2.2.3 | If the Test 3-1 in Clause 7.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 7.3.2.2.1. |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 1-4) |  |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | SDR | Clause 7.5A.1 | For UE capable of pdsch-256QAM-FR2 for certain band(s), mcs-Table is configured to ‘64QAM’ for SDR test. |  |
| Support of FR2 HST operation [(FR2 UE power class PC6 signalling is used to indicate support of feature group)] | FR2-1 TDD | PDSCH | [Clause 7.2.2.2.4] |  |  |
| Support of Single Carrier operations with 120kHz SCS for FR2-2 (initialAccessSSB-120kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-1, 4-2, 4-3, 4-4) |  |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-1, 1a-2, 1a-3) (Table 7.3.2.2.2-2, Test 3-1, 3-2) |  |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 3) |  |  |
| Support of 480kHz SCS for FR2-2 (ul-FR2-2-SCS-480kHz-r17 and initialAccessSSB-480kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-5, 4-6) |  |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-4) (Table 7.3.2.2.2-2, Test 3-3) |  |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 4) |  |  |
| Support simultaneous reception with different QCL Type-D RSs (simultaneousReceptionDiffTypeD-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.5 Clause 7.2.2.2.6 Clause 7.2.2.2.7 |  |  |
| Single DCI based SDM transmission for simultaneous reception support (singleDCI-SDM-scheme-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.7 |  |  |
| Multi DCI based simultaneous reception non-overlapping support (multiDCI-MultiTRP-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.5 |  |  |
| Multi DCI based simultaneous reception fully-overlapping support (overlapPDSCHsFullyFreqTime-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.6 |  |  |
| Support of 2-port DL PTRS (supportTwoPortDL-PTRS-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.7 Test 1-2 |  |  |
| Support of FR2 HST operation (FR2 UE power class PC6 signalling is used to indicate support of feature group) with simultaneous multiRX reception | FR2-1 TDD | PDSCH | Clause 7.2.2.2.8 | FR2 HST UE should support the following optional capabilities - simultaneousReceptionTwoQCL-r18; - multiDCI-MultiTRP-r16; - overlapPDSCHsFullyFreqTime-r16; Additionally, the UE should report maxNumberActiveTCI-PerBWP > 1 |  |