3GPP TSG-RAN WG4 Meeting #113	R4-2419897

Orlando, United States, November 18 – 22, 2024

Title: 	Way Forward for [113][310] NR_NTN_Ph3_UE_RF

Agenda Item:	7.26.5

Source: 	Qualcomm Inc.

Document for:	Approval

## Issue 1-1: PC3 output power for (e)RedCap

Agreement:

- FFS whether and how a potential new scheme to increase output power is addressed in specifications

  - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap

    - If confirmed, no further work is needed.

  - Alt 2: Power increase by network controlled optional new signalling (e.g capability)

  - Alt 3: Change in power tolerances

    - E.g. Upper tolerance exception to UE in specific regions via table note.

  - FFS whether solution is common for HD-FDD and FD-FDD

  - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB.

## Issue 1-2: HD-FDD refsens for 2 Rx

Agreement: Further discussion on potential improvement beyond TN HD-FDD 2 RX REFSENS level is limited to maximum 0.2 dB improvement based on average of inputs to RAN4.

## Issue 1-3: Simultaneous operation with GNSS

Agreement:

Simultaneous GNSS and NR-NTN operation for (e)Redcap cannot be guaranteed in some occasions for some NTN bands for some specific UE implementations.

Send LS to RAN2 requesting common IDC solution for (e)RedCap NR NTN and non-RedCap NR NTN which further improves the existing IDC framework to be more suitable for NR NTN – GNSS co-existence. Final solution is up to RAN2.

## Issue 2-1: UL capacity enhancements

Agreement: No RF impact on inter-slot OCC over 2 slots.