3GPP TSG-RAN WG4 Meeting #113											     R4-2420209

Orlando, US, 18 ‒ 22 November, 2024

Title: 	Simulation assumptions for CJT calibration

Agenda Item:	7.19.4

Source: 	MediaTek inc.

Document for:	Approval

# Introduction

In this meeting (RAN4#113), RAN4 has discussed the simulation assumptions for the delay and frequency offset reporting of CJT calibration for Rel-19 NR MIMO Phase 5 (Part2) and reached the agreement captured in section 2.

# UE reporting enhancement for CJT calibration

Issue 2-1: On the simulation assumption for CJT calibration

Agreement:

- Agree with the following table for delay/frequency offset reporting. FFS on phase offset.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

# References

- R4-2416921, “WF on RRM requirements for NR_MIMO_Ph5 (Part2)”, MediaTek inc.