3GPP TSG-RAN WG4 Meeting #113		 R4-2420335

Orlando, USA, 18th – 22nd November, 2024

Agenda item:	7.17.6

Source: 	Qualcomm Incorporated

Title: 	AI/ML ad-hoc meeting minutes

Document for:	Approval

# Introduction

The discussion on the AI/ML study is organized under two threads, [129] and [130] in RAN4#113. The ad-hoc meeting will discuss some of the topics from the moderator summary in [1], “.

# Discussion

## Beam Prediction

### Sub-topic 3-1

Relative RSRP accuracy

Issue 3-1: Relative RSRP Accuracy

- Proposals

  - Option 1: The definition of relative RSRP accuracy shall be:

The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i -  the ground truth of the L1-RSRP of the Top-1 ideal beam index j

  - Option 2: The definition of relative RSRP accuracy shall be:

predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams.

  - Option 3: The definition of relative RSRP accuracy shall be:

the RSRP difference between the predicted L1-RSRP of different beams.

  - Option 4: others

- Recommended WF

  - Option 1

Discussion:

Samsung: is this accuracy relative to ground truth? Is it ground truth relative to ground truth?

QC: this is respective to beam ID. This is a useful metric to define in the future. The others do not have any notion of the ground truth.

Samsung: this relative is not important, might not be needed.

vivo: we already have absolute RSRP accuracy. For relative accuracy, why do we need this if they are coming from different directions?

NTU: relative best beam RSRP accuracy, best beam is not a peticular index. Ground truth is the one listed here. Prediction is the index.

Oppo: we share similar view with QC. The definition is most important. The concern seems to be about the name of the requirement. Can we agree the definition and discuss the naming afterwards.

E///: the relative RSRP accuracy is related to the absolute. Why do we still need this one? Beam ID or accuracy can be discussed as a new issue.

Xiaomi: this is confusing whether this is about prediction or the relative beam index accuracy. In option 1 this is just about the index.

QC: beam prediction accuracy in terms of relative RSRP accuracy?

Apple:similar comments, what is the physical meaning? If we focus on the beam index, if it is within the top K, UE would pass. Another alternative is to look at the absolute RSRP accuracy. If it is accurate enough, it would be enough. We cannot yet agree on either of these options.

Nokia: this beam prediction accuracy in relative terms, this was option 2 in the last agreement. The option discussed here is option 3. We shouldn’t call this beam prediction accuracy. Option 1 should be modified , we would prefer to use the measured RSRP in the definition instead of ground truth.

Apple: for the ground truth, the best beam and 2nd best beam have a huge gap, based on any of thse definition UE would fail. UE cannot meet the first requirement. If the gap is large, option 1 doesn’t

Xiaomi: for Option 1 we should use the top 1 but sometimes the difference is small between top-1 and top-2

Samsung:we need further clarification on the intention of this discussion. Why do we need a relative one now? This is similar to absolute and relative definitions for legacy. If we are discussing something like that, this entire discussion needs to be revised.

CAICT: defintioin of relative accuracy is related to monitoring, this is also discussed in RAN1

R&S: with the prediction accuracy do we maintain a legacy approach or is this something new.

Apple: just to clarify. We have both absolute and relative is because for the relative we can remove the bias. When we look at this, we wouldn’t have a similar situation. It’s no difference here.

### Sub-topic 3-2

UE Rx beam known vs. unknown

In the previous meeting it was discussed whether the UE Rx beam to be used for receiving the predicted beam is known or not.

Issue 3-2: UE Rx beam knowledge

Proposals

  - Option 1: UE Rx beam is known

    - the UE Rx beam gain has to be taken into account when making the prediction

  - Option 2: UE Rx beam is unknown

    - Will the UE Rx spherical coverage be taken into account when making the prediction?

    - L1-RSRP input used for training and as input for inference takes into account the Rx beam gain (RSRP is measured at UE baseband)?

  - Option 3: depends on some conditions

    - which conditions?

  - Option 4: depends on UE capability

    - How to define the capability and how will the network use the capability?

  - Option 5: others

- Recommended WF

  - Option 1

Discussion:

MTK: the UE will take the Rx spherical coverage into account but the UE Rx beam index is not input into the prediction model. The Rx spherical coverage will be reflected in the RSRP of each Tx beam.

Samsung: if you do not consider the Rx beam index how can you make the prediction? This depends on 2 mehtods used in RAN1, one used to predict the tx beam RSRP as input. The other is to use the beam pair as input.

Apple: this is up to UE. Technically it would be helpful but we want to specify a minimum requirement, we shouldn’t assume a more advanced model. We support option 2, we shouldn’t assume the Rx beam is unknown.

QC: known or unknown will impact RAN4, it will impact the TCI state switch to the predicted beam. Even in RAN1, RAN1 assumes that the UE has the knowledge of the Rx beam. How can you predict the L1-RSRP is unknown?

Nokia: if we assume that the UE does not know the Rx beam, it would be an unknown TCI state change, it will increase the delay. If we go with option 2, we would fall back to the

Apple: Option 2 doesn’t mean that the UE doesn’t know but the model doesn’t take that into account. Anything beyond that is up to UE. Simplest assumption is UE Rx beam is unknown.

MTK: if it is beam in set B, it would be known, if it is in set A it would be unknown.

R&S: are we talking about the UE Rx beam ID?

Moderator: yes, this is about whether the UE knows which Rx beam to use to receive the predicted Tx beam or not.

Xiaomi: We support Option 4, we should have a capability.

MTK: we ask what is the difference between Tx beam prediction and Tx -Rx beam pair prediction. Requirements are related to whether the TCI beam is known or unknown.

Apple: we shouldn’t preclude the UE from further beam sweeping or not. We need to discuss further.

Nokia: this is nothing new

NTU: when you train your UE is associated with UE Rx beams.this is different compared to legacy definition of known or unknown.

Nokia: for a tradeoff between latency and robustness. What robustness we are getting here. We have some prediction here. We might have worse than legacy measurement and accuracy.

Agreement:

Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS

### Sub-topic 3-3

TCI State known vs. unknown

In the previous meeting it was discussed whether the TCI state associated with a predicted beam should be considered known or unknown.

Issue 3-3: TCI State knowledge

- Proposals

  - Option 1: Reuse the legacy conditions with the following additions (marked in red)

The TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state or AI/ML based inference to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or the RS resources for AI/ML based inference is the RS in SetB or in target TCI state or QCLed to the target TCI state.

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement or inference

-	The UE has sent at least 1 L1-RSRP report for the target TCI state or completes AI/ML based inference before the TCI state switch command

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the TCI state is unknown.

  - Option 2: TCI state QCL to an RS that is not in Set B is unknown even if this RS was included in the L1-RSRP report by UE within 1280ms before TCI state activation.

  - Option 3: The TCI state which is QCL to an RS in Set B is known:

    - if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction.

    - if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction.

  - Option 4: The known conditions of the predicted TCI state shall take below conditions into account:

    - The TCI state switch command is received during valid inference phase of AI/ML mode.

    - The predicted beam in Set A is the RS in target TCI state.

    - The predicted beam in Set A is detectable during training and inference phase, which implies the beam has been trained successfully and available in inference phase.

    - The UE knows the Rx beam for the predicted beam.

  - Option 5: RAN4 should consider relaxing known conditions for TCI states

    - TCI is known if measured within X+1280 ms.  Further analysis is required to define the exact value of X.

  - Option 6: others

- Recommended WF

  - To be discussed

Discussion:

QC: RAN1 agreed with a metric similar to what we have here. Predicition with and w/o margin. Due to some error uncertainty, we should wait for the sim results. We can agree to Option 3 now and then decide X later based on simulations.

Apple: we suggest to only consider only RSRP accuracy. If we want to pick the best beam out of 2, if they have a small gap, the UE might fail a top-1 prediction because of the small margin. Workload of options with the X margin is higher because we have to determine it.

Samsung: we do not agree to have Option 1, we should use option 2 and 3.

Tejas: we have Option 5 with some conditions. Consistency and accuracy should be maintained, we presented an approach in our paper.

Vivo: we support Option 6. RAN1 already agreed to use this accuracy.

Nokia: we tend to agree with Tejas that we should also consider Option 5. This guarantees some sort of consistencies. Option 5 is testing in different conditions whether the same prediction is expected. If in a certain set B we expect certain prediction, if we remove one input, the prediction should stay the same.

### Sub-topic 3-4

Beam prediction KPIs

Different metrics/KPIs have been discussed and were captured in the TR:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered

-	Option 1: RSRP accuracy

-	Option 2: Beam prediction accuracy

-Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

-Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

-Related measurement accuracy can be considered to determine x

-	Option 4: combinations of above options

Issue 3-4: Metrics/KPIs for beam prediction

Proposals

  - Option 1: Use Option 1

  - Option 2: Use Option 2

  - Option 3: Use combined Option 2 and 3:

    - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

      - FFS: K = 2 and x = 3 as baseline

  - Option 4: Add new metric:

      - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived.

  - Option 5: Add a requirement based on relative difference between predictions under different conditions/inputs (for example varying signal levels by N dBs but maintain same relative difference).

      - consider defining multiple tests under different conditions but with the same outcome(same predicted beam)

  - Option 6: both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction

      - RSRP absolute accuracy applies to all predicted beams

- Recommended WF

  - To be discussed

Companies suggesting to use different metrics(new) should come up with a concrete proposal

Discussion:

## CSI Compression

### Sub-topic 1-1

Option 3 simulation results for 2nd round

Issue 1-1: Simulation results discussion

- Proposals

  - Option 1: See simulation results in the table below.

- Recommended WF

Further discuss simulation results including the new data submitted to check alignment

### Sub-topic 1-2

Option 3 next step for 2-sided model

The next step for option 3 is to select one or more decoders from the ones submitted by companies such that other companies can try to train an encoder to work with it.

Issue 1-2: Option 3 next step - decoder(s) selection

- Proposals

  - Option 1: Select 3 decoders: with highest, lowest and closest to mean relative performance gain in SGCS over eTypeII decoder.

  - Option 2: Select 3 decoders ranodmly

  - Option 3: Select more than 3 decoders

    - Evenly distributed in performance?

  - Option 4: Select decoders after excluding any models for which the training dataset or SGCS CDFs appear to be outliers compared to others

  - Option 5: Select the decoder(s) from the encoder and decoder pair(s) which has the best SGCS performance on the aggregated dataset with only encoder input.

- Recommended WF

  - To be discussed

Nokia: The point of the study is to evaluate how the choice of test decoder impacts the possibility to formulate the requirements. We proposed previously to select the worst, average and best decoder. Another option to study is to not just select out of existing decoders but create a test decoder based on a mixed dataset. This may create resilience for the test decoder.

Moderator: Would this mean companies need to create decoders based on the mixed dataset and then again select ?

Nokia: Yes, but since the architecture and dataset is fixed it should not be much of an issue for misalignment.

Oppo: Share the view with Nokia. Suggest to use a mixed dataset first. If we choose just one or 3 models, we are choosing a dataset from one or 3 companies.

NTU: We are confused how we prove the feasibility for option 3. The question is whether we can converge to a decoder that is agreeable to all companies during the WI stage. The selection process is critical for feasibility. If we take worst, average and best decoders then in a WI stage we would need to decide which decoder to take. Mixed dataset if evaluated; what would be the metrics to evaluate. Should check robustness. Should check which decoder is best when compared to all of the encoders. Choose the one that has the best performance against the worst encoder.

Samsung: The mixed dataset method is interesting. The existing procedure we agree. If we see similar performance then alignment is achieved. It goes beyond what we agreed so far. Is the alignment we have satisfactory ? This could motivate the new approach.

Qualcomm: Samsung raise a good point. Last meeting we saw that the standard deviation was 0.2 and the improvement was 2.5%. This is a reasonable standard deviation to conclude that results are aligned. This should be sufficient to proceed to the next step.

Nokia: The problem with the alignment so far is that the training set is done on each individual dataset. The channel model will be implemented by TE vendor; it will not be what companies used to train their models. With a mixed dataset we can resolve the difference between results. Then we can compare the performance with different encoders.

Nokia: Need to clarify what is meant by “own” encoder. The companies should train the own encoder using only their own data.

Intel: For the selection of the decoder, we do not have a strong preference, we could select randomly. The question is then what we do next. We need to decide what is meant with datasets and own decoder. For own decoder, we can just follow the reference agreed for this analysis. Companies may want to check what happens if they do something else

Apple: When you say we pick a decoder, is is structure and parameters

Qualcomm: Options are not exclusive. We could do both options, then we can start now.

Samsung: Option 2 – not full convinced about the final step. Train encoder using own dataset. Could use the mixed dataset. In a real WI, if we train the encoder using the mixed dataset. From a UE vendor perspective, we could still use the mixed dataset.

Nokia: Why don’t you have the same concern about decoder based on a certain dataset… the UE vendor for that decoder gets an advantage.

Samsung: We practice here what would happen in a real WI. If we base a decoder on own dataset. With option 2, we would generate a mixed dataet.

Nokia: Option 1 is a single decoder; it is not company specific.

NTU: Training the decoder with a mixed dataset is a good idea. Option 2 wording agree. The encoder and decoder can be trained in any way, but for an end to end test the encoder input can only be own data. Mixed dataset is not available when you do end to end.

Vivo: The goal is to ensure that the UE can perform well in the field. It would be good if we could have a test that the UE trained with own dataset can pass the test. In that case, we need some mixed dataset. We don’t see much difference between option 1 and option 2. If we do not say how we select the decoders, we should take performance into consideration.

Huawei: For option 1, suggest to add “with aligned model structure”

Oppo: For option 1, should the selected decoder be uploaded now or after the meeting

Nokia: OK to follow both options

Vivo: How to select the 3 decoders.

Huawei: Prefer different encoder structure

Samsung: If the purpose is to enable RAN4 requirements, option 1a is more feasible. OR depends on whether designing own encoder

How to select 3 decoders:

- Option 1: Random

- Option 2: Highest SGCS

- Option3: Low, medium, high SGCS (Vivo, Nokia, Oppo)

- Option 4: Based on low, medium, high %tage improvement over etypeII (Qualcomm, Nokia)

Option 1a: Encoder has aligned model structure (

Option 1b: Encoder model structure may differ (Huawei)

Conclusion:

Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset)

- The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s).

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

### Sub-topic 1-3

Option 4a for 2-sided model

Option 4 was further split into 4 sub-options in the previous meeting in R4-2414323. The next steps on feasibility study for these options should be discussed

Issue 1-3: Option 4a(Dataset based) for 2-sided model

- Proposals

  - Option 1: Next steps:

      - Discuss/decide how to generate the training dataset and other assumptions

- need to align the ML model, what should the dataset contain (channel/eigenvector realization, etc), how to generate the dataset (aggregate dataset based on inputs from multiple companies, etc)

      - {encoder input, encoder output w/quantization}

      - {channel data, latent message}

      - Others?

- reuse the same assumptions/dataset from Option 3 study?

      - Check on performance alignment see simulation results from contributing companies

- repeat simulations until good alignment is achieved

- move to next step after alignment

- reuse Option 3?

      - Share models (encoder and/or decoder)/datasets (training/testing/inference) (sharing framework is currently under discussion)

        - Reuse Option 3?

      - Select one or more datasets for further analysis

- Selection criteria is TBD

      - Companies bring results for “own encoder and decoder” with selected dataset

- performance alignment to be checked (

      - Conclude Option 4a feasibility

    - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

  - Option 2:

    - Step 1-3: Reuse results of Option 3

    - Step 4: Select one or more dataset(s) for further analysis

      - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3.

    - Step 5: Company brings results for training of “own encoder(s) and decoder(s)” with selected dataset(s)

      - Performance alignment to be checked/discussed

    - Step 6: Conclude on overall feasibility of Option 4a

      - Feasibility criteria to be discussed

- Recommended WF

  - To be discussed

The steps should be discussed and if any changes and/or additions are needed

NTU: There is a difference between the dataset option 4 and 3. For option 3, the dataset contains encoder input and output. For option 4, the dataset contains the latent message. If 3 decoders are selected, one can be a priority. For the mixed dataset, it will be different to option 3. The mixed dataset will mix the latent message, which will be from different companies encoders

    - Step 1-3: Reuse results of Option 3

    - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3

      - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1

      - Or mixed dataset from track 2

    - Step 5: Label selected dataset with encoder input/output using encoder corresponding to the selected test decoder from option 3

    - Step 6: Company brings results for training of “own encoder(s) and decoder(s)” with selected dataset(s)

      - Performance alignment to be checked/discussed

    - Step 7: Conclude on overall feasibility of Option 4a

      - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

To be clarified: whether “own decoder” in step 5 is expected to have the agreed structure or could be different (in order to investigate whether the specification needs to capture a decoder structure for option 4a or not). This would apply to any 4b also.

Include also: Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset

# References

[1] R4-2418151, “Topic summary for [112-Bis][127] NR_AIML_air”, Moderator(Qualcomm Incorporated), RAN4#112-bis