3GPP TSG-RAN4 Meeting #113	                                                                 R4-2420338

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.10.5

Source:	CMCC

Title:	WF on NR ATG UE RF

Document for:	Approval

Introduction

This document captures the agreements on Rel-19 ATG UE RF requirements in RAN4#113.

Topic #1: R18 UE RF requirements maintenance

Issue 1-1: Tx requirements definition for two types of ATG UE antenna

Agreement during RAN4#112: - There are two types of antenna type for R18 ATG UE. For omni-direction antenna, the requirements should be measured at antenna connectors but for antenna array, the requirements should be measured at TAB connectors. - Following requirements are defined as the sum of all antenna connectors/TAB connectors - 6.3J.1 Minimum output power for ATG - 6.3J.4 Power control for ATG - 6.5J.2 Out of band emission for ATG - 6.5J.3 Spurious emissions for ATG - Following requirements are defined at each antenna connector/TAB connector - 6.3J.2 Transmit OFF power for ATG - 6.3J.3 Transmit ON/OFF time mask for ATG - FFS the following requirements. - 6.5J.1 Occupied bandwidth for ATG - 6.4J.1 Frequency error for ATG - EVM requirements

WF:

- Further check these two options:

- Option 1: The requirements for Frequency error, Error Vector Magnitude, Carrier leakage and EVM equalizer spectrum flatness are defined per layer.

- Option 2: All transmit signal quality requirements should be specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antennas(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array.

Issue 1-2: Rx requirements definition for two types of ATG UE antenna

Agreement:

- Rx antenna port requirements of the ATG UE need to be specified at the antenna connector(s) or TAB connectors.

- The power should be fed to each connector during the test

Issue 2-1: ATG UE antenna definition

WF:

- Further check the following definition for ATG UE antenna types.

- ATG UE with one or multiple passive  antenna(s) and ATG UE with the antenna array.

Topic #3: R19 UE RF for inter-band CA

Issue 3-1: Assumed parameters when supporting n39 UL for CA_n3A-n39A with common antenna.

WF:

- Further check these two sets of parameters

  - Option 1

| Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 50dB |
| n39 Tx filter rejection at 1805~1880MHz | 50dB |

  - Option 2

| Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 20dB |
| n39 Tx filter rejection at 1805~1880MHz | 40dB |

- Consider the assumed parameters with separate antenna.

Issue 3-3: MSD for CA_n3A-n39A with n39 UL

Agreement:

- Define the MSD value with 40dBm output power

- FFS on how to scale MSD for other output power level.

- Following the parameters in the table below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | [TBD] | ACLR2 |

Issue 3-4-1: whether preclude Case 4.

Issue 1-1-2: UE antenna type for inter-band CA Agreement: - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed. Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

WF:

- Further check whether preclude case 4.

Topic #4: R19 UE RF for UL-MIMO

Issue 4-1: whether preclude the single antenna port related requirement

Agreement:

- FFS

Issue 4-2: ULFPTx mode for UL-MIMO

WF:

- FFS

- Companies are courage to analyze the additional specification impact and benefit if ULFPTx mode for ATG is introduced.