3GPP TSG-RAN WG4 Meeting #113	R4-2420481

Orlando, US, 18th – 22nd November, 2024

Title: 	WF on PHS protection update

Agenda Item:	4.1.1

Source: 	KDDI, OPPO

Document for:	Approval

- Introduction

The moderator copies agreements and approved way forwards during the online session into this contribution in agenda 4.2.1. On top of that, we capture additional way forwards into this contribution.

Sub-topic 1-1 PHS protection update

- < Issue 1-1-1: Plan for NR A-MPR re-evaluation on n1 >

Agreement:

- If RAN4 can’t conclude n1 PC2 NS_05 and NS_05U related CRs in Feb., RAN4 conclude only n1 PC3 NS_05 and NS_05U related CRs in Feb.

Way Forward:

- Capture the updated Japanese regulations and simulation assumptions as follows.

- The PHS protection requirement will be modified to protect DECT as follows:

For n1/B1)

- BW = 5MHz

    - 1884.5-1910.0 MHz: -30 dBm/MHz

    - 1910.0-1915.7 MHz: -25 dBm/MHz

- BW = 10/15/20MHz

    - 1884.5-1906.6 MHz: -30 dBm/MHz

    - 1906.6-1915.7 MHz: -25 dBm/MHz

For other bands)

- 1884.5-1915.7 MHz: -30 dBm/MHz

- Invite re-evaluation/simulation for modified NS_05 and NS_05U for n1 PC3 and PC2 for RAN4#114 Feb. A-MPR is re-evaluated for the following limits and for channel positions in figure 1 and 2.

Band n1 – in 1920-1940 MHz case

Can smaller A-MPR values be expected than those of current NS_05/NS_05U for Case#1-#8?

Figure.1: 1920-1940 MHz case

Band n1 – in 1940-1960 MHz case

In current spec, A-MPR is defined for only 20 MHz CBW (Fc = 1950 MHz), no A-MPR for less than 20MHz CBW. With regulatory update, no A-MPR is needed even for 20MHz CBW for Digital cordless-phone service protection as it can be met by SEM and general spurious emission.

Note: NS_05U requires the same A-MPR with NS_100 to meet UTRA ACLR

Figure.2: 1940-1960 MHz case

- < Issue 1-1-2: Requirements to be updated >

Agreement:

- Update n1 additional emission requirements in 38.101-1 Table 6.5.3.3.4-1.

- Update spurious emission for UE coexistence and for NR CA in 38.101-1 and for EN-DC 38.101-3.

- Update B1 additional emission requirements in 36.101-1 Table 6.6.3.3.1.

- < Issue 1-1-3: NS value to be used to indicate the new A-MPR requirements >

Way Forward:

- Introduce new A-MPR in Rel-19 spec and reuse NS_05/NS_05U instead of introducing new NS.

  - UE can support it as optional before Rel-19.

- < Issue 1-1-5: PC3 A-MPR simulation results for NS_05 >

Agreement:

- Use the harmonized A-MPR region definition as below table as the starting point.

| NS_05 | Fc MHz | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5M |  | N/A | N/A | N/A | N/A |
| 10M | < 1930.7 | ≥ 1.5*RBstart + 4.7MHz/12/scs | >0 | <= 2.16MHz/12/scs | <= 1.8MHz/12/scs |
| 10M |  | N/A | N/A | N/A | N/A |
| 15M | 1927.5≦fc＜1932.5 | >= RBstart*1.45 + 4.68MHz/12/scs | >0 | <= 3.6MHz/12/scs | <= 3.6MHz/12/scs |
| 15M | 1932.5≦fc＜1938.2 | ≥ 12.96/12/scs | ≤ 0.54/12/scs | <= 2.16MHz/12/scs | <= 2.16MHz/12/scs |
| 20M | < 1945.7 | >= RBstart*1.5 + 4.14MHz/12/scs; | >0 | <= 5.76MHz/12/scs; <= RBstart*1.5 + 4.14MHz/12/scs; | <= 5.04MHz/12/scs |

- < Issue 1-1-6: PC3 A-MPR simulation results for NS_05U >

Way Forward:

- Continue further discussion based on more inputs in next meeting.

- < Issue 1-1-7: PC2 A-MPR proposals for NS_05 and NS_05U >

Way Forward:

- Continue further discussion based on more inputs in next meeting.

- < Issue 1-1-8: no AMPR regions >

Agreement:

- For the updated emission requirements which are covered by the SEM/SE, the AMPR can be considered as 0dB.

- Use the following observations for A-MPR evaluations.

- Observations: (R4-2418942, Skyworks)

- For other bands than band 1/n1 the DECT protection is guaranteed by the spurious emission level if the largest UL channels in operation in Japan are at least CBW+5MHz away from the 1884.5-1915.7 MHz DECT frequency range.

  - Based on our check this is the case for all bands of interest including band 3/n3

- For band 1/n1:

  - The SEM mask covers the -25 dBm/MHz level, and 5MHz lower in frequency, the spurious emission covers the -30dBm/MHz level, thus A-MPR would not be needed for all RB allocations as long as:

    - The 5MHz UL channel is at least 6MHz above 1915.7MHz. Thus, the 5MHz UL channels with Fc would not require any AMPR above 1924.2MHz.

    - The 10/15/20MHz UL channels are at least CBW above 1915.7MHz. Thus:

      - 10MHz UL channel Fc above 1930.7MHz would not require AMPR.

      - 15MHz UL channel Fc above 1938.2MHz would not require AMPR.

      - 20MHz UL channel Fc above 1945.7MHz would not require AMPR.

    - By analogy, accounting for the NR guard band, for lower frequency channels, MPR may be sufficient for any allocation above:

      - 1915.7+6+0.2425=1921.9425MHz for 5MHz CBW

      - 1915.7+10+0.3125=1926.0125MHz for 10MHz CBW

      - 1915.7+15+0.3825=1931.0825MHz for 15MHz CBW

      - 1915.7+20+0.4525=1935.7MHz for 20MHz CBW

  - With simulations/measurements it may be found that channels or allocations at lower frequency may pass but the above is a good start to limit the channel/allocation regions to be explored for the A-MPR work.

  - Since the -25dBm/MHz DECT protection range is always > 5MHz and the -25dBm/MHz SEM region is 5MHz, the -30dBm/MHz is de facto met once the -25dBm/MHz SEM coincides with the -25dBm/MHz DECT protection range.

Reference

[1] Rev2 R4-2418123 Topic summary for [113][101] Upto_R17_UERF_maintenance v2-with NWM comments, OPPO, RAN4#113