3GPP RAN WG4 Meeting #113															         	R4-2420502

Orlando, Fl, USA 18th – 22nd November, 2024

Agenda item:			5.9.1

Source:	EchoStar

Title:	Way Forward  on IoT-NTN in-band

Document for:	Approval

# Background

- NTN NB-IoT standalone operation was introduced from Rel-17 .

- NTN NB-IoT inband operation has not been introduced in specification till now and important for satellite operators.

- It was agreed in the March RAN plenary to address the NB-IoT NTN in-band/guardband operation as TEI-18 in RAN4.

- Contributions were submitted based on agreed WF from 112bis meeting that identified the issues to be resolved

- Based on contributions to this meeting, an offline discussion was held on 11/20 to reach agreement on issues related to NB-IoT NTN IB/GB operation (Topic 5, Sub-topic 1-1)

This document summarizes the agreements reached during offline discussions on some of the issues, while others need agreement based on further discussion during the Thursday online sesstion for thread 102.

# WF on NB-IoT NTN IB/GB operation

Topic 5, sub-topic 1-1, Issue 5.2, 5.6 – 3GPP Release Support

Offline agreements:

- RAN4 agrees to complete work under TEI-18 with Rel-18 CRs to be agreed to at the February meeting

  - Optional in Rel-18, Mandatory in Rel-19

  - Rel-18 Cat-B CR for approval in the February meeting

  - Rel-19 Cat-F CR for approval in the April/May meeting

- A separate WID is not required for completing this work

- Work distribution in terms of CR development will be agreed in the final WF agreement

o    TS 36.102 – Mediatek

o    TS 36.108 - TBD

o    TS 38.108 - TBD

o    TS 38.181 - TBD

o    TS 36.307 - TBD

Topic 5, sub-topic 1-1, Issues 5-3, 5.4– Signaling aspects and impact to other WGs

Offline agreements:

- Use operation of IB/GB NB-IoT in NR TN as baseline, i.e., utilize setting of operationModeInfo IE setting of “guardband-r13” to indicated IB/GB operation of NB-IoT in NR NTN channel

- Describe the signaling  to indicated IB/GB operation explicitly in TS 36.102 in a new sub-clause

- No impact to other WGs

Topic 4, sub-topic 1-1, Issues 5-5 – FDL/MDL determination

  - Option 1 (R4-2419537):

    - FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1); for the carrier including NPSS/NSSS for in-band operation, MDL is selected from {-2,- 1,0,1}.

  - Option 2 (Option 1a in R4-2149639):

NOTE 2:             For the carrier supporting in-band operation, the following equation is used.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL+1)

NOTE 3               For the carrier supporting in-band operation, MDL = -0.5 is not applicable for in-band operation.

NOTE 4               For the carrier including NPSS/NSSS for in-band operation, MDL is selected from {-2, -1, 0, 1}.

NOTE 5               Additional UE capability report for in-band operation would be required and introduced.

Option 3 ( Option 2a in R4-2149639):

NOTE 2:             For the carrier supporting in-band operation, in-band MDL = MDL + 0.5 and MUL = MUL + 0.5.

NOTE 3               For the carrier supporting in-band operation, MDL = -0.5 is not applicable for in-band operation.

NOTE 4               For the carrier including NPSS/NSSS for in-band operation, MDL is selected from {-2, -1, 0, 1}.

NOTE 5               Additional UE capability report for in-band operation would be required and introduced.

Option 4 (Option 1 in R4-2419583):

NOTE 1:             For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0.

NOTE 2:             For the carrier supporting in-band [and guard-band] operation, the following equation is used.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL+1)

NOTE 3               For the carrier supporting in-band [and guard-band] operation, MDL = -0.5 is not applicable for in-band[/guard-band] operation.

NOTE 4               For the carrier including NPSS/NSSS for in-band [and guard band] operation, MDL is selected from {-2, -1, 0, 1}.

NOTE 5               Additional UE capability for in-band operation would be required and introduced accordingly.

Option 4 (Option 2 in R4-2419583):

NOTE 1:             For the carrier supporting in-band [and guard band] operation, in-band[/guard-band] MDL = MDL + 0.5 and MUL = MUL + 0.5

NOTE 2:             For FDD MDL = 0 is not applicable for in-band [and guard band] operation.

NOTE 3:             For the carrier including NPSS/NSSS for in-band [and guard band] operation, in-band[/guard-band] MDL is selected from {-1.5, -0.5, 0.5, 1.5}.

NOTE 4:             For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0.

NOTE 5               Additional UE capability for in-band operation would be required and introduced accordingly.

Offline Agreements:

It is best to capture the modification using a separate sub-clause or a paragraph rather than NOTE

Final agreement to be paraphrased in a paragraph or a new sub-clause

NOTE 5 is not required since it was agreed that additional UE capability report is not required

Topic 5, sub-topic 1-1, Issues 5-1: Inclusion of guard-band support

Discuss the proposed text in the CR in R4-2419583 to specify support for both in-band and , guardband since there is no signaling impact

Offline Agreement:

# Reference

[1]	R4-2418124	Topic summary for [113][102] R18_UERF_maintenance_Part1_v0