3GPP TSG-RAN WG4 Meeting #113                                                               R4-2419815

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.20.4

Source:	Samsung

Title:	Ad-hoc meeting minutes for [113][306] NR_duplex_evo_General

Document for:	Information

# Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

## Open issues summary

### Sub-topic 1-1: SBFD applicability in NR band list

##### Issue 1-1-1: SBFD operation support in NR-U bands

- [Moderator] Based on the discussion in RAN4#112bis, it is agreed to “FFS the band for NR-U shall be supported”. Accordingly, companies have provided proposals on this issue.

- There is option to clearly indicate SBFD operation shall not be applied in NR-U bands:

  - Option-1 (CMCC, Samsung, Nokia, Ericsson): SBFD operation shall not be applied in NR-U bands.

- Other relevant observations/proposals:

  - Observation 1 (vivo): For NR-U supporting SBFD, these issues need further study:

    - Granularity of one subband: 20MHz

    - LBT threshold

  - Proposal 1 (ZTE): propose to clearly clarify whether NR-U bands in FR1 should be supported or not.

- Moderator Recommendation:

  - Agree on Option-1 to preclude SBFD operation in NR-U bands.

- Ad-Hoc agreement:

  - SBFD operation shall not be applied in NR-U bands, i.e., n46, n96 and n102.

##### Issue 1-1-2: SCS restriction for SBFD operation

- In this meeting, there is proposal to restrict SBFD to be used with 60kHz SCS in both FR1 and FR2-1:

  - Proposal 1 (Ericsson): For FR1 and FR2, do not use 60 kHz SCS for SBFD symbols.

- Moderator Recommendation:

  - Discussion on Proposal 1 firstly.

- Ad-Hoc discussion:

  - ZTE: if agree on this proposal, we foresee some of the inconsistence for the current BS RF requirements which includes 60kHz for the refsens requirements.

  - Ericsson: no one use 60kHz SCS in practice for both UE and gNB sides.

  - Huawei: similar view as ZTE. There is existing requirement in RRM requirements for 60kHz.

- Ad-Hoc agreement:

##### Issue 1-1-3: Specification implementation of operating bands support for SBFD

- [Moderator] Based on the discussion in RAN4#112bis, the following agreement is given for specification implementation for SBFD support for operating bands:

Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported

- Proposals:

  - Option-1 (Samsung): TP for clause 5.2

=================== Start of Text Proposal ===================

5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1.

Subband full duplex (SBFD) is designed to operate in all NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96 and n101, and all NR operating bands in FR2-1 which are defined in Table 5.2-2.

=================== End of Text Proposal ===================

  - Option-2 (Ericsson): To include the following text in the specification (either new specification or TS 38.104, which is still under discussion): subband full duplex can be applied to the following TDD bands given in Table 5.2-1 in TS 38.104: n38, n39, n40, n41, n48, n50, n77, n78, n79, n90, n102, n104 for FR1.

  - Option-3 (vivo): NR operating bands n38, n39, n40, n41, [n46,] n48, n50, n77, n78, n79, n90, [n96, ][n102,] n104 which are defined in Table 5.2-1, can be applied for subband full duplex operation.

- Moderator Recommendation:

  - Suggest to use Option 1 (given NR-U band is excluded based on the discussion results from Issue 1-1-1).

- Ad-Hoc discussion:

  - ZTE: prefer to exclude the bands not used for SBFD explicitly, i.e., Option-1, to avoid future revision if new bands are added.

  - Ericsson: If news bands which don’t have channel BW no less than 40MHz, even follow Option 1, we still need to change the wording, even if we follow Option-1.

  - Nokia: Another issue related is whether we have dedicated new specification. From readability perspective, we prefer Option-2.

  - Vivo: How about FR2-1? Should be FR2-1.

  - LGE: “All… except” is not good wording, and we already refer to the table.

##### Issue 1-1-4: Specification implementation of BS channel bandwidth support for SBFD

- [Moderator] Based on the discussion in RAN4#112bis, the following agreement is given for the support channel bandwidth for SBFD operation:

Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead.

- Proposals:

  - Option-1 (Samsung): introduce the following note in Table 5.3.5-1:

“Note 9: Subband full duplex (SBFD) can be applied to all supported channel bandwidth values except the ones smaller than 20MHz in band n39 and ones smaller than 40MHz in other TDD bands except bands n34, n39, n46, n51, n53, n54, n96 and n101.”

  - Option-2 (Charter): Add this note to Table 5.3.2-1:

    - Note: The channel bandwidth for a Subband Full Duplex carrier shall be no less than X = 40 MHz.

      - For band n39, X = 20MHz is used instead.

- Moderator Recommendation:

  - The similarity between two options has been observed, while it is suggested to discuss the note should be added in Table 5.3.5-1 or Table 5.3.2-1.

  - Suggest to collect views from other companies firstly.

- Ad-Hoc discussion:

  - Ericsson: postpose this discussion until we have agreement on the new or existing spec to be used for SBFD.

  - Nokia: same comment. That is the general comment to every TPs which we have.

  - Charter: The wording for Option 1, “all…except” is confusing.

### Sub-topic 1-2: Transmission bandwidth configuration for SBFD

##### Issue 1-2-1: Transmission bandwidth configuration support for SBFD

- [Moderator] In RAN4#112, much progress has been achieved on the definition clarification of BS channel bandwidth and BS transmission bandwidth configuration for SBFD

Agreement: - BS channel bandwidth for SBFD-capable BS: - BS channel bandwidth for SBFD-capable BS: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink or SBFD operation. Transmission bandwidth configuration for SBFD Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed

And in RAN4 #112bis, the following WF is provided:

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

- The following proposals are provided to discuss remaining general issues for transmission bandwidth configuration:

  - Proposal 1 (Tejas): Define the SBFD RBs requirement for all classes of base stations. (Moderator: my interpretation of this proposal is the SBFD transmission bandwidth configuration shall be defined for all kinds of BSs (WA, MR, LA))

- Moderator Recommendation:

  - Discussion the proposal first to see it can be covered by others as the common understanding or not. Further clarification on this proposal could be needed.

##### Issue 1-2-2: Restriction to use the same SCS for DL and UL subbands

- [Moderator] In RAN4 #112bis, the following WF is provided, in which it is discussion to “FFS the restriction of SCS for DL and UL subbands”:

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

- Based on companies’ proposals, the following options are given for the restriction of SCS for DL and UL subbands

  - Option 1 (ZTE): SCS for DL and UL sub-band should be the same.

  - Option 2 (vivo): Use the same SCS for DL and UL subbands in the illustrative figures for transmission bandwidth configuration for SBFD, while do not restrict different SCS for DL and UL subband.

- Moderator Recommendation:

  - Discussion the proposals first by collecting other companies’ preferences.

- Ad-Hoc discussion:

  - Huawei: maybe more prefer Option 2. Want to clarify is that possible to have different SCSs configured for UL and DL subbands?

  - ZTE: We refers to RAN1/2 agreement. According to current spec, SCS values for DL and UL subbands should be the same.

  - Vivo: UL and DL subbands are configured separately, according to our understanding from RAN1 agreement.

  - Ericsson: more flexibility given from other groups, while from RAN4 perspective, we should restrict it.

  - Tejas: We suppor the restriction of Option 1 from implementation persepctive.

  - Huawei: Restriction is already given from RAN2 spec. Even if there is any restriction, it should be captured in RAN2 spec.

  - CATT: For legacy gNB requirement, we don’t have such clarification in RAN4 spec. Why we have for SBFD?

- Conclusion from Ad-Hoc:

  - The restriction of SCS values for UL and DL subbands:

    - It is RAN4 common understanding the same SCS value should be used for UL and DL subbands

    - The restriction if any should be given in RAN2 spec, rather than in RAN4 spec.

##### Issue 1-2-3: Symbols for SBFD DUD configuration

- [Moderator] In RAN4#112, much progress has been achieved on the definition clarification of BS channel bandwidth and BS transmission bandwidth configuration for SBFD, transmission bandwidth configuration for SBFD DL subband NRB,SBFD,DL, transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL and guardband between SBFD UL and DL subband NRB,SBFD,GB, as follows:

Agreement: - Transmission bandwidth configuration for SBFD UL/DL subband: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - Guardband between SBFD UL and DL subband: - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks.

In RAN4#112bis, the illustrative figures are discussed and the group agree to use the figures from Ericsson [R4-2416134] used as the baseline for further study. In this study, we found it could be needed to “Revisit the DUD figure based on the updated definition of NRB,SBFD,DL”.

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

- [Moderator] The following proposals are provided to discuss the necessity of updating the definition for DUD case:

  - Option 1 (Samsung): Update the symbols for DUD case.

    - the transmission bandwidth configurations for two SBFD DL subbands are denoted by NRB,SBFD,DL,1 and NRB,SBFD,DL,2

    - the guardbands between SBFD UL and DL subbands are denoted by NRB,SBFD,GB,1 and NRB,SBFD,GB,2

    - The existing SBFD symbol explanation can be updated:

      - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks.

      - NRB,SBFD,DL,1: Transmission bandwidth configuration for the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

      - NRB,SBFD,DL,2: Transmission bandwidth configuration for the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

      - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

      - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband in SBFD DU or UD case, expressed in resource blocks.

      - NRB,SBFD,GB,1: The size of guardband between SBFD UL subband and the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

      - NRB,SBFD,GB,2: The size of guardband between SBFD UL subband and the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - Option 2 (Nokia): If needed to distinguish, define lower and upper DL sub-bands the following way:

    - NRB,SBFD,DL1: Transmission bandwidth configuration for SBFD lower DL subband for DUD configuration, expressed in resource blocks.

    - NRB,SBFD,DL2: Transmission bandwidth configuration for SBFD upper DL subband for DUD configuration, expressed in resource blocks.

- Moderator Recommendation:

  - Further discussion on the above options.

- Ad-Hoc discussion:

  - Ericsson: For the transmission bandwidth configuration, we are okay, while we prefer the same guardband size for DUD case.

- Ad-Hoc agreement:

  - Update the symbols as follows:

      - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks.

      - NRB,SBFD,DL,1: Transmission bandwidth configuration for the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

      - NRB,SBFD,DL,2: Transmission bandwidth configuration for the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

      - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

      - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband in SBFD DU, UD and DUD case, expressed in resource blocks.

##### Issue 1-2-4: Illustrative figures for transmission bandwidth configuration for SBFD

- [Moderator] The following agreement is provided in last meeting:

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

- Follow-up proposals for the corresponding figure and other spec. implementation issue:

  - Observation 1 (Samsung): The following issues are identified in the illustrative figures for SBFD DUD, DU and UD patterns:

    - Transmission bandwidth configuration for SBFD NRB,SBFD shall be used instead of  NRB

    - In SBFD DUD case, the values of NRB,SBFD,DL,1 and NRB,SBFD,DL,2 are used instead, which can be different

    - In SBFD DUD case, the two values of NRB,SBFD,GB,1 and NRB,SBFD,GB,2 are used instead, which can be different

    - WGuard is not defined which should not be used

    - “Channel Edge” can be added to illustrate channel bandwidth edge, the same as Figure 5.3.1-1 in TS38.104.

    - “Resource Block” can be added to illustrate the resource block represented by a rectangular, the same as Figure 5.3.1-1 in TS38.104.

- The figures for SBFD DUD, DU and UD are updated in Ericsson’s contribution:

Figure 1: Transmission bandwidth configuration for SBFD DUD pattern for one NR channel

Figure 2: Transmission bandwidth configuration for SBFD DU pattern for one NR channel

Figure 3: Transmission bandwidth configuration for SBFD UD pattern for one NR channel

- Moderator Recommendation:

  - Further discussion on the above illustrative figures.

### Sub-topic 1-3: DL/UL subband configurations support for SBFD

##### Issue 1-3-1: Clarification on declaration and flexibility

- [Moderator] For clarification on declaration and flexibility, the following proposal is given:

  - Proposal 1 (Ericsson): The SBFD capable BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL/UL/guardband setting other than those that have been declared and tested.

- Moderator Recommendation:

  - Further discussion on the above proposal.

- Ad-Hoc discussion:

  - Qualcomm: How to ensure this?

  - ZTE: We don’t see the necessity of this agreement. For legacy BS demod, we also have certain TDD pattern to be specified and tested, however we never have this kind of restriction on the practical deployment. we don’t understand how to ensure this in the practical network.

- Ad-Hoc agreement:

  - The SBFD capable BS product shall not operate on a SBFD configuration (DL/UL subband) other than those that have been declared [and tested].

    - FFS on how to capture this in the specification.

##### Issue 1-3-2: DL/UL subband configurations support for SBFD

- [Moderator] Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. In RAN4#112, the following three options with different levels of configuration flexibilities have been provided:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility.

- Companies’ preference on 3 identified options (based on last meeting’s contributions, and updates if any):

  - Option-1 (with the least flexibility): Nokia, Ericsson

  - Option-2 (with the middle flexibility): CMCC, Samsung, CATT, vivo, Huawei, ZTE, Qualcomm

- Furthermore, by following the procedure given in R4-2418790, the following table is given for Option 1:

  - Proposal 1 (Ericsson): RAN4 define SBFD subband configurations for WA BS as shown in Table 2 for FR1, by (1) Reuse the existing channel bandwidths for SBFD UL subbands, (2) For WA BSs, wider guardbands than existing ones are needed for SBFD.

|  |  | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
| DUD (SCS 30 kHz) | NRB,SBFD,DL | 49/50 | 5857 | 70/71 | 78 | 92 | 106 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 45 | 5 | 5 | 5 | 5 |
|  | D | 40 | 45 | 55 | 60 | 70 | 80 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
| DU/UD (SCS 30 kHz) | NRB,SBFD,DL | 104 | 119 | 146 | 161 | 189 | 217 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 5 | 5 | 5 | 5 | 5 |

- Moderator Recommendation:

  - Still need more discussion on three options.

##### Issue 1-3-3: Additional restriction for Option 2b - DL subband sizes to be declared/tested

- [Moderator] Based on last meeting’s agreement, the following additional restrictions are FFS for Option 2b:

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS.

- [Moderator] For the restriction (the number of DL subband sizes to be declared and tested), the following proposals/observations are provided.

- Among Alt-1 and Al-2:

  - Support Alt-1 (Nokia, Ericsson): One DL subband size to be declared and tested.

  - Support Alt-2 (Tejas, Samsung, CMCC, vivo, Qualcomm, ZTE, Huawei): For FR1 WA BS, all DL subband sizes declared to support SBFD shall be tested;

    - Qualcomm: FFS the number of downlink subband configuration supported for FR1 WA BS

- For the restriction to other FR1 BS classes and FR2 BS:

  - Option 1 (Tejas, Samsung, vivo, Qualcomm): This restriction is not applicable to other FR1 BS classes and FR2 BS.

  - Option 2 (Nokia): Apply Alt-1 also for FR1 MR BS subband configuration. More flexibility for subband configuration can be considered for FR1 LA BS. Subband configurations for FR2-1 is still FFS.

- Moderator Recommendation:

  - The discussion on this issue can be continued if the group see the possibility of adopting Option-2b.

    - Further discussion is still needed on above Alt-1 and Alt-2.

    - Further discussion is still needed on the restriction to other FR1 BS classes and FR2 BS.

- Ad-Hoc discussion:

  - Ericsson: Alt-1 is still different from option1, since it gives the flexibility for different guardband size to be used.

  - Huawei: for each uplink suband size, it is allowed for only one downlink subband size? How about multi-carrier, e.g., 100MHz for SBFD and 100MHz for TDD.

  - ZTE: If BS is capable with multiple analog filtering and switch between each other, we could allow to have multiple sub-band configuration, this agreement is restrictive to the implementation. For intra-band contiguous CC with one uplink sub-band configuration, this needs more discussions,  from our understanding, this release, we need to focus on single carrier.   If we support this feature combination, this is new type of SBFD BS.

  - CMCC: We still have some concerns about setting only specific UL subband size, as we believe this will impose restrictions on the deployment of macro sites.

- Ad-Hoc agreement:

  - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration:

    - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD

      - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

    - [For single carrier operation, for the supported channel BW size, only one specific UL subband size is allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

      - FFS the range of the ratio of the UL subband size relative to the channel bandwidth.

      - FFS different sizes for DUD and DU/UD

      - Note: RAN4 try to solve the FFS in this meeting

    - For multi-carrier operation, for one supported aggregated channel BW size, only one specific UL subband size is allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

      - FFS the range of the ratio of the UL subband size relative to the channel bandwidth.

      - FFS different sizes for DUD and DU/UD

      - Note: RAN4 try to solve the FFS in this meeting]

    - For a specific channel BW and a specific UL subband size,

      - only one DL subband size is allowed to be declared and tested for DU/UD case

      - only one DL subband size is allowed to be declared and tested for DUD case

      - The above restrictions shall be applied to FR1 WA BS only, as the starting point

        - FFS the restrictions are also applied to be FR1 MR BS

##### Issue 1-3-4: Additional restriction for Option 2b - UL subband size based on the specific channel BW

- [Moderator] Based on last meeting’s agreement, the following additional restrictions are FFS for Option 2b:

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case.

- [Moderator] For the restriction (on UL subband size based on the specific channel bandwidth), the following proposals/observations are provided:

  - Option 1 (CATT, ZTE): No restriction on the UL SB size ratio in the core requirements.

  - Option 2 (Tejas): The UL subband size relative to the channel bandwidth can range from 20% to 40%.

  - Option 2a (Samsung, vivo): For DUD and DU case, the ratio for UL subband size over channel bandwidth shall be no less than 20%

  - Option 3 (Huawei): As a compromise, only introduce the following bandwidth from Table 5.3.2-1 for SBFD UL subband configuration:

    - 10MHz, 20MHz, 30MHz, 40MHz, 50MHz, 60MHz 70MHz, 80MHz and 90MHz.

  - Option 4 (Ericsson, Nokia): For one supported channel bandwidth, only one fixed UL subband size is specified, which is around 20% of the supported channel bandwidth.

- For the restriction to other FR1 BS classes and FR2 BS:

  - Option 1 (Samsung)

    - The restriction can be applied to both FR1 and FR2.

- Moderator Recommendation:

  - The discussion on this issue can be continued if the group see the possibility of adopting Option-2b.

    - Further discussion is still needed on above options for the restriction on UL subband size based on the specific channel bandwidth.

- Ad-Hoc meeting:

  - Tejas: Why 25% for DUD case?

  - Ericsson: because of only 20% is studied, and the UL subband size can be increased a little bit.

##### Issue 1-3-5: Additional restriction for Option 2b - UL subband in the middle of channel BW

- [Moderator] Based on last meeting’s agreement, the following additional restrictions are FFS for Option 2b:

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth

- [Moderator] For the restriction (the UL subband should be placed in the middle of channel bandwidth for DUD), the following proposals/observations are provided:

  - Observation 1 (Samsung): For restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD), it is possible that UL subband can’t be located at the exact middle of channel bandwidth.

  - Proposal 1 (Samsung): The proponent of restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD) shall solve the issue in the Observation 1.

  - Proposal 2 (CATT): Flexibility of the UL SB transmission center frequency should be allowed that it may not be the center frequency of UL SB.

  - Proposal 3 (vivo): For DUD case, the UL subband should be placed in the middle of channel bandwidth

  - Proposal 4 (Ericsson, Nokia): Only symmetric DUD configurations are specified, as there is no foreseeable benefit from flexible placement of the UL subband.

- Moderator Recommendation:

  - The discussion on this issue can be continued if the group see the possibility of adopting Option-2b.

    - Further discussion is still needed on the restriction of the UL subband in the middle of channel BW for DUD case.

##### Issue 1-3-6: Other restriction on DL subband size

- [Moderator] Based on CATT’s analysis, the following proposals are given. For 2nd proposal, although it is related to channel bandwidth size, we still put it in this issue, because it is the proposal derived from 1st proposal.

  - Proposal 1 (CATT): For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

  - Proposal 2 (CATT): FR2 minimum CBW to support SBFD is 100MHz.

- Moderator Recommendation:

  - Discussion on the above proposals firstly.

- Ad-Hoc agreement:

  - FFS in this meeting:

    - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

    - FR2 minimum CBW to support SBFD is 100MHz.

##### Issue 1-3-7: Restriction on guardband size for SBFD

- [Moderator] Based on last meeting’s agreement, the following options for restriction on guardband size are identified:

Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

- Companies’ preference (based on this meeting’s contribution):

  - Option 3: CMCC, ZTE

  - Option 3a: Samsung, CATT, ZTE

- The following proposal is given for GB size, while it is related to restricting SCS for UL and DL subbands in Issue 1-3-1.

  - Proposal 3 (vivo): GB doesn’t have to be in the number of RB when different SCS are configured for UL and DL subbands. (Moderator: the proposal is given because of P2)

- Moderator Recommendation:

  - Further discussion on the above options, which are dependent on above issues within this sub-topic.

### Sub-topic 1-4: Other general issues for SBFD requirements

##### Issue 1-4-1: BS RF Specification structure for SBFD requirements

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved. Accordingly, BS RF specification structure for SBFD requirements shall be further discussed in this meeting.

<From RAN4#112> - Way forward: - RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS: - Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104) - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements - Option 3: A new specification for SBFD BS RF requirements

- Options of how to introduce BS RF new requirements for SBFD-capable BS:

  - Option 2 (CMCC, Samsung, vivo, Qualcomm, Huawei): Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD

    - Option 2b (CMCC, Samsung): Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements

  - Option 3 (Ericsson, Nokia): A new specification for SBFD BS RF requirements

- Furthermore, there are proposals related to how to handle this discussion:

  - Proposal 1 (Samsung): The issue of BS RF specification structure (or potentially new specification) is standardization problem for specification implementation, which shall be separated from other technical discussion.

- Moderator Recommendation:

  - Based on majority view, agree on Option 2 firstly.

##### Issue 1-4-2: CLI handling impact on RF requirement

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved.

<From RAN4#112> - Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Based on previous meeting’s discussion, there is no agreement to either revisit or confirming no revisit the “FFS” bullet.

- Further proposals are provided:

  - Proposal 1 (Nokia): RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Moderator Recommendation:

  - Suggest to deprioritize the discussion on this issue since it is not easily to be concluded, and this will also not directly make any difference for detailed RF requirements discussion till now.

# Topic #2: PUSCH with resource muting for CLI handling

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Open issues summary

### Sub-topic 2-1: RF requirement impact for PUSCH with resource muting

##### Issue 2-1-1: RF requirement impact for PUSCH with resource muting

- [Moderator] The following object is given in the updated WID:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-s-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap with UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability

- [Moderator] Based on moderator’s understanding, it is not clear this issue should be directly discussed in RAN4 or triggered to be discussed in RAN4 by RAN1 LS. The following observations are provided in companies’ contributions:

  - Observation 1 (Qualcomm): Resource muting changes the waveform characteristics and could have an impact on MPR.

  - Observation 2 (Qualcomm): With symbol level resource muting, emissions calculation (like ACLR, EVM, IBE, etc.)  needs to be revisited.

  - Observation 3 (Huawei): For UL resource muting, there is no transmit quality or MPR requirement impact can be identified for PUSCH as the spec is already support DMRS multiplexing as the data could be mapped according to comb-2 pattern. Further measurements or simulation can be conducted if RAN1 inputs on necessary assumptions is possible.

- Moderator Recommendation:

  - Collect companies’ view on the above observations firstly.

- Ad-Hoc dsicussion:

  - Huawei: already have offline with QC and need FFS

  - Qualcomm: it could be conflicting to previous proposal on CL handling.

# Topic #3: Potentially new BS RF requirements for SBFD

## Open issues summary

### Sub-topic 3-1: Requirements for Transient period between SBFD and non-SBFD

##### Issue 3-1-1: Transient period requirement applicability on frequency/antenna

- [Moderator] In RAN4#112 and #112bis, the following agreement and WF are achieved for the requirement for transient period between SBFD and non-SBFD:

<Agreement in RAN4#112> Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. <Agreement in RAN4#112bis> Way Forward: - FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498.

- Observation/Proposal from Samsung:

  - Observation 1 (Samsung): The transient period for SBFD operation is different from TDD operation in terms of:

    - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources, and

    - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements.

  - Observation 2 (Samsung): The transient period for SBFD operation could be further summarized as below, by elaborating the four different cases for different SBFD antenna configuration:

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

  - Proposal 1 (Samsung): Based on the example given in R4-2416498, the text proposal is updated for the clause of transmitter transient period for SBFD:

=================== Start of Text Proposal ===================

#### 6.4.2B	Transmitter transient period for SBFD

##### 6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the SBFD TX and RX antenna penal and transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases antenna configuration:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

- Observation/Proposal from Nokia:

  - Observation 1 (Nokia): On transient period requirements, that SBFD antenna configuration option 1, 2, 3 mentioned in R4-2416498 were discussed in Rel-18 by RAN1/RAN4 for the purposes of performance evaluation, but no agreement on normative specification impact has been made.

  - Proposal 1 (Nokia): Agreement on a text proposal on transmitter transient period for SBFD should wait for the discussion on whether SBFD would have a new dedicated specification for SBFD-capable BS RF requirements or sub-clauses for TS 38.104 would be used.

  - Proposal 2 (Nokia): Potential text proposal on transmitter transient period for SBFD should adhere to the agreement reached in RAN4#112 where four transition cases were identified as shown in Table 1, without detailing gNB antenna configuration options.

- Observation/Proposal from Ericsson:

  - Proposal 1 (Ericsson): RAN4 do not define an architect dependent requirement. Transient period between SBFD and non-SBFD is applicable to all four cases.

- Moderator Recommendation:

  - RAN4 agree on the following principle from Ericsson:

    - RAN4 do not define an architect dependent requirement. Transient period between SBFD and non-SBFD is applicable to all four cases.

  - Based on the above principle (if agreed), discuss the revised text proposal from Samsung.

- Ad-Hoc agreement:

  - RAN4 shall not define an SBFD-architect dependent requirement.

  - Transient period between SBFD and non-SBFD is applicable to all four cases.

##### Issue 3-1-2: Location of transient period between SBFD and non-SBFD

- [Moderator] In RAN4#112, the following agreement is achieved for “Location of transient period between SBFD and non-SBFD”:

- Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided.

- Location of transient period between SBFD and non-SBFD:

  - Option 1 (Samsung): No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not.

  - Option 1a (Ericsson): Place the transient period outside of both SBFD and non-SBFD slots.

  - Option 2 (CATT/Tejas/Huawei/ZTE): The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol).

- Furthermore, the following observations are provided to support the above options.

  - Observation 1 (Samsung): For the identified four cases of transitions to/from SBFD symbols, the following timing behaviors are expected:

|  | Expected timing behavior for DL/UL subbands respectively |  | Location of transient period (if any) |
| --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol. | N/A |
|  | UL subband | TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period. | Located in TA_offset in SBFD symbol |
| Case-B (SBFD symbol to UL symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbols reserved in SBFD symbols |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all | N/A |
| Case-C (DL symbol to SBFD symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol. | N/A |
|  | UL subband | UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbols reserved in non-SBFD DL symbols |
| Case-D (UL symbol to SBFD symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset | Located in TA_offset in non-SBFD UL symbol |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all. | N/A |

  - Observation 2 (Samsung): The location of transient periods should be located in (1) SBFD symbol for Case-A; (2) SBFD symbol(s) reserved as guard symbol(s) for Case-B; (3) Non-SBFD DL symbol(s) reserved as guard symbol(s) for Case-C; (4) Non-SBFD UL symbol for Case-D.

  - Observation 3 (Tejas): In general, the transient time should be the same for both full TDD switching and switching related to SBFD, as the total transmit (TX) switching time budget must be consistent for both switching between the complete uplink/downlink (UL/DL) and the SBFD resource blocks (RBs). For the transitions between DL-SBFD UL and SBFD-DL, the existing TDD switching time can be used, since the power amplifier (PA) will always remain on during this transition. Additionally, because the user equipment (UE) operates in half-duplex mode, the transition from non-SBFD DL to SBFD UL should account only for the PA switching time. As a result, no new requirements are anticipated from the base station (BS) side.

- Moderator Recommendation:

  - RAN4 can firstly discuss the technical details behind Option 1 and 2, while if no conclusion, consider to send LS to RAN1 to clarify this issue.

- Ad-hoc meeting:

  - ZTE: We still have some concerns on this one, we need to clarify it in the spec, otherwise it will be confusing for UE implementation.

  - CMCC: Similar to concern from ZTE

- Ad-Hoc agreement:

  - [No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not.]

### Sub-topic 3-2: In-channel adjacent subband leakage ratio requirements

##### Issue 3-2-1: Necessity of in-channel adjacent subband leakage ratio requirements

- [Moderator] The necessity of the requirement for in-channel adjacent subband leakage ratio requirements has not yet been confirmed, based upon previous meeting discussion. In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

- Proposals:

  - Option 1 (Samsung/CATT/Qualcomm/Huawei): no necessity to introduce new requirement for in-channel adjacent subband leakage ratio.

    - Option 1a (CMCC): In-channel adjacent sub-band leakage ratio should be defined or reflect by other requirements.

  - Option 2 (Nokia/Ericsson): New requirement for in-channel adjacent subband leakage ratio should be defined.

    - Option 2a (Nokia/Ericsson): RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering co-site inter-sector interference and inter-site interference. Existing ACLR requirements could be used as baseline depending on the ratio between the bandwidths of the DL and the UL subbands.

  - Option 3 (ZTE):

    - for the inter-site scenario, not to define in-channel sub-band leakage and leave this up to the implementation.

    - if companies have strong preference to define the Tx in-band leakage requirement, then prefer to define it similar as UE in-band emission general requirement which could be applicable for any DL sub-band transmission bandwidth configuration and also UL sub-band transmission bandwidth configuration.

- Moderator Recommendation:

  - This issue has been discussed for several meetings without any progress.

  - Since Option 3 is newly proposed, companies are encouraged to discuss this new option.

### Sub-topic 3-3: In-channel adjacent subband selectivity/blocking requirements

##### Issue 3-3-2: Necessity of in-channel adjacent subband blocking requirement

- [Moderator] The necessity of the requirement for in-channel adjacent subband blocking requirements has not yet been confirmed, based upon previous meeting discussion. In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

- Proposals:

  - Option 1 (Samsung/CATT/Qualcomm/Huawei/ZTE): no necessity to introduce new requirement for in-channel adjacent subband blocking.

  - Option 2 (Nokia/CMCC/Ericsson): New requirement for in-channel adjacent subband blocking should be defined.

    - Option 2a (Ericsson): Define a requirement on RX sub-band blocking similar to the in-band blocking requirement and use existing in-band blocking requirement as baseline.

- Moderator Recommendation:

  - This issue has been discussed for several meetings without any progress. Moderator suggest more detailed proposal on Option 2 for how this requirement shall be defined, such as interference level, etc.

  - Considering that the in-band blocking requirement will be specified anyway, can this new requirement can be guaranteed by the in-band blocking requirement specified for SBFD operation?

##### Issue 3-3-1: Necessity of in-channel adjacent subband selectivity requirements

- [Moderator] The necessity of the requirement for in-channel adjacent subband selectivity requirements has not yet been confirmed, based upon previous meeting discussion. In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

- Proposals:

  - Option 1 (Samsung/CATT/Qualcomm/Huawei/ZTE): no necessity to introduce new requirement for in-channel adjacent subband selectivity.

  - Option 2 (Nokia/CMCC): New requirement for in-channel adjacent subband selectivity should be defined.

  - Option 3 (Ericsson): No necessity for selectivity requirement if the in-channel adjacent subband blocking requirement is defined.

- Moderator Recommendation:

  - Discussion on the above issue firstly. Seems the selectivity requirement can be included in the blocking requirement (if any).

### Sub-topic 3-4: Inter-sector and inter-site interference

##### Issue 3-4-1: Necessity of new requirement for inter-sector and inter-site interference

- [Moderator] The following proposal is given in this meeting:

  - Proposal 1 (Ericsson): New requirement to capture Inter-sector interference shall be adopted.

Figure 1: Illustration of inter-sector interference

  - Additional explanation from Ericsson:

    - “In our view, inter-sector interference has been one of the main discussion points since start of SI. There are dedicated tables which take inter-sector islolation to account, e.g., Table 9.2.2.1-1: FR1 WA BS Co-site Inter-sector Co-channel Interference Analysis Summary, row ”Spatial isolation”.”

- Moderator Recommendation:

  - Considering this new requirement is not yet studied in Rel-18, suggest no discussion on this new requirement, considering the limited TU in Rel-19.