3GPP TSG-RAN WG4 Meeting #113	R4-2420087

Orlando, USA, November 18th ‒ 22nd, 2024

Title: 	WF on SL positioning and carrier phase positioning performance

Agenda Item:	5.1.2

Source: 	CATT

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [113][212] NR_pos_enh2_Part2.

Topic #1: Sidelink Positioning Requirements

Sub-topic 1-1 SL Positioning Performance Requirements

Issue 1-1-1: Configurations of TX UEs in TCs

Agreement:

- For TCs with 2 TX UEs

    - If UE supports at least 2 active slots, the 2 TX UEs are in consecutive SL slots.

    - If UE supports only 1 active slot, the 2nd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE.

- For TCs with 3 TX UEs

    - If UE supports more than 2 active slots, the 3 TX UEs can be in consecutive SL slots.

    - If UE supports 2 active slots, the first 2 TX UEs are in consecutive SL slots, and the 3rd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE

    - If UE supports only 1 active slot, the 2nd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE, and the 3rd Tx UE is in the next SL slot immediately after UE finishes processing the 2nd TX UE.

Issue 1-1-2: Reporting granularity for SL RTOA

Agreements:

- To align with SL RSTD measurements, remove timing resolutions k = {0, 1} from the reporting mapping for SL RTOA.

Topic #2: Carrier Phase Positioning Requirements

Sub-topic 2-1 Carrier Phase Positioning Performance Requirements

Issue 2-1-1: CPP accuracy measurements

Agreements from the ad-hoc meeting:

- Simulation results in R4-2419359 (Ericsson): update simulation summary (CATT), consider the results in deriving accuracy requirements.

- RAN4 to clarify that CPP measurement accuracy requirements do not apply for comb size 12.

- Larger BWs: Define accuracy requirements.

- Further discuss off-line to converge on numbers for Y values in RAN4#113, e.g.:

  - 14.2 deg for FR1

  - 28 deg for FR2

Further agreements:

- Keep the accuracy requirements for the configurations with 4 repetitions.

- The following Y values are confirmed and to be used in the CR:

  - 14 deg for FR1

  - 28 deg for FR2

Reference

- R4-2418270, Topic summary for [113][212] NR_pos_enh2_Part2, CATT, RAN4#113.

- R4-2420079, Ad-hoc minutes for NR_pos_enh2 WI, Ericsson, RAN4#113.