3GPP TSG-RAN WG4 Meeting #113	                                                           R4-2420094

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.15.4

Source:	CATT

Title:	WF on RRM requirements for NR_RRM_Ph5_Part2

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [113][219] NR_RRM_Ph5_Part2.

- Topic #1: Fast SCell activation for UE supporting Rel-18 EMR

  - Sub-topic 1-1 General

Issue 1-1-1: Applicability of fast SCell activation delay requirements

Agreement:

- For R19 fast SCell activation for UE supporting R18 EMR, when neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured:

  - Fast SCell activation delay requirements (same with conclusion of the case where either measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured) apply under the following conditions:

    - For EMR carriers:

      - Consider the case when the T331 timer hasn’t expired at the moment of initiation of RRC state transition to Connected mode. i.e., measIdleDuration-r16 is configured, and T331 is running.

  - Otherwise, follow the legacy unknown SCell activation delay requirements.

Issue 1-1-2: Target scenarios of fast SCell activation

Agreement:

- Fast SCell activation requirements defined in R19 do NOT apply to the following scenarios:

  - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA)

  - SCell being in the same band as PCell in FR2 (i.e. intra-band CA)

Issue 1-1-3: Work Schedule for fast SCell activation with Rel-18 eEMR

  - Sub-topic 1-2 SCell activation delay requirements

Issue 1-2-1a: The fast SCell activation delay components for direct SCell activation at SCell addition

Agreement:

- For single direct SCell activation at SCell addition, for FR1

  - provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

    - use existing SSB based known SCell activation delay requirement with 2 SSB samples, i.e. Tactivation_time is TFirstSSB_MAX + Trs + 5ms.

- For single direct SCell activation at SCell addition, for FR2

  - provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

    - use existing SSB based known SCell activation delay requirement with 2 SSB samples.

      - delay components: 2 SSB samples * beam sweeping +1 SSB sample

      - Further discuss the detailed requirements when drafting the CR

Issue 1-2-1b: The fast SCell activation delay components for MAC CE based single SCell activation

Agreement:

- For MAC CE based SCell activation delay requirements with eEMR report, reuse the same conclusions as direct SCell activation at SCell addition.

Issue 1-2-1c: The time gap condition for MAC CE based single SCell activation

Agreement:

- For MAC CE based SCell activation delay requirements with eEMR report, reuse the same conclusions as direct SCell activation at SCell addition, i.e., [Y] equals to 5s for FR1, and 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 for FR2.

Issue 1-2-1d: The SSB detectability condition for both direct SCell activation and MAC CE based single SCell activation

Agreement:

- The earliest possible RRC Idle/Inactive measurement time instance is defined as below:

  - From the time UE receives the configuration of measurement in idle and inactive modes.

- Discuss the details when drafting the CR.

Agreement:

- Agree on the following as baseline for CR:

  - If the configured validity duration is only based on Rel-16 EMR, it refers to the time instance when the UE obtained the configuration parameter (i.e., measIdleDuration-r16).

  - If the configured validity duration includes Rel-18 eEMR, the configured, it refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission.

Issue 1-2-2: Consideration on measurement period condition of FR1 known SCell activation

Agreement:

- The issue can be covered by issue 1-2-1a, and no further agreements are needed.

Issue 1-2-3: Whether the indication to network is needed

Agreement:

- It is RAN4 common understanding that fast SCell activation requirements apply for any of reported cells, as long as side conditions (agreed in previous issues) are met. No need to introduce additional indication to the network for fast SCell activation.

Issue 1-2-4: UE capability

Agreement:

- Discuss the UE capability by the end of core part.

Reference

- R4-2418277 Topic summary for [113][219] NR_RRM_Ph5_Part2, CATT, RAN4#113