3GPP TSG-RAN WG4 Meeting #113											     R4-2420108

Orlando, US, 18 ‒ 22 November, 2024

Title: 	WF on RRM requirements for NR_MIMO_Ph5_Part2

Agenda Item:	7.19.4

Source: 	MediaTek inc.

Document for:	Approval

# <Topic 1: CSI enhancement to support up to 128 ports>

Issue 1-1: Whether RRM impacts exist by the added SRS port grouping for CSI enhancement?

In the last RAN plenary meeting, RAN#105, another sub-bullet (d) was approved for CSI enhancement (Objective#2):

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

Agreement:

- No RRM impact by added SRS port grouping to support more than 4 layers.

- Note: the agreement can be revisited if any impact observed based on further RAN1 progress.

# <Topic 2: UE reporting enhancement for CJT calibration>

Issue 2-1: On the simulation assumption for CJT calibration

Agreement is captured in R4-2420209.

Issue 2-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration?

Way forward:

- Option 1: Define accuracy requirements.

- Option 2: FFS.

Issue 2-3: Whether report mapping tables should be specified by UE reporting enhancement for CJT calibration?

Way forward:

- Option 1: Define mapping tables for frequency/delay/phase offset reporting in performance part.

# <Topic 3: 3TX support>

Issue 3-1: RRM core/performance impacts by introducing 3TX support?

Previous RAN4 agreement:

Agreement (from RAN4#112-bis): - No new RRM core requirements by introducing 3TX support.

Agreement:

- No new RRM performance requirements by introducing 3TX support.

# <Topic 4: Enhancement for asymmetric DL sTRP/UL mTRP scenarios>

Issue 4-1: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

In the last meeting (RAN4#112-bis) RAN4 agreed to update the RRM requirements for 2TA enhancement:

Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting.

Way forward:

- Alt 1: Define requirement for 2TA following the legacy principle.

- Alt 2: Wait for RAN1.

Issue 4-2: Whether to define UL TCI state activation requirement based on SRS?

Way forward:

- Option 1: Not define requirement (Samsung, HW, vivo)

- Option 2: RAN4 to define UL TCI state activation requirement based on SRS. (Xiaomi, Ericsson, Nokia)

- Option 3: Further discuss based on the feasibility of defining requirement and testing (QC, Apple, MTK)

Issue 4-3: RRM core impacts of active uplink TCI state switching delay for unified TCI?

Way forward:

- Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Option 2: RAN4 to specify UL TCI state switching requirements.

- Option 3: FFS.