3GPP TSG-RAN WG4 Meeting #113	R4-2420131

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.18.4

Source:	Nokia

Title:	Coffee break discussion minutes for FS_NR_AIML_Mob

Document for:	Approval

# Introduction

This document contains the ad-hoc meeting minutes for the discussion on AI/ML mobility SI. The ad-hoc meeting discussed some of the topics from the moderator’s summary [1].

# Discussion.

## Topic#2: RAN4 Requirements

### Sub-topic 2-1: General and scope

#### Issue 2-1-9: Cell-level RSRP prediction accuracy definition

- Proposals and Observations:

  - Proposal 1 (Apple):

    - The proposed cell-level RSRP prediction accuracy is defined as follows: Absolute RSRP Accuracy for Cell-Level = predicted L3-RSRP of cell i - ideal received L3-RSRP of cell i

      - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy.

      - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test)

  - Proposal 2 (Huawei):

    - The methodology of prediction accuracy from the link level simulation perspective is,

      - Ideal RSRP at point C:

        - Input is at point A and output is at C (i.e., Input from point A, experiencing channel without noise=> ideal RSRP at point A1=> ideal RSRP at point B=> ideal RSRP at point C).

        - Ideal RSRP does not include any channel estimation noise.

      - Estimated RSRP at point C:

        - Sub-use case 1: Input from point A, experiencing channel with noise=> measured RSRP at point A1=>predicted RSRP at point A1=> generated RSRP at point B=> generated RSRP at point C

        - Sub-use case 2: Input from point A, experiencing channel with noise=> measured RSRP at point A1 => measured RSRP at point B=> measured RSRP at point C=>predicted RSRP at point C

        - Sub-use case 3: Input from point A, experiencing channel with noise=> measured RSRP at point A1 =>predicted RSRP at point C

- Recommended WF

  - Discuss above listed proposals.

Discussion:

Vivo:  This definition should not be valid for sub-use case 1 and 2.

Huawei: This is for sub-use case 2.

Qualcomm: Why different approaches for FR1 and FR2.

Nokia: In FR2, there are wide variations. In FR1, for conducted test, there is less uncertainty.

Qualcomm: Remove FR2 and FR1.

Nokia: We have two options at this stage. For FR1, ground can be derived by the TE. Reported measurement may not be the best option.

CMCC: In FR2, there is some UE Rx beam gain, therefore, we have reported measurement as the ground truth. The higher SNR should be used for accurate measurements.

OPPO: Same view as CMCC. It should not be limited to sub-use case 2.

MTK: It is only valid for single AoA. In FR1, with multi beams, we can’t know the ground truth using transmit power.

Xiaomi: There are two different implementations of the UE, sliding and non-sliding, we should first agree on the channel model.

Agreement:

- For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP

  - FFS Ground truth definition of L3-RSRP for FR1 and FR2

# References

[1] R4-2418278, “Topic summary for [113][220] FS_NR_AIML_Mob”, Moderator(Nokia), RAN4#113