3GPP TSG-RAN4 Meeting #113	R4-2420133

# Orlando, US, 18 – 22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5173 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on requirements for RedCap positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are agreements to update the definition of Lavailable_PRS,i, Nhop,i,j and NrepPRS in RAN4#112 and RAN4#112-bis, the changes are done for Connected requirements, but are missing in the requirements for Idle and Inactive. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the definition of Lavailable_PRS,i, Nhop,i,j and NrepPRS based on agreements in RAN4#112 and RAN4#112-bis for Idle and Inactive. The changes are same as those for Connected requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for RedCap UE with FH are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.6, 4.6.3.6, 5.6A.4.6, 5.6A.5.6, 5.6A.6.6, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on endosed big draftCR R4-2416884 from RAN4#112-bis |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 4.6.2.6	Measurement Period Requirements with RX FH

The requirements in this clause apply when a RedCap UE is requested by the LMF to perform measurements with FH, and UE reports measurement based on multiple hops.

The requirements in clause 4.5.2.5 shall apply with the following modifications.

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single time window.

-	Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i*Lper-hop,i, where

-	Nhop,i,j is the number of hops that UE can do within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 4.6.2.6-1, and RRTFH is the retuning time between Rx hops as reported by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop within a time window instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the time window instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 4.6.2.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Thop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz, 60kHz in FR2, 120kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max,i is the maximum number of Rx hops signaled as reported by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within the time window occasion, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap and Nhopsslot is the applicable number of hops per slot as defined in Table 4.6.2.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap

where

BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18.as specified in [34]

Nhop,i,j is number of hops within a single time window as defined above

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

<End of Change 1>

<Start of Change 2>

4.6.3.6	Measurement Period Requirements with RX FH

When physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], and UE is configured to perform PRS-RSRP measurement with RX FH via nr-DL-PRS-RxHoppingRequest-r18 as defined in TS 37.355 [34], the UE shall be able to measure multiple (up to the UE capability specified in 4.6.3.3) PRS-RSRP measurements, defined in TS38.215 [4], during the measurement period TPRS-RSRP,Total defined in Clause 4.6.3.5 with using the following definition for Nsample and Lavailable_PRS,i.

Nsample is the number of PRS-RSRP measurement samples, where

-	Nsample= 2 if the RedCap UE supports reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

A measurement sample under RX FH is defined as a PRS measurement over multiple hops.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i=j=1JNhop,i,j*Lper-hop,i,j Nhop,i×Lper-hop,i

where,

-	Nhop,i,j is the number of hops that UE can perform within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 4.6.3.6-1, and RRTFH is the retuning time between Rx hops indicated by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the measurement time window, defined in clause 4.6.1, taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 4.6.3.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz in FR1, and 60kHz, 120kHz in FR2 | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window Nhop is defined as

Nhopi,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max,i is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i.

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within a single time window, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 4.6.3.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap

where

BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in [34].

Nhop,i,j is number of hops within a single time window as defined above.

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

<End of Change 2>

<Start of Change 3>

#### 5.6A.4.6	Measurement Period Requirement with RX FH

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure DL RSTD measurement with RX FH via nr-DL-PRS-RxHoppingRequest-r18 as defined in TS 37.355 [34], the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in Clause 5.6A.4.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in Clause 5.6A.4.5 with using the following definition for Nsample and Lavailable_PRS,i.

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

A measurement sample under RX FH is defined as a PRS measurement over multiple hops.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i×Lper-hop,i

where,

-	Nhop,i,j is the number of hops that UE can perform within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 5.6A.4.6-1, and RRTFH is the RF retuning time between Rx hops indicated by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the measurement time window, defined in clause 5.6A.1, taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table5.6A.4.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz in FR1, and 60kHz, 120kHz in FR2 | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window Nhop is defined as

Nhop=minNhops,effect,N3iLper-hop,i,j,Nhop,max

where

-	Nhop,max is the maximum number of Rx hops as reported by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i ,

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within a single time window, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 5.6A.4.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap

where

BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18.as specified in [34]

Nhop,i,j is number of hops within a single time window as defined above

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

<End of Change 3>

<Start of Change 4>

#### 5.6A.5.6	Measurement Period Requirement with RX FH

When physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], and UE is configured to perform PRS-RSRP measurement with RX FH via nr-DL-PRS-RxHoppingRequest-r18 as defined in TS 37.355 [34], the UE shall be able to measure multiple (up to the UE capability specified in 5.6A.5.3) PRS-RSRP measurements, defined in TS38.215 [4], during the measurement period TPRS-RSRP,Total defined in Clause 5.6A.5.5 with using the following definition for Nsample and Lavailable_PRS,i.

Nsample is the number of PRS-RSRP measurement samples, where

-	Nsample= 2 if the RedCap UE supports reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

A measurement sample under RX FH is defined as a PRS measurement over multiple hops.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i×Lper-hop,i

where,

-	Nhop,i,j is the number of hops that UE can perform within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in a time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 5.6A.5.6-1, and RRTFH is the retuning time between Rx hops indicated by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the measurement time window, defined in clause 5.6A.1, taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table5.6A.5.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz in FR1, and 60kHz, 120kHz in FR2 | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window Nhop is defined as

Nhop=minNhops,effect,N3iLper-hop,i,j,Nhop,max

where

-	Nhop,max is the maximum number of Rx hops as reported by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i ,

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within a single time window, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 5.6A.5.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap

where

BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18.as specified in [34]

Nhop,i,j is number of hops within a single time window as defined above

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

<End of Change 4>

<Start of Change 5>

#### 5.6A.6.6	Measurement Period Requirements with RX FH

When physical layer receives NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], and UE is configured to perform PRS-RSRP measurement with RX FH via nr-DL-PRS-RxHoppingRequest-r18 as defined in TS 37.355 [34], the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in Clause 5.6A.6.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4], during the measurement period TUE RxTx,  Total defined in Clause 5.6.4.5 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

A measurement sample under RX FH is defined as a PRS measurement over multiple hops.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i×Lper-hop,i

where,

-	Nhop,i,j is the number of hops that UE can perform within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 5.6A.6.6-1, and RRTFH is the retuning time between Rx hops indicated by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the measurement time window, defined in clause 5.6A.1, taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table5.6A.6.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz in FR1, and 60kHz, 120kHz in FR2 | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window Nhop is defined as

Nhop=minNhops,effect,N3iLper-hop,i,j,Nhop,max

where

-	Nhop,max is the maximum number of Rx hops as reported by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i ,

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within a single time window, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 5.6A.6.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap

where

BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18.as specified in [34]

Nhop,i,j is number of hops within a single time window as defined above

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

<End of Change 5>