3GPP TSG-RAN5 Meeting #104	R5-244696

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3287 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of PDSCH-Config for DCI format 1_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | DCI format 1-3 configuration is need to be added in PDSCH-config.It is used in singleDCI test case 7.1.1.2.8.x, 7.1.1.2.9.x, 7.1.1.7.2.x, 7.1.1.7.3.x, 7.1.1.7.4.x and 7.1.1.7.5.x. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add pdsch-ConfigDCI-1-3-r18 in PDSCH-config. When the PDSCH is scheduled by DCI 1-3, pdsch-ConfigDCI-1-3-r18 will be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MC_enh-UEConTest WI is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	PDSCH-Config

Table 4.6.3-100: PDSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPDSCH | 0 |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeB | Not present |  |  |
| tci-StatesToAddModList | Not present |  | MBS_Multicast |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1 entry |  |  |
| TCI-State[1] | TCI-State | entry 1 |  |
| } |  |  |  |
| tci-StatesToReleaseList | Not present |  |  |
| vrb-ToPRB-Interleaver | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
|  | resourceAllocationType0 |  | Used_for_Type0 |
| pdsch-TimeDomainAllocationList | Not present |  |  |
| pdsch-AggregationFactor | Not present |  |  |
| rateMatchPatternToAddModList | Not present |  |  |
| rateMatchPatternToReleaseList | Not present |  |  |
| rateMatchPatternGroup1 | Not present |  |  |
| rateMatchPatternGroup2 | Not present |  |  |
| rbg-Size | config1 |  |  |
| mcs-Table | Not present | qam64 per default |  |
| maxNrofCodeWordsScheduledByDCI | Not present |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | wideband |  |  |
| } |  |  |  |
| } |  |  |  |
| zp-CSI-RS-ResourceToAddModList | Not present |  |  |
| zp-CSI-RS-ResourceToReleaseList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| p-ZP-CSI-RS-ResourceSet | Not present |  |  |
| advancedReceiver-MU-MIMO-r18 | Not present |  |  |
| pdsch-ConfigDCI-1-3-r18 | Not present |  |  |
| pdsch-ConfigDCI-1-3-r18 CHOICE { |  |  | DCI_1_3 |
| setup SEQUENCE { |  |  |  |
| resourceAllocationDCI-1-3-r18 | resourceAllocationType1 |  |  |
| rbg-SizeDCI-1-3-r18 | config2 |  |  |
| resourceAllocationType1GranularityDCI-1-3-r18 | Not present | granularity is 1 PRB |  |
| numberOfBitsForRV-DCI-1-3-r18 | 2 |  |  |
| harq-ProcessNumberSizeDCI-1-3-r18 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Used_for_Type0 | Used for RF performance test cases |
| MBS_Multicast | Used for MBS Multicast reception |
| DCI_1_3 | Transmission of DCI_1_3 is required |

<End of modified section 1>