3GPP TSG-RAN5 Meeting #105	R5-246535r2

# Orlando, United States, 18th - 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.551 | CR | 0030 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Charging Cable Verification Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, ETS-Lindgren |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MASC test cannot be performed without a verification test procedure to demonstrate that the impact of a charging cable is negligible |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a verification test procedure in Clause G.4. The goal for the verification check is to make sure that the sensitivities with and without charging cable for three different ranges are within [1.5]dB. Checking the sensitivity in each range (A: yellow, B: green, C: cyan) illustrated below would increase confidence that none of the antenna modules was covered by the charging cable. If only one test point was verified, there is a residual risk that all but one module could be covered by the charging cable. Test procedure moved from Clause G.2.3 to Clause 7.1.2.4.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MASC test cannot be performed with a charging cable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.4.2, E.3, G.2.3, G.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR to 38.508-2 in R5-246534 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-246535r1: remove the addition of 38.508-2 as reference [11] in this CR and add the corresponding reference as reference [12] in R5-246536r2 instead to address overlap with R5-247421 and R5-246536 R5-246535r2: remove changes over changes in moved content in clause 7.1.2.4.2, addition of clauses for better clarity on where changes are made, removal of comment |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

### 7.1.1	MIMO Average Spherical Coverage (MASC)

<<< Skip Unchanged Sections >>>

7.1.2.4	Test Description

FFS

7.1.2.4.1	Initial Conditions

FFS

7.1.2.4.2	Test Procedure

Before throughput testing, the initial conditions shall be confirmed to reach the correct measurement state for each test case.

1.	Ensure environmental requirements of Annex E are met.

2.	Configure the test system according to Annex E.2 and Annex I.1 for the applicable test case.

3.	Verify the implementation of the channel model as specified in Annex I.3.

4.	Position the UE in the chamber according to Annex G.3.

5.	Power on the UE.

6.	Set up the connection.

Note:	For step 3, the verification of the channel model implementation is usually performed once for each channel model and for each validation frequency listed in Table I.3.1-1 as part of the laboratory accreditation process, and will remain valid as long as the setup and instruments remain unchanged. Otherwise the channel model validation may need to be performed prior to starting each throughput test.

For throughput testing, the following steps shall be followed in order to evaluate FR2 MIMO OTA performance of the DUT:

1.	Position the DUT in the default P0 alignment option (Orientation 1), as defined in Clause G.3.

2.	Measure MIMO OTA throughput, the maximum downlink power is TBD. MIMO OTA throughput is the minimum downlink signal power resulting in a pre-defined throughput value (70%) of the maximum theoretical throughput. The downlink signal power step size shall be no more than 0.5 dB when RF power level is near the NR MIMO sensitivity level.

3.	Rotate the UE to the next test point. Table 7.1.2.4.2-1 lists 36 evenly spaced test points determined using the charged particle approach and with test point #1 centred at (0,0).

4.	Repeat the test from step 2 for each specified test point. If the re-positioning concept is applied, the device needs to be positioned in P0 Orientation 2 (either option 1 or option 2), see Clause G.3.

5.	The postprocessing method and the performance metric are defined in Clause 7.1.

Note:	For step 2 of throughput testing, the measurement is not needed to start from maximum downlink power each time. To save testing time, the starting downlink power can be set as a proper value (lower than maximum downlink power supported by test system) as long as the throughput curve can reach at least 70% of the maximum theoretical throughput.

Table 7.1.2.4.2-1: Evenly spaced FR2 test points with a constant density

| Test Point Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 0.0 | 0.0 |
| 2 | 33.5 | 139.7 |
| 3 | 33.9 | 49.7 |
| 4 | 35.5 | -142.9 |
| 5 | 35.5 | -76.9 |
| 6 | 37.6 | -17.2 |
| 7 | 52.3 | 94.7 |
| 8 | 56.9 | 175.7 |
| 9 | 62.5 | 20.4 |
| 10 | 63.7 | -99.8 |
| 11 | 67.1 | -55.0 |
| 12 | 69.3 | -139.5 |
| 13 | 69.5 | 130.1 |
| 14 | 70.3 | 60.8 |
| 15 | 72.1 | -16.2 |
| 16 | 88.7 | -167.5 |
| 17 | 88.7 | 98.5 |
| 18 | 89.3 | 157.0 |
| 19 | 93.9 | -78.9 |
| 20 | 94.6 | 31.6 |
| 21 | 95.3 | -115.6 |
| 22 | 99.6 | -38.3 |
| 23 | 103.8 | -1.1 |
| 24 | 104.4 | 66.3 |
| 25 | 110.1 | 127.5 |
| 26 | 115.1 | -145.6 |
| 27 | 120.8 | 171.9 |
| 28 | 125.3 | -60.7 |
| 29 | 128.2 | -104.1 |
| 30 | 128.8 | 91.3 |
| 31 | 129.9 | 35.8 |
| 32 | 136.0 | -13.4 |
| 33 | 145.8 | 138.1 |
| 34 | 150.2 | -153.3 |
| 35 | 160.6 | -67.4 |
| 36 | 161.7 | 59.1 |

<<< Skip Unchanged Sections >>>

# E.3	Operating voltage

For FR1 MIMO OTA, all test cases shall be performed in the normal voltage condition with the DUT operated in stand-alone battery powered mode. No extreme voltage testing is required. It is recommended to start testing with a fully charged battery and conclude and/or pause testing before the battery has completely lost its charge.

For FR2 MIMO OTA, all test cases shall be performed in the normal voltage condition with the DUT operated in stand-alone battery powered mode or external power source. No extreme voltage testing is required. As outlined in Clause G.4, it shall be demonstrated that the impact of external power source to device performance is negligible compared to stand-alone battery powered mode. It is recommended to start testing with a fully charged battery and conclude and/or pause testing before the battery has completely lost its charge.

<<< Skip Unchanged Sections >>>

## G.2.1	System setup

The 3D MPAC test method is the reference methodology for FR2 NR MIMO OTA testing. By arranging an array of antennas around the Equipment Under Test (EUT), a spatial distribution of angles of arrival in the 3D MPAC system may be simulated to expose the EUT to a near field environment that appears to have originated from a complex multipath far field environment.

As illustrated schematically in Figure G.2.1-1, signals propagate from the base station/communication tester to the EUT through a simulated multipath environment known as a spatial channel model, where appropriate channel impairments such as Doppler and fading are applied to each path prior to injecting all of the directional signals into the chamber simultaneously through the probe array. The resulting field distribution in the test zone is then integrated by the EUT antenna(s) and processed by the receiver(s) just as it would do so in any non-simulated multipath environment. The 3D MPAC system with 6 dual-polarized probes (illustrated with black dots in Figure G.2.1-1) placed on a sector with minimum radius of 0.75m from the centre of the test zone is permitted for NR FR2 MIMO OTA testing.

Figure G.2.1-1: 3D MPAC system layout for NR FR2 MIMO OTA testing

The exact probe locations with respect to the OTA test system coordinate system are tabulated in Table G.2.1-1.

Table G.2.1-1: FR2 3D MPAC Probe Locations in OTA test system coordinate system

| Probe Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 0.0 | 0.0 |
| 2 | 11.2 | 116.7 |
| 3 | 20.6 | -104.3 |
| 4 | 20.6 | 104.3 |
| 5 | 20.6 | 75.7 |
| 6 | 30.0 | 90.0 |

The 3D MPAC probes in Table G.2.1-1 can be implemented using conventional millimetre-wave probes as well as IFF-based probes as long as the same probe configuration and same number of probes is used.

The channel model parameters and probe locations for channel model implementation are defined in a channel model coordinate system, which is illustrated in figure G.2.1-2. The channel model coordinate axes xCM, yCM, and zCM correspond to the OTA test system coordinate axes z, y, and -x, respectively.

Figure G.2.1-2: Channel Model Coordinate Axes in FR2 3D-MPAC system

The probe locations with respect to channel model coordinate axes are tabulated in table G.2.1-2.

Table G.2.1-2: FR2 3D MPAC Probe Locations in Channel Model Coordinate System

| Probe Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 90 | 0 |
| 2 | 85 | 10 |
| 3 | 85 | -20 |
| 4 | 85 | 20 |
| 5 | 95 | 20 |
| 6 | 90 | 30 |

The channel model rotations assumed for this probe configuration are tabulated in Table G.2.1-3.

Table G.2.1-3: Channel Model Rotations

| UMi CDL-C |  |
| --- | --- |
| Phi [deg] | Theta [deg] |
| 32 | 15.0 |

This channel model rotation assumes the relative orientations of BS and UE antennas displayed in Figure G.2.1-3, i.e., the DUT antenna is pointed towards the BS in channel model coordinate system.

In order to avoid positioning ambiguities, the turntable implementing the rotation in  shall match the intended DUT  for P0 Orientation 1 without the re-positioning approach, as defined in Clause G.3, applied. With the re-positioning approach applied, the relative orientation between the DUT and the probes for P0 Orientation 2 shall be the same the relative orientation between DUT and probes as for P0 Orientation 1.

Figure G.2.1-3: Relative orientations of BS and UE antennas

Since the test points are uniformly spaced in 3D already, Table 7.1.2.4.2-1, there is no need to adjust/rotate the DUT rotations by the channel model rotations.

<<< Skip Unchanged Sections >>>

## G.2.3	VOID

Before throughput testing, the initial conditions shall be confirmed to reach the correct measurement state for each test case.

1.	Ensure environmental requirements of Annex E are met.

2.	Configure the test system according to Annex E.2 and Annex I.1 for the applicable test case.

3.	Verify the implementation of the channel model as specified in Annex I.3.

4.	Position the UE in the chamber according to Annex G.3.

5.	Power on the UE.

6.	Set up the connection.

Note:	For step 3, the verification of the channel model implementation is usually performed once for each channel model as part of the laboratory accreditation process, and will remain valid as long as the setup and instruments remain unchanged. Otherwise the channel model validation may need to be performed prior to starting each throughput test.

For throughput testing, the following steps shall be followed in order to evaluate FR2 MIMO OTA performance of the DUT:

1.	Position the DUT in the default P0 alignment option (Orientation 1), as defined in Annex A.3 in TR38.827 [10].

2.	Measure MIMO OTA throughput, the maximum downlink power is TBD. MIMO OTA throughput is the minimum downlink signal power resulting in a pre-defined throughput value (70%) of the maximum theoretical throughput. The downlink signal power step size shall be no more than 0.5 dB when RF power level is near the NR MIMO sensitivity level.

3.	Rotate the UE to the next test point. Table G.2.3-1 lists 36 evenly spaced test points determined using the charged particle approach and with test point #1 centred at (0,0).

4.	Repeat the test from step 2 for each specified test point. If the re-positioning concept is applied, the device needs to be positioned in P0 Orientation 2 (either option 1 or option 2).

5.	The postprocessing method and the performance metric are defined in Clause 7.1.

Note:	For step 2 of throughput testing, the measurement is not needed to start from maximum downlink power each time. To save testing time, the starting downlink power can be set as a proper value (lower than maximum downlink power supported by test system) as long as the throughput curve can reach at least 70% of the maximum theoretical throughput.

Table G.2.3-1: Evenly spaced FR2 test points with a constant density

<<< Skip Unchanged Sections >>>

# G.4	Use of Charging Cable

The DUT may be tested with a charging cable for any band and channel if declared by the vendor and proper cable routing instructions are provided, as outlined in Table A.4.3.9-15 [12].

The following verification tests shall be performed in each band that utilized the charging cable:

1. Identify test points in test point ranges A, B, and C defined in Table G.4-1 that yielded the best sensitity within each range when performed with the charging cable, i.e., PA,cable, PB,cable, PC,cable after the executing test procedure described in 7.1.2.4.2 with a charging cable.

Table G.4-1: Verification test points for charging cable

| Test Point Range | f | q | Test Poins in Test Point Range, see Table 7.1.2.4.2 |
| --- | --- | --- | --- |
| A | 0°≤f<120° | 45°≤q<135° | 7, 9, 14, 17, 20, 24, 30, 31 |
| B | 120°≤f<240° |  | 8, 12, 13, 16, 18, 25, 26, 27 |
| C | 240°≤f<360° |  | 10, 11, 15, 19, 21, 22, 23, 28, 29 |

2. For each of the test points in ranges A, B, and C identified in step 1, perform sensitivity measurements with the standalone battery (with the charging cable removed), i.e., PA,standalone, PB, standalone, PC, standalone

3. Determine whether all sensitivities performed in ranges A, B, and C are within [1.5]dB, i.e.,

|PA,cable – PA,standalone| ≤ [1.5]dB	(4.1)

|PB,cable – PB,standalone| ≤ [1.5]dB	(4.2)

|PC,cable – PC,standalone| ≤ [1.5]dB	(4.3)

4. If the all 3 conditions in step 3 are met, the MASC results performed with the charging cable are applicable for each band. However, if any of the conditions in step 3 are not met, the MASC test procedure in Clause 7.1.2.4.2 shall be repeated with the standalone battery, i.e., without the charging cable, for the respective band.

If a charging cable was used, it is recommended to follow the test sequences as outlined below:

i. Perform the MASC test, Clause 7.1.2.4.2, while following the guidelines of the vendor declaration outlined in Table A.4.3.9-15 [12].

ii. Remove the charging cable with as few changes to the device and test setup as possible.

iii. Perform the verification tests above, Steps 1 through 4,  after the conclusion of the MASC tests, step i, and the removal of the charging cable, step ii

<<< END OF CHANGES >>>