3GPP TSG-RAN5 Meeting #105	R5-246588r1

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4722 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of XR test case 7.1.3.5.12 - PDCP SN gap report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case is added to test the UE behaviour when it receive an PDCP SN gap report from NW. After receiving PDCP SN gap Report, UE update RX_DELIV when (1) RX_DELIV is equal to any COUNT associated with the discarded PDCP SDU or (2) RX_DELIV is equal to COUNT associated with the received PDCP SDU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new XR test case 7.1.3.5.12 PDCP SN gap report / Receive operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_XR_enh_plus_CT1-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.12 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0523 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Clarify that “PDCP SDU # & #4” means “PDCP SDU #3 and SDU #4” - Delete the “stop t-Reordering” from the check point in Step 12 and Step18 because SS cannot observe UE internal timer stop. - Update TP 1-5 and add TP6 (checked in step21-24 ) to check that UE can correct updating RX_NEXT upon reception fo PDCP SN Gap Report. - Add check in step 8 because it is part of TP2. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.3.5.12	PDCP SN gap report / Receive operation

7.1.3.5.12.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is running }

ensure that {

when { UE receives PDCP SN gap Report, RX_DELIV is less than any COUNT value associated with the discarded PDCP SDUs but not equal to any COUNT value associated with the discarded PDCP SDUs }

then { UE does not update RX_DELIV, and UE does not deliver any stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1 to upper layers }

}

(2)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is running }

ensure that {

when { t-Reordering expires }

then { UE updates RX_DELIV to the COUNT value of the first PDCP SDU with COUNT >= RX_REORD which has not been delivered to upper layers and is not considered as discarded, and UE deliver to the upper layers all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD and all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded. }

}

(3)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is running }

ensure that {

when { UE receives PDCP SN gap Report, RX_DELIV is equal to any COUNT value associated with the discarded PDCP SDUs }

then { UE deliver all stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1 to upper layers where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded, and then updates RX_DELIV }

}

(4)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is not running }

ensure that {

when { UE receives PDCP SN gap Report, RX_DELIV is less than any COUNT value associated with the discarded PDCP SDUs but not equal to any COUNT value associated with the discarded PDCP SDUs, and RX_DELIV < RX_NEXT after updating RX_NEXT }

then { UE starts t-reordering and UE does not update RX_DELIV }

}

(5)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is running }

ensure that {

when { UE receives PDCP DATA COUNT = RX_DELIV }

then { UE deliver to the upper layers all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded, and then UE updates RX_DELIV to the COUNT value of the first PDCP SDU with COUNT value > RX_DELIV which has not been delivered to upper layers and is not considered as discarded.}

}

(6)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is not running }

ensure that {

when { UE receives PDCP SN gap Report, RX_DELIV is less than any COUNT value associated with the discarded PDCP SDUs but not equal to any COUNT value associated with the discarded PDCP SDUs, and RX_NEXT <= COUNT value associated with the last discarded PDCP SDU indicated in the PDCP SN gap report }

then { UE update RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU + 1 }

}

7.1.3.5.12.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.323, clause 5.2.2.1, 5.2.2.2, 5.16.2, 6.1.2, 6.2.3.5, 6.3.8, 6.3.15 and 6.3.16. Unless otherwise stated these are Rel-18 requirements.

[TS 38.323, clause 5.2.2.1]

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

…

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV;

[TS 38.323, clause 5.2.2.2]

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value >= RX_REORD;

[TS 38.323, clause 5.16.2]

At reception of a PDCP SN gap report from lower layers, the receiving PDCP entity shall:

-	consider each PDCP SDU, if any, with the bit in the discard bitmap set to '1', or with the associated COUNT value equal to the value of FDC field as discarded;

-	if RX_DELIV is less than or equal to the largest COUNT value associated with the discarded PDCP SDUs:

-	if RX_NEXT <= COUNT value associated with the last discarded PDCP SDU indicated in the PDCP SN gap report:

-	update RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU + 1;

-	if RX_DELIV is equal to any COUNT value associated with the discarded PDCP SDU(s):

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 6.1.2]

The PDCP Control PDU is used to convey one of followings in addition to the PDU header:

…

-	a PDCP SN gap report.

[TS 38.323, clause 6.2.3.5]

Figure 6.2.3.5-1 shows the format of the PDCP control PDU carrying the PDCP SN gap report. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.3.5-1: PDCP Control PDU format for PDCP SN gap report

[TS 38.323, clause 6.3.8]

Length: 3 bits

This field indicates the type of control information included in the corresponding PDCP Control PDU.

Table 6.3.8-1: PDU type

| Bit | Description |
| --- | --- |
| 000 | PDCP status report |
| 001 | Interspersed ROHC feedback |
| 010 | EHC feedback |
| 011 | UDC feedback |
| 100 | PDCP SN gap report |
| 101-111 | Reserved |

[TS 38.323, clause 6.3.15]

Length: 32 bits

First discarded COUNT. This field indicates the smallest COUNT value among the COUNT value(s) associated with the discarded PDCP SDU(s).

[TS 38.323, clause 6.3.16]

Length: Variable. The length of the discard bitmap field can be 0.

This field indicates which SDUs are discarded and which SDUs are not discarded in the transmitting PDCP entity. The bit position of the Nth bit in the Discard Bitmap is N, i.e., the bit position of the first bit in the Discard Bitmap is 1.

Table 6.3.16-1 Discard Bitmap

| Bit | Description |
| --- | --- |
| 0 | PDCP SDU with COUNT = (FDC + bit position) modulo 232 is not discarded. |
| 1 | PDCP SDU with COUNT = (FDC + bit position) modulo 232 is discarded. |

7.1.3.5.12.3	Test description

7.1.3.5.12.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 exception of PDCP parameters according to Table 7.1.3.5.12.3.1-1.

Table 7.1.3.5.12.3.1-1: PDCP parameters

| t-Reordering | ms500 |
| --- | --- |

7.1.3.5.12.3.2	Test procedure sequence

Table 7.1.3.5.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends the PDCP SDU #0 D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | (PDCP SDU #0) | - | - |
| 2 | The UE transmits a PDCP SDU #0 with PDCP SN=0. | --> | (PDCP SDU #0) | - | - |
| 3 | The SS sends the PDCP SDU #2 D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. The UE starts t-Reordering. Note T1. | <-- | (PDCP SDU #2) | - | - |
| 4 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 5 | The SS sends the PDCP SN gap Report indicating PDCP SDU #3 and SDU #4 are discarded. D/C field = 0 (PDCP Control PDU), PDU Type field = 100 (PDCP SN gap report), FDC field = 3 and Discard Bitmap1 field = 00000001’B. | <-- | (PDCP SN gap Report) | - | - |
| 6 | Check 1: Does the UE transmit the PDCP SDU #2 with PDCP SN=1 after t-Reordering expiry? Note T2 Check 2: Is (T2 – T1) > t-Reordering? | --> | (PDCP SDU #2) | 1,2 | P |
| 7 | The SS sends the PDCP SDU #5 D/C field = 1 (PDCP Data PDU) and PDCP SN = 5. | <-- | (PDCP SDU #5) | - | - |
| 8 | Check: Does the UE transmit a PDCP SDU #5 with PDCP SN=2? Note: UE updates RX_DELIV to 5 after step6. | --> | (PDCP SDU #5) | 2 | P |
| 9 | The SS sends the PDCP SDU #7 D/C field = 1 (PDCP Data PDU) and PDCP SN = 7. The UE starts t-Reordering. Note T3. | <-- | (PDCP SDU #7) | - | - |
| 10 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 11 | The SS sends the PDCP SN gap Report indicating PDCP SDU #3 and SDU #4 and SDU #6 and SDU #8 are discarded. D/C field = 0 (PDCP Control PDU), PDU Type field = 100’B (PDCP SN gap report), FDC field = 3 and Discard Bitmap1 field = 00010101’B. | <-- | (PDCP SN gap Report) | - | - |
| 12 | Check 1: Does the UE transmit the PDCP SDU #7 with PDCP SN=3? Note T4 Check 2: Is (T4 – T3) < t-Reordering? | --> | (PDCP SDU #7) | 3 | P |
| 13 | The SS sends the PDCP SDU #8 D/C field = 1 (PDCP Data PDU) and PDCP SN = 8. | <-- | (PDCP SDU #8) | - | - |
| 14 | Check: Does the UE transmit the PDCP SDU in next 1s? Note: UE shall not transmit PDCP SDU #8 because RX_DELIV is updated to 9 after step 11. | --> | (PDCP SDU) | 3 | F |
| 15 | The SS sends the PDCP SN gap Report indicating PDCP SDU #3 and SDU #4 and SDU #6 and SDU #8 and SDU #10 are discarded. D/C field = 0 (PDCP Control PDU), PDU Type field = 100’B (PDCP SN gap report), FDC field = 3 and Discard Bitmap1 field = 01010101’B. The UE starts t-Reordering. Note T5. | <-- | (PDCP SN gap Report) | - | - |
| 16 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 17 | The SS sends the PDCP SDU #9 D/C field = 1 (PDCP Data PDU) and PDCP SN = 9. | <-- | (PDCP SDU #9) | - | - |
| 18 | Check 1: Does the UE transmit the PDCP SDU #9 with PDCP SN=4? Note T6 Check 2: Is (T6 – T5) < t-Reordering? | --> | (PDCP SDU #9) | 4,5 | P |
| 19 | The SS sends the PDCP SDU #10 D/C field = 1 (PDCP Data PDU) and PDCP SN = 10. | <-- | (PDCP SDU #10) | - | - |
| 20 | Check: Does the UE transmit the PDCP SDU in next 1s? Note: UE shall not transmit PDCP SDU #10 because RX_DELIV is updated to 11 after step 17. | --> | (PDCP SDU) | 5 | F |
| 21 | The SS sends the PDCP SN gap Report indicating PDCP SDU #3 and SDU #4 and SDU #6 and SDU #8 and SDU #10 and SDU #12 are discarded. D/C field = 0 (PDCP Control PDU), PDU Type field = 100’B (PDCP SN gap report), FDC field = 3 and Discard Bitmap1 field = 0101010101’B. The UE starts t-Reordering. | <-- | (PDCP SN gap Report) | - | - |
| 22 | Wait for 600ms (>configured t-Reordering) | - | - | - | - |
| 23 | The SS sends the PDCP SDU #11 D/C field = 1 (PDCP Data PDU) and PDCP SN = 11. | <-- | (PDCP SDU #11) | - | - |
| 24 | Check: Does the UE transmit the PDCP SDU in next 1s? Note: UE shall not transmit PDCP SDU #11, because RX_NEXT is updated to 13 after step 21, and then t-Reordering started due to RX_NEXT > RX_DELIV. When t-Reordering expire, RX_DELIV is updated to 13. | --> | (PDCP SDU) | 6 | F |

7.1.3.5.12.3.3	Specific message contents

None.

<End of modified section 1>