3GPP TSG-RAN5 Meeting #105	R5-246903r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4764 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 12.3.1.2.10 Sidelink Relay / L2 U2N / Relay UE / RRC connection resume trggered by Remote UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new test case 12.3.1.2.10 Sidelink Relay / L2 U2N / Relay UE / RRC connection resume trggered by Remote UE has been endorsed within the work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 12.3.1.2.10 Sidelink Relay / L2 U2N / Relay UE / RRC connection resume trggered by Remote UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.2.10(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0532 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Fixed some editorial errors. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.3.1.2.10	Sidelink Relay / L2 U2N / Relay UE / RRC connection resume triggered by Remote UE

12.3.1.2.10.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE and acting as a L2 U2N Relay UE }

ensure that {

when { UE receives message from a L2 U2N Remote UE via SL-RLC0 }

then { UE initiates RRC connection resume procedure }

}

(2)

with { UE in NR RRC_INACTIVE and acting as a L2 U2N Relay UE and initiating RRC connection resume triggered by reception of RRCSetupRequest from a L2 U2N Remote UE via SL-RLC0 }

ensure that {

when { The establishmentCause is not emergency, mps-PriorityAccess, or mcs-PriorityAccess of the received RRCSetupRequest from L2 U2N Remote UE }

then { UE transmits an RRCResumeRequest message with resumeCause which is not any of emergency, mps-PriorityAccess, or mcs-PriorityAccess }

}

(3)

with { UE in NR RRC_INACTIVE and acting as a L2 U2N Relay UE and initiating RRC connection resume triggered by reception of RRCResumeRequest from a L2 U2N Remote UE via SL-RLC0 }

ensure that {

when { The resumeCause is not emergency, mps-PriorityAccess, or mcs-PriorityAccess of the received RRCResumeRequest from L2 U2N Remote UE }

then { UE transmits an RRCResumeRequest message with resumeCause which is not any of emergency, mps-PriorityAccess, or mcs-PriorityAccess }

}

12.3.1.2.10.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.13.1a, 5.3.13.2, 5.3.8.3 and 5.3.11. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.13.1a]

For L2 U2N Relay UE in RRC_INACTIVE, an RRC connection establishment is resumed in the following cases:

1>	if any message is received from the L2 U2N Remote UE via SL-RLC0 as specified in 9.1.1.4 or SL-RLC1 as specified in 9.2.4;

[TS 38.331, clause 5.3.13.2]

Upon initiation of the procedure, the UE shall:

…

NOTE 2:	In case the L2 U2N Relay UE initiates RRC connection resume triggered by reception of message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1 as specified in 5.3.13.1a, the L2 U2N Relay UE sets the resumeCause by implementation, but it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as resumeCause, if the same cause value in the message received from the L2 U2N Remote UE via SL-RLC0.

[TS 38.331, clause 5.3.8.3]

The UE shall:

…

1>	if the RRCRelease includes suspendConfig:

…

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUE-Config, if configured;

-	sl-L2RemoteUE-Config, if configured;

NOTE 1c:	suspendConfig is not stored as part of UE Inactive AS Context, except for the fields explicitly specified.

3>	store any previously or subsequently received application layer measurement report containers for which no segment, or full message, has been submitted to lower layers for transmission;

NOTE 2:	NR sidelink communication/discovery related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

2>	release Uu Relay RLC channel(s), if configured;

2>	release PC5 Relay RLC channel(s), if configured;

2>	release the SRAP entity, if configured;

NOTE 2a:	A L2 U2N Relay UE may re-establish the SL-RLC0, SL-RLC1 and SRAP entity after release.

…

1>	else:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

[TS 38.331, clause 5.3.11]

The UE shall:

…

1>	release all radio resources, including release of the RLC entity, the BAP entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs (except for broadcast MRBs), BH RLC channels, Uu Relay RLC channels, PC5 Relay RLC channels and SRAP entity;

NOTE 0:	A L2 U2N Relay UE may re-establish the SL-RLC0, SL-RLC1 and SRAP entity after release.

12.3.1.2.10.3	Test Description

12.3.1.2.10.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1.

-	System information combination NR-31 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used.

-	NR-SS-UE

-	NR-SS-UE1 operating as L2 U2N Remote UE.

-	NR-SS-UE1 is synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.4 and ProSeP using condition Announcing_Restricted_Model_A as per TS 38.508-1 [4] clause 4.7C.1.

- UE is synchronised on GNSS.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

12.3.1.2.10.3.2	Test procedure sequence

Table 12.3.1.2.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side in Table 4.9.41.2.2-1 in TS 38.508-1[4] is performed. | - | - | - | - |
| 2 | The SS-NW transmits an RRCRelease message to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: RRCRelease | - | - |
| 3 | The UE relays the RRCRelease message received to NR-SS-UE1. | --> | PC5 Relay RLC Channel: RRCRelease | - | - |
| 4 | The NR-SS-UE1 transmits an RemoteUEInformationSidelink message with sl-PagingInfo-RemoteUE set to setup. | <-- | PC5 RRC: RemoteUEInformationSidelink | - | - |
| 5 | The SS-NW transmits an RRCRelease message including suspendConfig to Relay UE. | <-- | NR RRC: RRCRelease | - | - |
| 6 | The NR-SS-UE1 transmits an RRCSetupRequest message with establishmentCause set to mo-Signalling via Relay UE. | <-- | PC5 Relay RLC Channel: RRCSetupRequest | - | - |
| 7 | Check: Does the UE transmit a RRCResumeRequest message with resumeCause not set to emergency, mps-PriorityAccess, or mcs-PriorityAccess message? | --> | NR RRC: RRCResumeRequest | 1,2 | P |
| 8 | The SS-NW transmits an RRCResume message to L2 U2N Relay UE. | <-- | NR RRC: RRCResume |  |  |
| 9 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete |  |  |
| 10-12 | Steps 4 to 6 of the procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side in Table 4.9.41.2.2-1 in TS 38.508-1[4] is performed. | - | - | - | - |
| 13 | The UE relays the RRCSetupRequest message received at step 6 to SS-NW. | --> | Uu Relay RLC Channel: RRCSetupRequest | - | - |
| 14 | The SS-NW transmits an RRCSetup message to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: RRCSetup | - | - |
| 15 | The UE relays the RRCSetup message received to NR-SS-UE1. | --> | PC5 Relay RLC Channel: RRCSetup | - | - |
| 16 | The NR-SS-UE1 transmits an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment via Relay UE. | <-- | PC5 Relay RLC Channel: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 17 | The UE relays the RRCSetupComplete message received to SS-NW. | --> | Uu Relay RLC Channel: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 18 | The NR-SS-UE1 transmits an RemoteUEInformationSidelink message with sl-PagingInfo-RemoteUE set to release. | <-- | PC5 RRC: RemoteUEInformationSidelink | - | - |
| 19 | The SS-NW transmits an SecurityModeCommand message to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: SecurityModeCommand | - | - |
| 20 | The UE relays the SecurityModeCommand message received to NR-SS-UE1. | --> | PC5 Relay RLC Channel: SecurityModeCommand | - | - |
| 21 | The NR-SS-UE1 transmits an SecurityModeComplete message via Relay UE. | <-- | PC5 Relay RLC Channel: SecurityModeComplete | - | - |
| 22 | The UE relays the SecurityModeComplete message received to SS-NW. | --> | Uu Relay RLC Channel: SecurityModeComplete | - | - |
| 23 | The SS-NW transmits an RRCRelease message including suspendConfig to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: RRCRelease | - | - |
| 24 | The UE relays the RRCRelease message including suspendConfig received to NR-SS-UE1. | --> | PC5 Relay RLC Channel: RRCRelease | - | - |
| 25 | The NR-SS-UE1 transmits an RemoteUEInformationSidelink message with sl-PagingInfo-RemoteUE set to setup. | <-- | PC5 RRC: RemoteUEInformationSidelink | - | - |
| 26 | The SS-NW transmits an RRCRelease message including suspendConfig to Relay UE. | <-- | NR RRC: RRCRelease | - | - |
| 27 | The NR-SS-UE1 transmits an RRCResumeRequest message with resumeCause set to mo-Signalling via Relay UE. | <-- | PC5 Relay RLC Channel: RRCResumeRequest | - | - |
| 28 | Check: Does the UE transmit a RRCResumeRequest message with resumeCause not set to emergency, mps-PriorityAccess, or mcs-PriorityAccess message? | --> | NR RRC: RRCResumeRequest | 1,3 | P |
| 29 | The SS-NW transmits an RRCResume message to L2 U2N Relay UE. | <-- | NR RRC: RRCResume |  |  |
| 30 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete |  |  |
| 31-33 | Steps 4 to 6 of the procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side in Table 4.9.41.2.2-1 in TS 38.508-1[4] is performed. | - | - | - | - |
| 34 | The UE relays the RRCResumeRequest message received at step 27 to SS-NW. | --> | Uu Relay RLC Channel: RRCResumeRequest | - | - |
| 35 | The SS-NW transmits an RRCResume message to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: RRCResume | - | - |
| 36 | The UE relays the RRCResume message received to NR-SS-UE1. | --> | PC5 Relay RLC Channel: RRCResume | - | - |
| 37 | The NR-SS-UE1 transmits an RRCResumeComplete message via Relay UE. | <-- | PC5 Relay RLC Channel: RRCResumeComplete | - | - |
| 38 | The UE relays the RRCResumeComplete message received to SS-NW. | --> | Uu Relay RLC Channel: RRCResumeComplete | - | - |
| 39 | The NR-SS-UE1 transmits an RemoteUEInformationSidelink message with sl-PagingInfo-RemoteUE set to release. | <-- | PC5 RRC: RemoteUEInformationSidelink | - | - |

12.3.1.2.10.3.3	Specific message contents

Table 12.3.1.2.10.3.3-1: RemoteUEInformationSidelink (Step 4, Table 12.3.1.2.10.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-2A with condition Paging_Setup

Table 12.3.1.2.10.3.3-2: RemoteUEInformationSidelink (Step 18 and 39, Table 12.3.1.2.10.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-2A with condition Paging_Release

Table 12.3.1.2.10.3.3-3: RemoteUEInformationSidelink (Step 25, Table 12.3.1.2.10.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-2A with condition Paging_Setup and NR_RRC_RESUME

Table 12.3.1.2.10.3.3-4: RRCSetupRequest (step 6, Table 12.3.1.2.10.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.3.1.2.10.3.3-5: RRCResumeRequest (step 7, step 27 and step 28, Table 12.3.1.2.10.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mo-Signalling |  | step 27 |
|  | Any value except emergency, mps-PriorityAccess, or mcs-PriorityAccess |  | step 7 and step 28 |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>