3GPP TSG-RAN5 Meeting #105	R5-246951r2

# Orlando, US, 18th - 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1107 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of General ONOFF time mask for CA (3UL CA) & (4UL CA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton International INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A relevante configuration for CA (3UL CA) & (4UL CA) in PRD.21 is ongoing, so test case for CA (3UL CA) & (4UL CA) should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of General ONOFF time mask for CA (3UL CA) & (4UL CA) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Cae 6.3A.3.1.2 & 6.3A.3.1.3 will be still incompleting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.1.2, 6.3A.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 CR 1106 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Annex F.1.2, F.3.2 for MU/TT is updated in R5-246950 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - MU/TT information is noted in column of other comments and redundant clauses is removed from column of clauses affected. R2: - Editor’s note is updated & Table title in test requirement is updated as well |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.3A.3.1.2	General ON/OFF time mask for CA (3UL CA)

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz is TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, 4, 5 and 7.

- This test case is incomplete until a suitable solution for preventing SCell drop is implemented in the test procedure.

6.3A.3.1.2.1	Test purpose

To verify that the general ON/OFF time mask for CA meets the requirements given in 6.3A.3.1.2.5. Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3A.3.1.2.2	Test applicability

The requirements of this test apply to all types of NR UE release 16 and forward supporting 3UL CA.

6.3A.3.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.3.0.

6.3A.3.1.2.4	Test description

Same as in clause 6.3A.3.1.1.4 with following exceptions:

-	Instead of Table 6.3A.3.1.1.4.1-1  use Table 6.3A.3.1.2.4.1-1.

Table 6.3A.3.1.2.4.1-1: Intra-band Contiguous 3UL CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH (NOTE 2) |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest NRB_agg, Highest NRB_agg (≤ 400 MHz aggregated channel bandwidth) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping ( NOTE 4） | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | - |  | DFT-s-OFDM QPSK | Inner Full |
|  | SCC1 |  |  |  |  |  |
|  | SCC2 |  |  |  |  |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Test environment for UE Max TRP is normal only. NOTE 3: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 4: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. NOTE 5: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: “The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier”. |  |  |  |  |  |  |

6.3A.3.1.2.5	Test requirements

The requirement for the power measured in steps 7, 8 and 9 of the test procedure shall not exceed the values specified in Table 6.3A.3.1.2.5-1 and Table 6.3A.3.1.2.5-2.

Table 6.3A.3.1.2.5-1: Test requirement of OFF power of General ON/OFF time mask for 3UL CA

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Transmit OFF power | ≤ -30+TT dBm |  |  |  |
| Transmission OFF Measurement bandwidth | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

Table 6.3A.3.1.2.5-2: Test requirement of ON power of General ON/OFF time mask for 3UL CA

|  | SCS | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | [kHz] | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Transmission ON | 60 | FFS | FFS | FFS | FFS |
| Measured power for CP-OFDM | 120 | FFS | FFS | FFS | FFS |
| Expected Transmission ON power for DFT-s-OFDM | 60 | For intra-band contiguous CA configurations, The test requirement of transmit on power of 3UL CA is the same as Table 6.2A.1.1.2.5-1 as appropriate. |  |  |  |
|  | 120 |  |  |  |  |

Table 6.3A.3.1.2.5-3: Test Tolerance for OFF power for 3UL CA for Power class 3

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | Same as Table 6.3.3.2.5-5 | Same as Table 6.3.3.2.5-5 |

Table 6.3A.3.1.2.5-4: Test Tolerance for ON power for 3UL CA for Power class 3

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | Same as in table Table 6.2A.1.1.2.5-1a, 6.2A.1.1.2.5-1b | Same as in table Table 6.2A.1.1.2.5-1a, 6.2A.1.1.2.5-1b |

6.3A.3.1.3	General ON/OFF time mask for CA (4UL CA)

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz is TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, 4, 5 and 7.

- This test case is incomplete until a suitable solution for preventing SCell drop is implemented in the test procedure.

6.3A.3.1.3.1	Test purpose

To verify that the general ON/OFF time mask for CA meets the requirements given in 6.3A.3.1.3.5. Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3A.3.1.3.2	Test applicability

The requirements of this test apply to all types of NR UE release 16 and forward supporting 4UL CA.

6.3A.3.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.3.0.

6.3A.3.1.3.4	Test description

Same as in clause 6.3A.3.1.1.4 with following exceptions:

-	Instead of Table 6.3A.3.1.1.4.1-1  use Table 6.3A.3.1.3.4.1-1.

Table 6.3A.3.1.3.4.1-1: Intra-band Contiguous 4UL CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH (NOTE 2) |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest NRB_agg, Highest NRB_agg (≤ 400 MHz aggregated channel bandwidth) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping ( NOTE 4） | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | - |  | DFT-s-OFDM QPSK | Inner Full |
|  | SCC1 |  |  |  |  |  |
|  | SCC2 |  |  |  |  |  |
|  | SCC3 |  |  |  |  |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Test environment for UE Max TRP is normal only. NOTE 3: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 4: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. NOTE 5: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: “The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier”. |  |  |  |  |  |  |

6.3A.3.1.3.5	Test requirements

The requirement for the power measured in steps 7, 8 and 9 of the test procedure shall not exceed the values specified in Table 6.3A.3.1.3.5-1 and Table 6.3A.3.1.3.5-2.

Table 6.3A.3.1.3.5-1: Test requirement of OFF power of General ON/OFF time mask for 4UL CA

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Transmit OFF power | ≤ -30+TT dBm |  |  |  |
| Transmission OFF Measurement bandwidth | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

Table 6.3A.3.1.3.5-2: Test requirement of ON power of General ON/OFF time mask for 4UL CA

|  | SCS | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | [kHz] | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Transmission ON | 60 | FFS | FFS | FFS | FFS |
| Measured power for CP-OFDM | 120 | FFS | FFS | FFS | FFS |
| Expected Transmission ON power for DFT-s-OFDM | 60 | For intra-band contiguous CA configurations, The test requirement of transmit on power of 2UL CA is the same as Table 6.2A.1.1.3.5-1 as appropriate. |  |  |  |
|  | 120 |  |  |  |  |

Table 6.3A.3.1.3.5-3: Test Tolerance for OFF power for 4UL CA for Power class 3

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | Same as Table 6.3.3.2.5-5 | Same as Table 6.3.3.2.5-5 |

Table 6.3A.3.1.3.5-4: Test Tolerance for ON power for 4UL CA for Power class 3

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | Same as in table Table 6.2A.1.1.2.5-1a, 6.2A.1.1.2.5-1b | Same as in table Table 6.2A.1.1.2.5-1a, 6.2A.1.1.2.5-1b |

## << End of changes >>