3GPP TSG-RAN5 Meeting #105	R5-247531

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5389 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new PUR test case for NTN User Plane CIoT Optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the LS exchange between GSMA and RAN5 in R5-243449 and R5-243972 test coverage for Rel-16 NB-IoT/PUR was missing although there is industry requirement for it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PUR test case for NTN / User Plane CIoT Optimization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PUR test case for NTN / User Plane CIoT Optimization will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-2 CR 1479 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is an update to R5-247204 and the outcome of 1 revision of this |  |  |  |  |  |  |  |  |

#### 22.1.X	NB-IoT / NTN / User Plane CIoT Optimization / PUR

##### 22.1.X.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and PUR-timeAlignmentTimer has expired }

ensure that {

when { The UE has UL data to send}

then { UE sends RRCConnectionResumeRequest-NB without using the PUR uplink grant }

}

(2)

with { UE in E-UTRA RRC_IDLE state with pur-Config }

ensure that {

when { UE has data to be sent in the UL using PUR}

then { UE sends RRCConnectionResumeRequest-NB using the PUR uplink grant, and the associated HARQ information }

}

(3)

with { UE in E-UTRA RRC_IDLE state with pur-ResponseWindowTimer running }

ensure that {

when { UE receives a PDCCH transmission addressed to the PUR-RNTI and the MAC PDU contains only Timing Advance Command MAC control element }

then { UE considers the transmission using PUR successful }

}

(4)

with { UE in E-UTRA RRC_IDLE suspended state with pur-Config }

ensure that {

when { UE has data to be sent in the UL using PUR}

then { UE sends RRCConnectionResumeRequest-NB using the PUR uplink grant, and the associated HARQ information }

}

(5)

with { UE in E-UTRA RRC_IDLE suspended state and PUR-timeAlignmentTimer running and having previously sent an RRCConnectionResumeRequest-NB  message using PUR}

ensure that {

when { UE receives a PDCCH transmission addressed to the PUR-RNTI and contains an UL grant for a retransmission }

then { UE retransmits the RRCConnectionResumeRequest-NB message using the UL grant provided by the SS }

}

(6)

with { UE in E-UTRA RRC_IDLE state and PUR-timeAlignmentTimer running and having previously sent an RRCConnectionRequest-NB message using PUR}

ensure that {

when { UE receives fallback indication for PUR from the lower layers }

then { UE indicate to upper layers PUR fallback indication was received}

}

##### 22.1.X.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.321 clauses 5.4.7.1 and 5.4.7.2 and TS 36.331 clause 5.3.3.1c. Unless otherwise stated these are Rel-17 requirements

[TS 36.321 clause 5.4.7.1]

Transmission using PUR is initiated by the RRC layer. When transmission using PUR is initiated, RRC layer provides MAC with the following information:

-	PUR-RNTI;

-	Duration of PUR response window pur-ResponseWindowTimer;

-	UL grant information.

If the MAC entity has a PUR-RNTI, the MAC entity shall for each TTI for which RRC layer has provided uplink grant for transmission using PUR:

-	deliver the uplink grant, and the associated HARQ information to the HARQ entity for this TTI.

After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer, which starts at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes. While pur-ResponseWindowTimer is running, the MAC entity shall:

-	if the PDCCH transmission is addressed to the PUR-RNTI and contains an UL grant for a retransmission:

-	restart pur-ResponseWindowTimer at the last subframe of a PUSCH transmission corresponding to the retransmission indicated by the UL grant plus 4 subframes.

-	if L1 ACK for transmission using PUR is received from lower layers; or

-	if PDCCH transmission is addressed to the PUR-RNTI and the MAC PDU is successfully decoded:

-	stop pur-ResponseWindowTimer;

-	if L1 ACK for transmission using PUR is received from lower layers or the MAC PDU contains only Timing Advance Command MAC control element:

-	indicate to upper layers the transmission using PUR was successful;

-	if repetition adjustment for transmission using PUR is received from lower layers:

-	indicate the value of the repetition adjustment to upper layers.

-	discard the PUR-RNTI.

-	else if fallback indication for PUR is received from lower layers:

-	stop pur-ResponseWindowTimer;

-	indicate to upper layers PUR fallback indication is received;

-	if repetition adjustment for transmission using PUR is received from lower layers:

-	indicate the value of the repetition adjustment to upper layers.

-	discard the PUR-RNTI.

-	if the pur-ResponseWindowTimer expires:

-	indicate to upper layers the transmission using PUR has failed;

-	discard the PUR-RNTI.

[TS 36.321, clause 5.4.7.2]

MAC entity may be configured with timer pur-TimeAlignmentTimer by upper layers as specified in TS 36.331 [8], clause 5.3.8.3, TS 36.331 clause 5.3.3.1c.

The MAC entity shall:

-	when pur-TimeAlignmentTimer configuration is received from upper layers:

-	start or restart pur-TimeAlignmentTimer.

-	when pur-TimeAlignmentTimer is released by upper layers:

-	stop the pur-TimeAlignmentTimer, if running.

-	when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5] and if a NTA has been stored or maintained:

-	if the Timing Advance Command MAC control element or PDCCH indicating timing advance adjustment is addressed with a PUR-RNTI:

-	apply the Timing Advance Command or the timing advance adjustment;

-	start or restart the pur-TimeAlignmentTimer, if configured;

-	indicate to upper layers that the Timing Advance value has been adjusted.

-	upon considering a Random Access procedure successfully completed:

-	start or restart the pur-TimeAlignmentTimer, if configured;

-	indicate to upper layers that the Timing Advance value has been adjusted;

-	if a temporary NTA has been stored, delete the stored temporary NTA.

-	upon considering a Random Access procedure unsuccessfully completed, if a temporary NTA has been stored:

-	set the NTA to the stored temporary NTA;

-	delete the stored temporary NTA.

Upon request from upper layers, MAC entity shall indicate whether pur-TimeAlignmentTimer is running.

[TS 36.331, clause 5.3.3.1c]

A BL UE, UE in CE or NB-IoT UE can initiate transmission using PUR when all of the following conditions are fulfilled:

1>	the UE has a valid PUR configuration for the serving cell as specified in 5.3.3.20;

1>	the UE has a valid timing alignment value as specified in 5.3.3.19;

1>	the upper layers request establishment of an RRC connection; or the upper layers request resumption of an RRC connection and the UE has a stored value of the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure;

1>	the establishment or resumption request is for mobile originating calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess;

1>	for CP transmission using PUR, the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS configured for PUR.

NOTE 1:	Upper layers request or resume an RRC connection. The interaction with NAS is up to UE implementation.

NOTE 2:	It is up to UE implementation how the UE determines whether the establishment or resumption request is suitable for transmission using PUR.

##### 22.1.X.3	Test description

22.1.X.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

The UE is configured to use PUR to establish or resume the RRC connection whenever it is possible as per TS 36.331 clause 5.3.3.1c.

Preamble

-	UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) with test loop mode B and configured to return no data in UL according to [18] in Ncell 1.

22.1.X.3.2	Test procedure sequence

Table 22.1.X.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | SS transmits 1 IP packet. | <-- | IP packet | - | - |
| 2 | The SS releases the RRC connection with PUR-Config (pur-TimeAlignmentTimer<loop back delay). Note : pur-TimeAlignmentTimer=1x8x10.24s and loop back delay=90s | <-- | RRCConnectionRelease-NB | - | - |
| 3 | Check: Does the UE send RRCConnectionRequest-NB without using PUR? | --> | RRCConnectionRequest-NB | 1 | P |
| 4 | Steps 3 to 8 of test procedure in Table 8.1.5A.9.3-1is TS 36.508. | - | - | - | - |
| 5 | The UE loops back the IP packet received in Step 1. | --> | IP packet | - | - |
| 6 | SS transmits 1 IP packet. | <-- | IP packet | - | - |
| 7 | The SS releases the RRC connection with PUR-Config (pur-TimeAlignmentTimer>loop back delay). Note : pur-TimeAlignmentTimer=4x8x10.24s and loop back delay=90s | <-- | RRCConnectionRelease-NB | - | - |
| 8 | The SS is configured not to send HARQ ACK. | - | - | - | - |
| 9 | Check: Does the UE send RRCConnectionRequest-NB using the PUR uplink grant, and the associated HARQ information? | --> | RRCConnectionRequest-NB | 2 | P |
| 10 | The SS transmits a Timing Advance Command addressed with the PUR-RNTI without any additional padding (i.e. TBS=16). Start Timer_1 = Time Alignment timer value. The Timing Advance Command indicates a TA index of 31 resulting in no change of timing advance (see TS 36.213 clause 4.2.3) | <-- | NPDCCH (DCI N1)) MAC Control PDU (Timing Advance) | - | - |
| 11 | Check: Does the UE send HARQ ACK NPDSCH? | --> | HARQ ACK | 3 | P |
| 12 | The SS is configured to send HARQ ACK. | - | - | - | - |
| 13 | Steps 3 to 8 of test procedure in Table 8.1.5A.9.3-1is TS 36.508. | - | - | - | - |
| 14 | The UE loops back the IP packet received in Step 6. | --> | IP packet | - | - |
| 15 | SS transmits 1 IP packet. | <-- | IP Packet | - | - |
| 16 | The SS releases the RRC connection with PUR-Config (pur-TimeAlignmentTimer>loop back delay). Note : pur-TimeAlignmentTimer=4x8x10.24s and loop back delay=90s | <-- | RRCConnectionRelease-NB | - | - |
| 17 | The SS is configured not to send HARQ ACK. | - | - | - | - |
| 18 | Check: Does the UE send RRCConnectionResumeRequest-NB using PUR? | --> | RRCConnectionResumeRequest-NB | 4 | P |
| 19 | The SS sends a PDCCH transmission addressed to the PUR-RNTI and contains an UL grant for a retransmission. | <-- | (UL Grant) | - | - |
| 20 | Check: Does the UE send RRCConnectionResumeRequest-NB using the provided grant? | --> | RRCConnectionResumeRequest-NB | 5 | P |
| 21 | The SS sends HARQ ACK. | <-- | HARQ ACK | - | - |
| 22 | The SS is configured to send HARQ ACK | - | - | - | - |
| 23 | The SS sends RRCConnectionResume-NB | <-- | RRCConnectionResume-NB |  |  |
| 24 | Steps 5 to 8 of test procedure in Table 8.1.5A.9.3-1is TS 36.509. | - | - | - | - |
| 25 | The UE loops back the IP packet received in Step 19. |  | IP Packet |  |  |
| 16 | SS transmits 1 IP packet. |  | IP Packet |  |  |
| 27 | The SS releases the RRC connection with PUR-Config (pur-TimeAlignmentTimer>loop back delay). Note : pur-TimeAlignmentTimer=4x8x10.24s and loop back delay=90s | <-- | RRCConnectionRelease-NB | - | - |
| 28 | The SS is configured to indicate fallback for PUR. |  |  |  |  |
| 29 | The UE send RRCConnectionRequest-NB without using PUR? | --> | RRCConnectionRequest-NB | - | - |  |
| 30 | Check: Does the UE send RRCConnectionRequest-NB without using PUR? | --> | RRCConnectionRequest-NB | 6 | P |
| 31 | Steps 3 to 8 of test procedure in Table 8.1.5A.9.3-1is TS 36.508. | - | - | - | - |
| 32 | The UE loops back the IP packet received in Step 15. | --> | IP Packet | - | - |

22.1.X.3.3	Specific message contents

Table 22.1.5.3.3-1: CLOSE UE TEST LOOP (preamble)

| Derivation path: Table 4.7A-3 in 38.508, Condition: UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup. | '01011010 'B | 90 second delay |  |

Table 22.1.X.3.3-1: RRCConnectionRelease-NB (steps 2,7,16 and 27, Table 22.1.X.3.2-1)

| Derivation path: 36.508 table 8.1.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| rrcConnectionRelease-r13 SEQUENCE { |  |  |  |
| releaseCause-r13 | rrc-Suspend |  | step 16 |
| resumeIdentity-r13 | BIT STRING (SIZE(40)) |  | step 16 |
| nonCriticalExtension SEQUENCE { |  |  | step 16 |
| redirectedCarrierInfo-v1430 | Not present |  | step 16 |
| extendedWaitTime-CPdata-r14 | Not present |  | step 16 |
| nonCriticalExtension SEQUENCE { |  |  | step 16 |
| drb-ContinueROHC-r15 | Not present |  | step 16 |
| nextHopChainingCount-r15 | NCC |  | step 16 |
| nonCriticalExtension | Not present |  | step 16 |
| } |  |  | step 16 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pur-Config-r16 SEQUENCE { |  |  |  |
| pur-ConfigID-r16 | 1 |  |  |
| pur-TimeAlignmentTimer-r16 | 1 |  | step 2 |
| pur-TimeAlignmentTimer-r16 | 4 |  | all other steps |
| pur-NRSRP-ChangeThreshold-r16 | Not present |  |  |
| pur-ImplicitReleaseAfter-r16 | Not present |  |  |
| pur-RNTI-r16 |  |  |  |
| pur-ResponseWindowTimer-r16 |  |  |  |
| pur-StartTimeParameters-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-r16 CHOICE { |  |  |  |
| periodicity8 | 1 |  | Step 2 |
| periodicity2048 | 2000 |  | All other steps |
| } |  |  |  |
| startSFN-r16 |  |  |  |
| startSubframe-r16 |  |  |  |
| hsfn-LSB-Info-r16 |  |  |  |
| } |  |  |  |
| pur-NumOccasions-r16 | infinite |  |  |
| pur-PhysicalConfig-r16 SEQUENCE { |  |  |  |
| carrierConfig-r16 |  |  |  |
| npusch-NumRUsIndex-r16 |  |  |  |
| npusch-NumRepetitionsIndex-r16 |  |  |  |
| npusch-SubCarrierSetIndex-r16 |  |  |  |
| npusch-MCS-r16 CHOICE { |  |  |  |
| singleTone | 1 |  |  |
| } |  |  |  |
| p0-UE-NPUSCH-r16 |  |  |  |
| alpha-r16 |  |  |  |
| npusch-CyclicShift-r16 |  |  |  |
| npdcch-Config-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |