3GPP TSG-RAN5 Meeting #105	R5-247663

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3350 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Clause 4.9.6 test procedures for Switch off / Power off UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AP#104.01 was defined to update the TS 38.508-1 procedures which currently allows non-conformant UE behavior. This document addresses the issue. As per TS 24.501, clause 5.5.2.2.3 If the access type in the DEREGISTRATION REQUEST message indicates that the de-registration procedure is for 3GPP access, the AMF shall trigger the SMF to perform a local release of the PDU session(s) established over 3GPP access, if any, for this UE. The UE shall perform a local release of the PDU session(s) established over 3GPP access, if any From the above reference, it is clear that UE shall do a local release of active PDU sessions and not send OTA signalling to release the active PDU sessions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the optional PDU session release procedure from test procedures 4.9.6.1, 4.9.6.2 and 4.9.6.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test procedures remain non-compliant with 3GPP core specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-246456->R5-247663 |  |  |  |  |  |  |  |  |

### 4.9.6	Test procedures for Switch off / Power off UE

#### 4.9.6.1	Switch off / Power off procedure in RRC_IDLE

Table 4.9.6.1-1: Switch off procedure in RRC_IDLE

| Step | Procedure | Message Sequence |  |  |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |  |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) | - | - |  |
| 1a2 | UE transmits an RRCSetupRequest message. | --> | RRC: RRCSetupRequest |  |
| 1a3 | SS transmit an RRCSetup message. | <-- | RRC: RRCSetup |  |
| - | EXCEPTION: Steps 1a4Aa1 to 1a4Aa6 specify optional behaviour if the UE has previously performed IMS registration | - | - |  |
| 1a4Aa1 | The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. | --> | RRC: RRCSetupComplete 5GMM: SERVICE REQUEST |  |
| 1a4Aa2 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |  |
| 1a4Aa3 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |  |
| 1a4Aa4 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT |  |
| - | EXCEPTION: In parallel to the event described in step 1a4Aa5 below, the generic test procedure in TS 34.229-5 [47] Annex A.11 may be performed. | - | - |  |
| 1a4Aa5 | The UE transmits an RRCReconfigurationComplete message. | --> | RRC: RRCReconfigurationComplete |  |
| 1a4Aa5a1 - 1a4Aa5b1 | Void | - | - |  |
| 1a4Aa6 | The UE transmits a DEREGISTRATION REQUEST message | - | 5GMM: DEREGISTRATION REQUEST |  |
| - | EXCEPTION: Step 1a4Ab1 below specifies the behaviour if the UE has not previously performed IMS registration | - | - |  |
| 1a4Ab1 | The UE transmits an RRCSetupComplete message including the DEREGISTRATION REQUEST message. | --> | RRC: RRCSetupComplete 5GMM: DEREGISTRATION REQUEST |  |
| 1a4 | Void | - | - |  |
| 1a5 | The SS transmits an RRCRelease message | <-- | RRC: RRCRelease |  |
| 1b1 | ELSE power off UE (Note 2) | - | - |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |

#### 4.9.6.2	Switch off / Power off procedure in RRC_INACTIVE

4.9.6.2.1	Procedure

Table 4.9.6.2.1-1: Switch off procedure in RRC_INACTIVE

| Step | Procedure | Message Sequence |  |  |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |  |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) | - | - |  |
| 1a2 | UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest |  |
| 1a3 | SS transmit an RRCResume message. | <-- | NR RRC: RRCResume |  |
|  | EXCEPTION: Steps 1a3Aa1 to 1a3Ab1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place depending on the UE implementation. |  |  |  |
| 1a3Aa1 | The UE transmits an RRCResumeComplete message to confirm the successful resumption of the RRC connection not including any 5G NAS message | --> | NR RRC: RRCResumeComplete |  |
| 1a4Aa1-1a4Aa5 | Void | - | - |  |
| - | EXCEPTION: The generic test procedure in TS 34.229-5 [47] Annex A.11 may be performed depending on UE implementation. | - | - |  |
| - | EXCEPTION: Steps 1a3Aa2a1 - 1a3Ab1 may be performed depending on UE implementation | - | - |  |
| 1a3Aaa2a1 - 1a3Aa2a2 | Void | - | - |  |
| 1a3Aa3 | The UE transmits a DEREGISTRATION REQUEST message | - | 5GMM: DEREGISTRATION REQUEST |  |
| 1a3Ab1 | The UE transmits an RRCResumeComplete message to confirm the successful resumption of the RRC connection, including the DEREGISTRATION REQUEST message | --> | NR RRC: RRCResumeComplete 5GMM : DEREGISTRATION REQUEST |  |
| 1a4 | Void | - | - |  |
| 1a5 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease |  |
| 1b1 | ELSE power off UE (Note 2) | - | - |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |

4.9.6.2.2	Specific Message contents

Table 4.9.6.2.2-1: RRCResumeRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |  |
| resumeIdentity |  | Not checked |  |  |
| resumeMAC-I |  | Not checked |  |  |
| resumeCause |  | Not checked |  |  |
| spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 4.9.6.3	Switch off / Power off procedure in RRC_CONNECTED

Table 4.9.6.3-1: Switch off procedure in RRC_CONNECTED

| Step | Procedure | Message Sequence |  |  |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |
| 1a1-1a3 | Void | - | - |  |
| 2-4 | Void | - | - |  |
| - | EXCEPTION: Steps 5a1 to 5b1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if a particular implementation is under test. | - | - |  |
| 5a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) |  |  |  |
| - | EXCEPTION: Step 5a1Aa1 to 5a2Aa2 below specifies optional behaviour if the UE has previously performed IMS registration | - | - |  |
| 5a1Aa1 | The UE may perform the generic test procedure described in TS 34.229-5 [47] Annex A.11 | - | - |  |
| 5a2Aa1a1 - 5a2Ab1 | Void | - | - |  |
| 5a2Aa2 | The UE transmits a DEREGISTRATION REQUEST message. | --> | 5GMM: DEREGISTRATION REQUEST |  |
| 5a3 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease |  |
| 5b1 | ELSE power off UE (Note 2) | - | - |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |

#### 4.9.6.3A	Switch off / Power off procedure in RRC_CONNECTED with T3540 started

Table 4.9.6.3A-1: Switch off procedure in RRC_CONNECTED with T3540 started

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |  |
| 1 | SS starts timer1 = T3540 (10 sec). | - | - |
| 2 | The SS locally releases the RRC connection. | - | - |
| 3 | SS waits for Timer1 expires. NOTE: On expiry of T3540 UE shall locally release the established N1 NAS signalling connection | - | - |
| 4 | The test procedure as described in Table 4.9.6.1-1: Switch off procedure in RRC_IDLE take place. | - | - |

#### 4.9.6.4	Switch off / Power off procedure in State DEREGISTERED

Table 4.9.6.4-1: Switch off procedure in State DEREGISTERED

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |  |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE (Note 1) | - | - |
| 1b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal. |  |  |  |

#### 4.9.6.5	Switch off / Power off procedure in WLAN Ipsec_SA_Established

Table 4.9.6.5-1: Switch off procedure in Ipsec_SA_Established

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |  |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [30] pc_SwitchOnOff or [31] pc_USIM_Removal is supported | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE (Note 1) | - | - |
| 1a2 | The UE transmits a DEREGISTRATION REQUEST message. | --> | 5GMM: DEREGISTRATION REQUEST |
| 1a3 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5, takes place performing disconnection of security association | - | - |
| 1b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |

#### 4.9.6.6	Switch off / Power off procedure in MA PDU session Established on NR and WLAN

Table 4.9.6.6-1: Switch off procedure in MA PDU session Established

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |  |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [30] pc_SwitchOnOff or [31] pc_USIM_Removal is supported | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE (Note 1) | - | - |
| - | EXCEPTION: The Step 5a1Aa1 to 5a3 specified in Table 4.9.6.3-1 may take place on the NR Cell before or after the events described in steps 1a2-1a3 below. | - | - |
| 1a2 | The UE transmits a DEREGISTRATION REQUEST message on the WLAN cell. | --> | 5GMM: DEREGISTRATION REQUEST |
| 1a3 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5, takes place performing disconnection of security association on the WLAN cell. | - | - |
| 1b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |