3GPP TSG-RAN WG5 Meeting #105	R5-247765

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3054 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update AMPR TC for n41 CBW 5MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-9-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AMPR TC for n41 CBW 5MHz needs to be updated according to WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AMPR TC for n41 CBW 5MHz has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The band can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 3385 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related CR is R5-247043 (CR#3385). R5-247043 introduces the test frequencies of n41 5MHz in TS 38.508-1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, NS_07, NS_27 and NS_35 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.3.4 | n12xx, n85 | 3, 5, 10, 15 |  | N/A |
|  |  | n14 | 5, 10 |  |  |
| NS_07 | 6.5.2.3.3.4, 6.5.3.3.5.26 | n13 | 5,10 | Table 6.2.3.3.29-1 | Table 6.2.3.3.29-2 |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 3, 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 3, 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 3, 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n83xx | 3, 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n83xx | 3, 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.2.3.3.9 6.5.3.3.3.12 | n30 | 5,10 |  | Clause 6.2.3.3.14 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 25, 30, 40, 50 | Table 6.2.3.3.17-1 | Table 6.2.3.3.17-2 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 Table 6.2.3.3.18-3 | Table 6.2.3.3.18-2 Table 6.2.3.3.18-4 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.26-1, Table 6.2.3.3.26-3 | Table 6.2.3.3.26-2, Table 6.2.3.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.27-1 Table 6.2.3.3.27-3 | Table 6.2.3.3.27-2, Table 6.2.3.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.3.16 | n39 | 10, 15, 20, 25, 30, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.3.27 | n24, n99 | 5, 10 | Table 6.2.3.3.30 | Clause 6.2.3.3.30 |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz, 10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [6] absent is default for all NR bands. NOTE 9: 3 MHz channel bandwidth is not applicable. |  |  |  |  |  |

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_46 | 6.5.3.3.3.25 | n7 | 25, 30, 40, 50 | Table 6.2.3.3.17-1 Table 6.2.3.3.17-311 | Table 6.2.3.3.17-2 Table 6.2.3.3.17-411 |

<< UNCHANGED PARTS SKIPPED >>

Table 6.2.3.4.1-2: Test Configuration table for NS_04 (Power Class 1.5, 2 and 3)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 3 (Note 3) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 4 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 7 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 8 (Note 4, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 9 (Note 4) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 10 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 11 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 12 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 14 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 16 | Low |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 17 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 18 | High |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 23 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 25 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 26 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 27 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 28 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 29 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 30 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 31 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 32 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 33 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 34 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 35 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 36 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 37 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 38 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 39 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 40 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 41 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 42 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 43 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 44 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 45 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 46 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 47 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 48 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 49 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |

| A-MPR test parameters for NS_04 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 50 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 51 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 52 | Low |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 53 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 54 | High |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 55 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 56 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 57 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 58 | Low |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 59 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 60 | High |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth |  |  |  |  |  |  |

Table 6.2.3.4.1-2a: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Same values as for Low range of n41 CBW 5MHz in TS 38.508-1 [5] Table 4.3.1.1.1.41-1 SCS 15 kHz and ΔFRaster 15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 52 |  |  | 2505 | 501000 | 2500.32 | 500064 | 0 | 15 | 6255 | 500430 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2512.5 | 502500 | 2505.39 | 501078 | 0 | 15 | 6270 | 501630 | 4 | 1 | 2 (4) | 5 |

Table 6.2.3.4.1-2b: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2505 | 501000 | 2500.68 | 500136 | 0 | 30 | 6261 | 500910 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2512.5 | 502500 | 2505.66 | 501132 | 0 | 30 | 6273 | 501870 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2c: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2505 | 501000 | 2501.04 | 500208 | 0 | 15 | - | 500568 |
| 15 | 18 |  |  | 2512.5 | 502500 | 2506.02 | 501204 | 0 | 15 | - | 501564 |

Table 6.2.3.4.1-2d: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 |  |  | 2508.5 | 501700 | 2506.25 | 501250 | 0 | 15 | 6269 | 501610 | 0 | 0 | 0 (0) | 0 |
| 10 | 52 |  |  | 2511 | 502200 | 2506.32 | 501264 | 0 | 15 | 6270 | 501630 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2513.505 | 502701 | 2506.395 | 501279 | 0 | 15 | 6273 | 501870 | 5 | 2 | 2 (4) | 6 |
| 20 | 106 |  |  | 2520 | 504000 | 2510.46 | 502092 | 0 | 15 | 6282 | 502590 | 10 | 1 | 1 (2) | 3 |
| 30 | 160 |  |  | 2535 | 507000 | 2520.6 | 504120 | 0 | 15 | 6306 | 504510 | 10 | 0 | 0 (0) | 0 |
| 40 | 216 |  |  | 2550 | 510000 | 2530.56 | 506112 | 0 | 15 | 6333 | 506670 | 6 | 1 | 2 (4) | 5 |
| 50 | 270 |  |  | 2565 | 513000 | 2540.7 | 508140 | 0 | 15 | 6357 | 508590 | 6 | 0 | 1 (2) | 2 |

<< END OF CHANGES >>