# 3GPP TSG-SA Meeting #105	SP-241326

Melbourne, Australia, Sept 10 – 13, 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 22.850 AIML terminology analysis across 3GPP

# Document for:	Approval

# Agenda Item:	7

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

None.

# 3	Rationale

This pCR aims to capture AI/ML-related terminology from Release 18 and Release 19 AI/ML activities across 3GPP. It seeks to identify inconsistencies and misalignments among them and proposes solutions to resolve these issues.

# 4	Detailed proposal

Start of modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]					3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[a]					3GPP TS 23.288: “Architecture enhancements for 5G System (5GS) to support network data analytics services (Release 18)”.

[b]					3GPP TR 23.700-84: “Study on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (Release 19)”.

[c]					3GPP TS 28.105: “Artificial Intelligence / Machine Learning (AI/ML) management (Release 18)”.

[d]					3GPP TR 23.700-82: “Study on application layer support for AI/ML services (Release 19)”.

[e]					3GPP TR 38.843: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”.

[f]					3GPP TS 38.300: “NR and NG-RAN Overall Description; Stage 2 (Release 18)”.

[g]					3GPP TR 26.927: “Study on Artificial Intelligence and Machine Learning in 5G media services (Release 19)”

Start of next modification

# 5	AI/ML related activities in all Working Groups

## 5.1 	General

This clause will investigate and identify AI/ML related activities of all working groups of Rel-18 features and Rel-19 studies and work items in TSG CT, TSG RAN and TSG SA Working Groups.

Editor's Note: The AI/ML related content captured in TR 21.918 ("Release 18 Description; Summary of Rel-18 Work Items") can be considered as a starting point.

## 5.2	AI/ML related activities in TSG SA & CT Working Groups

### 5.2.X	AI/ML related activities on WID/SID#X

### 5.2.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG SA working groups of Rel-18 features and Rel-19.

### 5.2.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

### 5.2.Y		AI/ML related terminology

#### 5.2.Y.1		TSG SA WG2

The following definitions are provided in clause 3 of 3GPP TS 23.288 [a]:

- Analytics Accuracy Information: Represent a performance measure of an analytics ID provided by an NWDAF containing AnLF, which is composed of the number of correct predictions of the analytics ID out of all predictions and the corresponding number of samples.

- ML Model Accuracy Information: Represent a performance measure of a ML Model provided by an NWDAF containing MTLF, which is composed of the number of correct predictions by the ML Model out of all predictions and the corresponding number of samples.

- Analytics Feedback Information: Indicates that the consumer NF has taken action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data.

The following definitions are provided in clause 5 of  3GPP TS 23.288 [a]:

- Analytics logical function (AnLF): A logical function in NWDAF, which performs inference, derives analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and exposes analytics service i.e. Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo.

- Model Training logical function (MTLF): A logical function in NWDAF, which trains Machine Learning (ML) models and exposes new training services (e.g. providing trained ML Model) as defined in clause 7.5 and clause 7.6.

The following definitions are provided in clause 3 of 3GPP TR 23.700-84 [b]:

- Horizontal Federated Learning (HFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs).

- Vertical Federated Learning (VFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs).

- Label: A label is the training objective in supervised machine learning.

- VFL Server: An NWDAF or AF that integrates local training results, computes gradient information or loss information and send them to VFL client(s) for the local ML model update in VFL training process. It also coordinates the VFL training process by discovering and selecting VFL clients. In VFL inference process, The VFL server aggregates local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer. Only one VFL server may exist for each VFL process.

- VFL Client: An NWDAF or AF that holds the local dataset and performs local training and inference as asked by VFL Server. There can be multiple VFL Clients in VFL training and inference.

The following definitions are provided in clause 6.15 of 3GPP TR 23.700-84 [b]:

- VFL active participant: A VFL function that owns part of an ML model for an analytic ID and knows the labels for the ML model. The active participant is the main function for training an ML model for an analytic ID.

- VFL passive participant: A VFL function that owns part of an ML model for an analytic ID but does not know the labels of the ML model but is able to collect local data for one or more features.

#### 5.2.Y.2		TSG SA WG5

The following definitions are provided in clause 3 of 3GPP TS 28.105 [c]:

- ML model: a manageable representation of an ML model algorithm.

  - NOTE 1: an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output.

  - NOTE 2: ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification.

  - NOTE 3: ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context.

- ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model.

- ML model initial training: a process of training an initial version of an ML model.

- ML model re-training: a process of training a previous version of an ML model and generate a new version.

  - NOTE 4: a new version of a trained ML model supports the same type of inference as the previous version of the ML model, i.e., the data type of inference input and data type of inference output remain unchanged between the two versions of the ML model, but parameter values might be different for the re-trained model.

- ML model joint training: a process of training a group of ML models.

- ML training function: a logical function with ML model training capabilities.

- AI/ML inference function: a logical function that employs an ML model to conduct inference.

- ML model testing: a process of testing an ML model using testing data.

- ML testing function: a logical function with ML model testing capabilities.

- AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

  - NOTE 5: the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

- AI/ML inference function: a logical function that employs trained ML model(s) to conduct inference.

- AI/ML inference emulation: running the inference process to evaluate the performance of an ML model in an emulation environment before deploying it into the target environment.

- ML model deployment: a process of making a trained ML model available for use in the target environment.

#### 5.2.Y.3		TSG SA WG6

The following definitions are provided in clause 3 of 3GPP TR 23.700-82 [d]:

- ML model: According to 3GPP TS 28.105 [c], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information.

- ML model lifecycle: The lifecycle of an ML model includes data collection, data processing, model training, model verification, model, instantiation and deployment, model monitoring and termination of ML model components.

- ML model training: According to 3GPP TS 28.105 [c], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss.

- ML model inference: According to 3GPP TS 28.105 [c], ML model training includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference.

- AI/ML enablement: an application enablement framework consisting of one or more AI/ML enabler capabilities based on implementation. Such function can be deployed as (or within) an enablement layer server (e.g. SEAL or ADAES) or client.

- AI/ML client: an application layer entity (also referred as ML client) which is an AI/ML endpoint and performs client-side operations (e.g. related to the ML model lifecycle). Such AI/ML client can be a VAL client or AIML enabler client and may be configured e.g. to provide ML model training and inference locally e.g. at the VAL UE side.

- AI/ML server: an application layer entity which is an AI/ML endpoint and performs server-side operations (e.g. related to the ML model lifecycle). Such AI/ML server can be a VAL server or AIML enabler server.

- FL member: An FL member or participant is an entity which has a role in the FL process. An FL member can be an FL client performing ML model training, or an FL server performing aggregation/collaboration for the FL process. The FL member in this study is assumed to be either a functionality at the VAL UE or at the network/server side (AIML enablement server, VAL server).

- FL client: An FL member which locally trains the ML model as requested by the FL server.

- FL server: An FL member which generates global ML model by aggregating local model information from FL clients.

## 5.3	AI/ML related activities in TSG RAN Working Groups

### 5.3.X	AI/ML related activities on WID/SID#X

### 5.3.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG SA working groups of Rel-18 features and Rel-19.

### 5.3.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

### 5.3.Y		AI/ML related terminology

#### 5.3.Y.1		TSG RAN WG1

The following definitions are provided in clause 3 of 3GPP TR 38.843 [e]:

- AI/ML-enabled Feature: refers to a Feature where AI/ML may be used.

- AI/ML Model: A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs.

- AI/ML model delivery: A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.

- AI/ML model ID:  A logical AI/ML model is identified by a Model ID. The Model ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

- AI/ML model Inference:  A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs.

- AI/ML model testing: A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model.

- AI/ML model training: A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference.

- AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

- AI/ML model validation: A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training.

- Data collection: A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference.

- Federated learning / federated training: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples.

- Functionality identification: A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE. Note: Information regarding the AI/ML functionality may be shared during functionality identification. Where AI/ML functionality resides depends on the specific use cases and sub use cases.

- Management instruction: Information needed to ensure proper inference operation. This information may include selection/(de)activation/switching of AI/ML models or AI/ML functionalities, fallback to non-AI/ML operation, etc.

- Model activation: enable an AI/ML model for a specific AI/ML-enabled feature.

- Model deactivation: disable an AI/ML model for a specific AI/ML-enabled feature.

- Model download: Model transfer from the network to UE.

- Model identification: A process/method of identifying an AI/ML model for the common understanding between the NW and the UE. Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.

- Model monitoring: A procedure that monitors the inference performance of the AI/ML model.

- Model parameter update: Process of updating the model parameters of a model.

- Model selection: The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation.

- Model switching: Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific AI/ML-enabled feature.

- Model update: Process of updating the model parameters and/or model structure of a model.

- Model upload: Model transfer from UE to the network.

- Network-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the network.

- Offline field data: The data collected from field and used for offline training of the AI/ML model.

- Offline training: An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions.

- Online field data: The data collected from field and used for online training of the AI/ML model.

- Online training: An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples.

- Reinforcement Learning (RL): A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a.  reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with.

- Semi-supervised learning: A process of training a model with a mix of labelled data and unlabelled data.

- Supervised learning: A process of training a model from input and its corresponding labels.

- Test encoder/decoder for TE: AI/ML model for UE encoder/gNB decoder implemented by TE.

- Two-sided (AI/ML) model: A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa.

- UE-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the UE.

- Unsupervised learning: A process of training a model without labelled data.

- Proprietary-format models: ML models of vendor-/device-specific proprietary format, from 3GPP perspective. They are not mutually recognizable across vendors and hide model design information from other vendors when shared. Note: An example is a device-specific binary executable format.

- Open-format models: ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. They are mutually recognizable between vendors and do not hide model design information from other vendors when shared.

The following definitions are provided in clause 4.2 of 3GPP TR 38.843 [e]:

- (ML-enabled) Functionality (Available in the TR but no as part of terminology definitions): An AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

- Functionality-based LCM (Available in the TR but no as part of terminology definitions): Signaling procedure where network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI); operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG

- AI/ML model-based LCM: Operates based on identified logical models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. The models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID

#### 5.3.Y.2		TSG RAN WG3

The following definitions are provided in clause 16.20 of 3GPP TS 38.300 [f]:

- AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [c].

- AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [c].

Start of next modification

# 6	Analysis on AI/ML across 3GPP

## 6.1 General

This clause will identify any potential misalignments and inconsistencies for AI/ML across 3GPP, based on Clause 5.

NOTE 1:  Any RAN related aspects are subject to early coordination and feedback from TSG RAN.

## 6.2	AI/ML related terminology

### 6.2.a 		Analysis on ML model

The term ‘ML model’ has been defined differently by SA5, SA6 and RAN1, as illustrated in Table A. Consequently, a unified definition that can be adopted by all 3GPP Working Groups is necessary.

| TSG (TS/TR) | ML model |
| --- | --- |
| 3GPP SA5 TS 28.105 [c] | A manageable representation of an ML model algorithm. NOTE 1: an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output. NOTE 2: ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification. NOTE 3: ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context. |
| 3GPP SA6 TR 23.700-82 [d] | According to 3GPP TS 28.105 [c], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information. |
| 3GPP RAN1 TR 38.843 [e] | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |

Table A: Definition of ML model as defined across 3GPP WGs.

The following definition for ‘ML model’ is proposed for adoption by all 3GPP Working Groups:

ML model:  A mathematical algorithm that applies ML techniques to generate a set of outputs based on a set of inputs. It may include metadata which consists of, e.g., information related to the trained model, and applicable runtime context.

- Note: An ML model can be managed, stored, and transferred as artifacts, which may be containers, images, or proprietary file formats.

### 6.2.b		Analysis on ML model training

The term ‘ML model training’ has been defined differently by SA5, SA6 and RAN1, as illustrated in Table B. RAN3 follows the definition of SA5. Consequently, a unified definition that can be adopted by all 3GPP Working Groups is necessary.

| TSG (TS/TR) | ML model training |
| --- | --- |
| 3GPP SA5 TS 28.105 [c] | A process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model. |
| 3GPP SA6 TR 23.700-82 [d] | According to 3GPP TS 28.105 [c], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss. |
| 3GPP RAN1 TR 38.843 [e] | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference. |
| 3GPP RAN3 TS 38.300 [f] | AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [c]. |

Table B: Definition of ML model training as defined across 3GPP WGs.

The following definition for ‘ML model training’ is proposed for adoption by all 3GPP Working Groups:

ML model training:  A process to train an ML Model by learning the input/output relationship in a data driven manner and obtain the trained ML Model for inference.

### 6.2.c	Analysis on ML model re-training

The term ‘ML model re-training’ has been defined differently by SA5 and RAN1, as illustrated in Table C. RAN1 introduces two new terms, i.e., ML model parameter update and ML model update, which is nothing but ML model re-training. Consequently, a unified term and definition that can be adopted by all 3GPP Working Groups is necessary.

| TSG (TS/TR) | ML model re-training / ML model parameter update / ML model update |
| --- | --- |
| 3GPP SA5 TS 28.105 [c] | ML model re-training: A process of training a previous version of an ML model and generate a new version. |
| 3GPP RAN1 TR 38.843 [e] | ML model parameter update: A process of updating the model parameters of a model. Model update: A process of updating the model parameters and/or model structure of a model |

Table C: Definition of ML model re-training / ML model parameter update as defined across 3GPP WGs.

The term ML model re-training is proposed to be adopted by all 3GPP Working Groups rather than using different terms such as ‘ML model parameter update’ or ‘ML model update’ which means exactly the same. The following definition for ‘ML model re-training’ is proposed for adoption by all 3GPP Working Groups:

ML model re-training:  A process of training a previous version of an ML model and generate a new version.

### 6.2.d	Analysis on ML model testing

The term ‘ML model testing’ has been defined differently by SA5 and RAN1, as illustrated in Table D. Consequently, a unified definition that can be adopted by all 3GPP Working Groups is necessary.

| TSG (TS/TR) | ML model testing |
| --- | --- |
| 3GPP SA5 TS 28.105 [c] | A process of testing an ML model using testing data. |
| 3GPP RAN1 TR 38.843 [e] | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |

Table D: Definition of ML model testing as defined across 3GPP WGs.

The following definition for ‘ML model testing’ is proposed for adoption by all 3GPP Working Groups:

ML model testing:  A process of evaluating the performance of an ML model using testing data different from data used for model training and validation.

### 6.2.e		Analysis on ML model inference

The term ‘ML model inference’ has been defined differently by SA5, SA6 and RAN1, as illustrated in Table E. RAN3 follows the definition of SA5. Consequently, a unified definition that can be adopted by all 3GPP Working Groups is necessary.

| TSG (TS/TR) | ML model inference |
| --- | --- |
| 3GPP SA5 TS 28.105 [c] | A process of running a set of input data through a trained ML model to produce set of output data, such as predictions. |
| 3GPP SA6 TR 23.700-82 [d] | According to 3GPP TS 28.105 [c], ML model training includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference. |
| 3GPP RAN1 TR 38.843 [e] | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs. |
| 3GPP RAN3 TS 38.300 [f] | AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [c]. |

Table E: Definition of ML model inference as defined across 3GPP WGs.

The following definition for ‘ML model inference’ is proposed for adoption by all 3GPP Working Groups:

ML model inference:  A process of running a set of inputs through a trained ML model to produce a set of outputs.

### 6.2.f	Analysis on ML model activation & ML model de-activation

The term ‘ML model activation’ and ‘ML model deactivation’ have been defined by RAN1, as illustrated in Table F. SA5 mentions the terms ML activation and ML deactivation several times in 3GPP TS 28.105 [c] but does not provide a definition. Consequently, a unified definition that can be adopted by all 3GPP Working Groups is necessary.

| TSG (TS/TR) | ML model inference |
| --- | --- |
| 3GPP RAN1 TR 38.843 [e] | ML Model activation: enable an AI/ML model for a specific AI/ML-enabled feature. ML Model deactivation: disable an AI/ML model for a specific AI/ML-enabled feature. |

Table F: Definition of ML model inference as defined across 3GPP WGs.

The following definition for ‘ML model activation’ and ‘ML model deactivation’ is proposed for adoption by all 3GPP Working Groups:

ML model activation: A process to enable an ML model for inference.

ML model deactivation: A process to disable an ML model for inference.

### 6.2.g	Analysis on ML model lifecycle

The term ‘ML model lifecycle’ has been defined by SA6, as illustrated in Table G. However, 3GPP SA2 TS 23.288 [a] 3GPP SA2 TR 23.700-84 [b], 3GPP SA4 TR 26.927 [g], 3GPP SA5 TS 28.105 [c], 3GPP SA6 TR 23.700-82 [d], 3GPP RAN1 TR 38.843 [e] and RAN3 also mentions one or more phases of ML model life cycle without providing a clear definition of ML model lifecycle. Consequently, a unified definition that can be adopted by all 3GPP Working Groups is necessary.

| TSG (TS/TR) | ML model lifecycle |
| --- | --- |
| 3GPP SA6 TR 23.700-82 [d] | The lifecycle of an ML model includes data collection, data processing, model training, model verification, model, instantiation and deployment, model monitoring and termination of ML model components. |

Table G: Definition of ML model lifecycle as defined across 3GPP WGs.

The following definition for ‘ML model lifecycle’ is proposed for adoption by all 3GPP Working Groups:

ML model lifecycle:  The end-to-end process typically consisting of data processing, model training, model testing, model deployment, model monitoring and model maintenance.

- Note 1: Data processing includes collecting and preparing the data for model training and model inference.

- Note 2: Model training includes training and validating the model before deployment.

- Note 3: Model testing includes testing the model before deployment.

- Note 4: Model deployment includes making a trained ML model available for use in the target environment.

- Note 5: Model monitoring includes observing the performance of the model once deployed for inference.

- Note 6: Model maintenance includes updating the model, retraining the model and (de-)activating the model.

### 6.2.h		Analysis on ML model lifecycle management / Functionality-based lifecycle management

The terms ‘ML model-based lifecycle management’, ‘ML-enabled functionality’, and ‘Functionality-based lifecycle management’ have been defined by RAN1, as illustrated in Table H. SA5 describes the lifecycle management capabilities for ML model training, ML model testing, ML inference emulation, ML model deployment and ML inference in clause 6 of 3GPP TS 28.105 [c]. In clause 8.12 of 3GPP TR 23.700-82 [d], SA6 describes solutions to support AI/ML model lifecycle management for ML model re-training and update when model performance degradation is observed by AI/ML Enablement. Consequently, a unified definition that can be adopted by all 3GPP SA and 3GPP RAN Working Groups is necessary.

| TSG (TS/TR) | ML model lifecycle management / Functionality-based lifecycle management |
| --- | --- |
| 3GPP RAN1 TR 38.843 [e] | ML model-based lifecycle management: Operates based on identified logical models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature / Feature Group and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. The models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. (ML-enabled) Functionality: An AI/ML-enabled Feature/Feature Group enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Functionality-based lifecycle management: Signaling procedure where network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI); operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature / Feature Group. |

Table H: Definitions of ML model-based lifecycle management, ML-enabled functionality and Functionality-based lifecycle management as defined across 3GPP WGs.

The following definition for ‘ML model lifecycle management’ is proposed for adoption by all 3GPP SA Working Groups:

ML model lifecycle management:  The management capabilities allowing a consumer to manage different phases of the ML model lifecycle as defined in clause 6.2.g.

The following definition for ‘ML model lifecycle management’ is proposed for adoption by all 3GPP RAN Working Groups:

ML model lifecycle management: Operates based on identified logical models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature / Feature Group and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. The models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

- Note 1: In the context of RAN1 RAN2 and RAN4, ML Model lifecycle management does not consider training, testing and maintenance phases and consider them as implementation-specific.

The following definition for ‘Functionality-based lifecycle management’ is proposed for adoption by all 3GPP RAN Working Groups:

Functionality-based lifecycle management: Signaling procedure where network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI); operates based on, at least, one configuration of AI/ML-enabled Feature / Feature Group or specific configurations of an AI/ML-enabled Feature/FG.

- Note 1: Applicability of this definition to/in TSG SA WGs is optional.

### 6.2.i	Analysis on Horizontal Federated Learning

The term ‘Horizontal Federated Learning’ has been defined in SA2 and RAN1 defines ‘Federated Learning’, as illustrated in Table I. Consequently, a unified definition for Horizontal Federated learning that can be adopted by all 3GPP Working Groups is necessary.

| TSG (TS/TR) | ML model inference |
| --- | --- |
| 3GPP SA2 TR 23.700-84 [b] | Horizontal Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs). |
| 3GPP RAN1 TR 38.843 [e] | Federated Learning: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |

Table I: Definition of Horizontal Federated Learning as defined across 3GPP WGs.

The following definition for ‘Horizontal Federated Learning’ is proposed for adoption by all 3GPP Working Groups:

Horizontal Federated Learning:  A federated learning technique without exchanging/sharing local data set, wherein the local data set in different HFL clients for local model training have the same feature space for different samples.

### 6.2.j	Analysis on Vertical Federated Learning

The term ‘Vertical Federated Learning’ has been defined in SA2 and RAN1 defines ‘Federated Learning’, as illustrated in Table J. Consequently, a unified definition for Vertical Federated Learning’ that can be adopted by all 3GPP Working Groups is necessary.

| TSG (TS/TR) | ML model inference |
| --- | --- |
| 3GPP SA2 TR 23.700-84 [b] | Vertical Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). |
| 3GPP RAN1 TR 38.843 [e] | Federated Learning: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |

Table J: Definition of Vertical Federated Learning as defined across 3GPP WGs.

The following definition for ‘Vertical Federated Learning’ is proposed for adoption by all 3GPP Working Groups:

Vertical Federated Learning:  A federated learning technique without exchanging/sharing local data set and local ML models, wherein the local data set in different VFL clients for local model training have different feature spaces for the same samples.

### 6.2.k	Analysis on Decision vs Prediction vs Output

RAN1 and RAN3 only uses “prediction” in all corresponding ML related TRs/TSs. SA2 uses “output” in all corresponding TRs/TSs where output may include both statistics and predictions. SA5 uses “decision” in all corresponding TRs/TSs with few occurrences of “prediction”. Consequently, a unified term that can be adopted by all 3GPP Working Groups is necessary.

The term “output” is proposed to be used by all 3GPP Working Groups since output may include decision or prediction or statistic or recommendation.

### 6.2.l	Analysis on ML vs AI vs AI/ML

RAN1, RAN2, RAN3 and SA1 only uses “AI/ML” in all corresponding ML related TRs/TSs. SA2 uses “ML” in all corresponding ML related TRs/TSs. SA3, SA4 and SA6 uses a mix of “AI/ML”, “AI” and “ML” in all corresponding ML related TRs/TSs. SA5 uses “ML” for training/testing/emulation and “AI/ML” for inference in all corresponding ML related TRs/TSs.

The term “ML” is proposed to be used by all 3GPP Working Groups considering only ML related aspects of AI are discussed in Rel-18 and Rel-19 and nothing about other aspects of AI.

End of modifications