3GPP TSG-SA WG6 Meeting #63	S6-244249

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0097 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of SEALDD Background data transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SEALDD Background Data Transfer supports BDT DL (e.g. streaming a video clip to the VAL client), and in UL direction, (e.g. uploading drone captured data to the VAL server). However: - figure 9.11.2.1-1 and the precondition of 9.11.2.2 refer to subscription to BDT in DL direction. - Table 9.11.3.2-1 and 9.11.3.5-1 indicate that the granted time window applies only for UL direction. The steps for the procedure for SEALDD BDT subscription can be further detailed to split the BDT negotiation and the application of the negotiated BDT reference. The application of the negotiated BDT reference can be split in its application for ongoing sessions and the network configuration for its application for future sessions. The subscription response step should be placed after the BDT negotiation takes place. Similar changes for the procedure for SEALDD BDT subscription update. The IEs included in subscription, update subscription requests and related responses need to be aligned consistently. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of figure 9.11.2.1-1 and the precondition of 9.11.2.2 to remove the reference to the DL direction. Remove UL direction in tables 9.11.2.1-1 and 9.11.3.5-1. Separation of the BDT negotiation and the application of the negotiated BDT reference into two steps. Separation of the application of the BDT reference to distinguish the procedure for ongoing sessions and for future sessions. Alignment of the IEs. In addition, the Background Data Transfer functionality is introduced in the functional overview for the SEALDD service, data transmission requirements and SEALDD server/client description. SEALDD-X reference point is added to represent the interface with the NRM server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and inconsistent definition of the BDT configuration related procedures and IEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x (NEW), 6.2.2, 7.3.2, 7.4.x (NEW), 7.5.3.x (NEW), 9.11.2.1, 9.11.2.2, 9.11.2.3, 9.11.3.1, 9.11.3.2, 9.11.3.3, 9.11.3.4, 9.11.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 4.x	Background Data Transfer

This SEALDD functionality enables to keep data transmission costs lower by favouring time windows for data transfer to/from specific UEs in a geographical area, e.g. during non-busy hours, that are less costly and able to handle larger bitrates. The background data transfer procedures are specified in clause 9.11.

* * * Next Change * * * *

### 6.2.2	Requirements

[AR-6.2.2-a] The SEALDD service shall provide a mechanism for application signalling data transmission and application media data transmission between VAL client(s) and VAL server(s).

[AR-6.2.2-b] The SEALDD service shall provide a mechanism to support the data transmission quality requirement configurations and measurements for the application data transmission between VAL client(s) and VAL server(s).

[AR-6.2.2-c] The SEALDD service shall provide a mechanism for application data transmission between VAL client(s) and VAL server(s) with guaranteed quality.

[AR-6.2.2-d] The SEALDD service shall provide a mechanism for application data packaging and un-packaging to support the data transmission between VAL client(s) and VAL server(s).

[AR-6.2.2-e] The SEALDD service shall provide a mechanism for E2E redundant data transmission between VAL client and VAL server.

[AR-6.2.2-f] The SEALDD service shall provide a mechanism to support the packet/data duplication, elimination and error recovery between VAL client and VAL server.

[AR-6.2.2-g] The SEALDD service shall provide a mechanism to support favouring time windows for data transfer between VAL client(s) and VAL server(s) that are less costly and/or able to handle larger bitrates.

* * * Next Change * * * *

### 7.3.2	SEAL Data Delivery server

The SEAL data delivery server functional entity acts as the application server for the data delivery enablement. The SEALDD server supports the following capabilities:

a)	Suppport the signalling interaction with VAL server to negotiate the data delivery aspects including QoS requirement, protocols information, bandwidth settings, delivery policy provisioning, transmission quality measurement, background data transfer.

b)	Support the signalling interaction with the SEALDD client to management the data delivery between the SEALDD server and the SEALDD client, including the establish/update/release of signalling plane and user plane of SEALDD-UU, bandwidth control, transmission quality measurement.

c)	Providing the application data/media storage.

d)	Interacting with 5GC via N33/N5 (i.e. send control plane requirements or receive control plane notification) with usage of capability exposed by 3GPP network.

e)	Support user plane handlings including obtaining the application data/media and/or application signalling data from VAL server via SEALDD-S, delivering it to the SEALDD client via the SEALDD-UU, and receiving the application data/media and/or application signalling data from SEALDD client and providing it to the VAL server via SEALDD-S, enforce the rate control.

* * * Next Change * * * *

### 7.4.x	SEALDD-X

Reference point enables interactions between the SEALDD server and other SEAL enablers to exchange information for SEALDD service provisioning, control, reporting etc.

* * * Next Change * * * *

#### 7.5.3.x	SEALDD-X (Between SEALDD server and other SEAL server)

The following cardinality rules apply for the reference of SEALDD-X:

a)	One SEALDD server may communicate with one or more SEAL server(s) concurrently.

* * * Next Change * * * *

#### 9.11.2.1	SEALDD Background data transfer subscription

Figure 9.11.2.1-1 illustrates the procedure for SEALDD Background data transfer subscription.

Figure 9.11.2.1-1: SEALDD Background data transfer

1.	The VAL server sends a SEALDD Background data transfer subscription request to the SEALDD server. The request includes the VAL service ID, a list of VAL UE IDs or VAL group ID, the data volume per UE and may also include the desired time window, the desired area information (e.g., when a UE enters a geographical area), Policy Selection Guidance.

2.	The SEALDD server authorizes the request from the VAL server.

3.	The SEALDD server starts the BDT configuration negotiation with the SEAL NRM server as described in clause 14.3.13 of TS 23.434 [4].

NOTE:	If the NRM server is an external control plane functionality for the SEALDD server, then the SEALDD server uses step 3, otherwise SEALDD server can use procedure defined in 3GPP TS 23.502 [6] clause 4.16.7.2.

4.	The SEALDD server sends the SEALDD Background data transfer subscription response, containing the result of the operation which includes the result and may include the BDT subscription identifier.

5.	If the SEALDD server determines that the negotiated BDT policy applies to ongoing sessions, the SEALDD server performs Network resource adaptation negotiation with the SEAL NRM server as described in clause 14.3.3.3 of TS 23.434 [4]. Policies are set for the BDT data transfer using the previously negotiated BDT Reference ID. This step may be performed simultaneously with step 7.

6.	If the SEALDD server determines that the negotiated BDT policy applies to future sessions, the SEALDD server performs the Set BDT policy for future sessions as specified in clause 4.15.6.8 of 3GPP TS 23.502 [6].

7.	The VAL server uses clause 9.5.2 procedure to send the BDT data to the SEALDD server for DL centric direction. During the negotiated transmission time window, the SEALDD server sends the BDT data to SEALDD client(s) and monitors the quality, e.g., latency, jitter, bitrate, packet loss rate, of the transmission. The SEALDD client forwards the BDT data received to the VAL client.

Equivalently, for UL centric direction, during the negotiated transmission window, the SEALDD client(s) sends the BDT data to the SEALDD server. The SEALDD server forwards the BDT data received to the VAL server.

8.	The SEALDD server may notify the VAL server about:

i.	the delivery result, by sending the SEALDD Background data transfer notification including the BDT subscription identifier and optionally the list of VAL UEs which received/sent the BDT data, the executed time window and quality of the BDT data transmission based on the measurements in step 5; or

ii.	the renogiated granted time window or the removal of BDT policy, if there is any change in the network impacting the current BDT policy and the SEAL NRM server performs BDT policy re-selection as described in in cl 14.3.13.3 of 3GPP TS 23.434 [4].

* * * Next Change * * * *

#### 9.11.2.2	SEALDD Background data transfer subscription update

Figure 9.11.2.2-1 illustrates the procedure for SEALDD Background data transfer subscription update from VAL server to the SEALDD server.

Pre-condition:

-	The SEALDD Background data transfer subscription procedure has been performed.

Figure 9.11.2.2-1: SEALDD Background data transfer subscription update

1.	The VAL server can trigger a BDT update by sending a SEALDD Background data transfer subscription update request. The request contains BDT subscription identifier and may contain e.g. updated desired time window, desired area information, Policy Selection Guidance, updated data volume per UE.

2.	The SEALDD server starts a new BDT configuration negotiation with the SEAL NRM server as described in clause 14.3.13.5 of TS 23.434 [4].

NOTE:	If the NRM server is an external control plane functionality for the SEALDD server, then the SEALDD server uses step 3 otherwise SEALDD server can directly use procedure defined in 3GPP  TS 23.502  [6] clause 4.16.7.2

3.	The SEALDD server sends the SEALDD Background data transfer subscription update response to the VAL server, containing result and the BDT subscription identifier, and, if updated, the granted time window.

4.	If the SEALDD server determines that the negotiated BDT policy applies to ongoing sessions, the SEALDD may perform Network resource adaptation negotiation with the SEAL NRM server as described in clause 14.3.3.3 of TS 23.434 [4] for the updated BDT subscription. This step may occur simultaneously with the deivery of the BDT data.

5.	If the SEALDD server determines that the negotiated BDT policy applies to future sessions, the SEALDD server may perform the Set BDT policy for future sessions as specified in clause 4.15.6.8 of 3GPP TS 23.502 [6].

* * * Next Change * * * *

#### 9.11.2.3	SEALDD Background data transfer unsubscribe

Figure 9.11.2.3-1 illustrates the procedure for SEALDD Background data transfer unsubscribe from the VAL server to the SEALDD server.

Pre-condition:

-	The SEALDD Background data transfer subscription procedure has been performed.

Figure 9.11.2.3-1: SEALDD Background data transfer unsubscribe

1.	The VAL server may trigger a BDT deletion by sending a SEALDD Background data transfer unsubscribe request to the SEALDD server. The request contains the BDT subscription identifier.

2.	The SEALDD server requests the deletion of the BDT configuration to the SEAL NRM server as described in clause 14.3.13.6 of TS 23.434 [4] and deletes any stored application data related to the BDT subscription identifier.

3.	The SEALDD server sends the SEALDD Background data transfer unsubscribe response to the VAL server, containing result of the operation.

* * * Next Change * * * *

#### 9.11.3.1	SEALDD Background data transfer subscription request

Table 9.11.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD background data transfer request.

Table 9.11.3.1-1: SEALDD Background data transfer subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | Identity of the VAL service for which the background data transfer is requested. |
| List of VAL UE IDs | O (See NOTE) | List of VAL UE IDs for which the transfer policy applies. |
| VAL group ID | O (See NOTE) | VAL group ID for which the transfer policy applies. |
| Desired time window | O | Desired time window for the background data transfer. |
| Data volume per UE | M | Expected data volume for the background data transfer. |
| Desired area information | O | Desired geographical area for the background data transfer. |
| Policy Selection Guidance | O | List that includes guidance in selecting from multiple transfer policies provided by underlying network. Possible values include: ”lowest cost”, “highest throughput given maximum cost of X”, etc. If not included, local and ASP-provided policies decide one among multiple transfer policies. |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * Next Change * * * *

#### 9.11.3.2	SEALDD Background data transfer subscription response

Table 9.11.3.2-1 describes the information flow from the SEALDD server to the VAL server.

Table 9.11.3.2-1: SEALDD Background data transfer subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure. |
| subscription identifier | O | Indicates the Background data transfer subscription identifier. Applicable for successful result. |
| Granted time window | O | Granted time window for the background data transfer. Applicable for successful result. |

* * * Next Change * * * *

#### 9.11.3.3	SEALDD Background data transfer notification

Table 9.11.3.2-1 describes the information flow from the SEALDD server to the VAL server.

Table 9.11.3.3-1: SEALDD Background data transfer notification

| Information element | Status | Description |
| --- | --- | --- |
| subscription identifier | M | Indicates the Background data transfer subscription identifier |
| List of VAL UE IDs | O | List of VAL UE IDs or VAL group ID. |
| Executed time window | O | The actual time window the BDT data was transferred. |
| Granted time window | O | If BDT renegotiation takes place, and the previously provided granted time window changed, the current granted time window for the background data transfer. |
| BDT transmission quality | O | Indicates the transmission quality (e.g., QoE) when transferring the BDT data between SEALDD client and SEALDD server. |

* * * Next Change * * * *

#### 9.11.3.4	SEALDD Background data transfer update subscription request

Table 9.11.3.4-1 describes the information flow from the VAL server to the SEALDD server for sending the SEALDD background data transfer update subscription request.

Table 9.11.3.4-1: SEALDD Background data transfer update subscription request

| Information element | Status | Description |
| --- | --- | --- |
| subscription identifier | M | Indicates the Background data transfer subscription identifier. |
| Desired time window | O (See NOTE) | Desired time window for the background data transfer. |
| Desired area information | O (See NOTE) | Desired geographical area for the background data transfer. |
| Policy Selection Guidance | O (See NOTE) | List that includes guidance in selecting from multiple transfer policies provided by underlying network. Possible values include: ”lowest cost”, “highest throughput given maximum cost of X”, etc. |
| Data volume per UE | O (See NOTE) | Updated expected data volume for the background data transfer. |
| NOTE: At least one of these IEs shall be present in the message. |  |  |

* * * Next Change * * * *

#### 9.11.3.5	SEALDD Background data transfer update subscription response

Table 9.11.3.5-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD Background data transfer update subscription request.

Table 9.11.3.5-1: SEALDD Background data transfer update subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure. |
| subscription identifier | O | Indicates the Background data transfer subscription identifier. Applicable for successful result. |
| Granted time window | O | Granted time window for the background data transfer. Applicable for successful result. |

* * * END of Changes * * * *