3GPP TSG-SA WG6 Meeting #63	S6-244356

Hyderabad, India 14th – 18th October 2024	(revision of S6-244302)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0110 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEALDD enabled data transmission quality guarantee using a Non-3GPP RAT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_PH2 |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This procedure solves the NOTE2 in clause 9.9.2.1 NOTE 2: This procedure cannot solve the QoS deterioration issue caused by NG-RAN (e.g. RAN congestion). The 3GPP access may get congestion, or RAN performance can be degraded due to the large number of users, or user entering in a very high foliage or large obstcales, indoors where coverage is very bad. This degradation in RAN impacts the SEALDD data transmission quality. Therefore, the SEALDD server should mitigate the effects of the 3GPP RAN congestion to meet the data transmission quality requirements and application layer QoS. In order to solve the congestion effects and maintain the transmission quality, the traffic can be offloaded or served via available non-3GPP access. It is also evident the WLAN deployment is ubiqutious,especially in Indoors and for the usecases like XR, where XR devices are equipped with WLAN capability and low latency,high data rate requirements, it is important to offload traffic from 3GPP access to WLAN access points, closer to the XR users. Another scneario for users which are stationary, or low activity period, offload to closest WLAN could help the network and UE to save energy. This procedure enables the capability for the SEALDD server to use the available non-3gpp access for the SEALDD data connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new procedure to use Non-3GPP access(WLAN) based on analytics and measurements to switch the DD connection from 3GPP access to Non-3GPP access in the event of RAN congestion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEALDD data transmission connection quality would be degraded and applications QoS would not be met |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.2.x(new), 9.7.2.3, 9.7.3.6, 9.7.3.8, 9.10.3.1, 9.2.3.3, 9.2.2.6, 9.2.3.7, 9.2.3.10, 9.2.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.9.2.x	SEALDD enabled data transmission quality guarantee using Non-3GPP access

Figure 9.9.2.x-1 illustrates the procedure of using non-3gpp access for SEALDD enabled data transmission quality guarantee.

Pre-conditions:

- The SEALDD Client is authorized to use non-3GPP access on behalf of the VAL client.

Figure 9.9.2.x-1: SEALDD enabled data transmission quality guarantee using Non-3GPP access

- A regular data transmission connection is established according to clause 9.2.2.2 or via clause 9.3.2.3 and SEALDD data transmission measurement reporting as per clause 9.7.2.1 or clause 9.7.2.2 to get the data transmission quality.

- If Non-3GPP access measurement policy is available in the SEALDD server, the SEALDD server can request the SEALDD client to report the latest Non-3GPP access measurements(like RSRP, RSRQ, RSSI, SINR for the WLAN SSIDs) as per steps 3-4 of the procedure defined in clause 9.2.2.6.

If Non-3GPP access measurement policy is not available, the SEALDD server fetches the UE location using the SEAL location service defined in 3GPP TS 23.434 [4] clause 9.3.12  and requests the SEALDD client to report the latest Non-3GPP access measurements of the nearby WLAN SSIDs. The SEALDD server uses the steps 3-4 of the procedure defined in clause 9.2.2.6 to get the report from the SEALDD client.

If the Non-3GPP access measurement values in the data transmission quality report or SEALDD connection status report defined in clause 9.2.2.6 are above the Signal strength thresholds mentioned in SEALDD Non-3GPP access measurement policy then the SEALDD server can offload the SEALDD-UU connection from 3GPP access to Non-3GPP access(WLAN) using steps 5-6.

- The SEALDD server subscribes to NWDAF analytics service for subscription to network performance analytics with Analytics ID as Network Performance using the procedure described in clause 6.6.4 3GPP TS 23.288. It also subscribes to PDU Session traffic analytics service using the procedure defined in clause 6.20 3GPP TS 23.288.

The SEALDD server can subscribe to WLAN performance analytics using the procedure defined in clause 6.11 3GPP TS 23.288. The SEALDD server uses the SEALDD Non-3GPP access measurement policy information or information received in step 2 for subscription to WLAN performance analytics.

- Based on SEALDD client Non-3GPP access measurement reports, NWDAF analytics measurement reports(including predicted values), and data transmission quality measurement reports, the SEALDD server detects the 3GPP RAN congestion and determines to use non-3GPP access to meet the data transmission quality requirements.

- The SEALDD server can request 5GS to create or update URSP rules using clause 4.15.6.10 of 3GPP TS 23.502 [6] to establish a PDU session connection over Non-3GPP access. The SEALDD server can create a URSP rule with Access Type Route selection descriptors as non-3GPP to establish PDU session. The request may include the UE ID and application traffic descriptor containing the addresses or ports allocated by SEALDD server.

- The UE receives the new or updated URSP rules from the 5G core network. Based on the URSP rules, the UE establishes a PDU session connectivity over non-3GPP access with the 5GS.

- On the successful establishment of PDU session connection over Non-3GPP access, the SEALDD client updates the SEALDD data transmission connection with the updated SEALDD client IP address as per steps 7-8 mentioned in clause 9.3.2.2.

Based on continuous monitoring and measurement reports, the SEALDD server can decide to switch back to 3GPP access and use the SEALDD-UU connection over 3GPP access for data transmission.

#### 9.7.2.3	Data transmission quality measurement reported by SEALDD client

Figure 9.7.2.3-1 illustrate the procedure for SEALDD enabled data transmission quality measurement for VAL traffic. The SEALDD client receives transmission quality measurement requirement, decides to start VAL data transmission monitoring and generates measurement reports.

Figure 9.7.2.3-1: VAL data transmission quality measurement reported by SEALDD client

1.	An on-going regular data transmission connection is established according to clause 9.2.2.2.

The transmission quality measurement can be triggered by VAL server or VAL client, which is described in step 2 to step 5 and step 6, correspondingly.

The request may include the crossflow measurement information (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2/DL]}) associated in the same multi-modal service for crossflow RTT measurement.

2.	The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), and may also include reporting criteria, reporting frequency, spatial condition and temporal condition.

NOTE 1:	The spatial and/or temporal condition can be used by SEALDD client to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday.

3.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server responds to the VAL server.

4-5.	The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server. The SEALDD client, based on the received service quality guarantee policy including thresholds and action, can take corrective action as described in clause 9.7.2.3.

If crossflow RTT measurement requirement is received from the VAL server, the SEALDD server identifies the SEALDD connections for the involved VAL UEs and sends to the SEALDD client(s) with crossflow RTT measurement requirement including corresponding UL/DL flow information.

The SEALDD server sends the Non-3GPP access measurement information in the SEALDD transmission quality measurement subscription request to the SEALDD client.

6.	The VAL client triggers the SEALDD transmission quality measurement procedure to the SEALDD client, in order to collect the measurement report information.

7.	After SEALDD client determines to start measurement process, upon UL packet arrival, the SEALDD client initiates the UL packet delay measurement. The SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload for VAL data transmission quality monitoring) with local time T1 when the SEALDD client sends out the UL monitoring packet. The SEALDD client considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD client stops/suspends the transmission quality measurement.

For crossflow RTT measurement, the SEALDD client received UL flow information starts recording local time T1 when sending out the 1st UL packet matching the UL flow information to the SEALDD server. The T1 is sent in the encapsulated UL monitoring packet to the SEALDD server.

8.	The SEALDD server receives the UL monitoring packet, and records the local time T2.

9.	Similarly, the SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring in case there is no DL VAL traffic for DL packet delay monitoring) with local time T2 recorded in step 8 and local time T3 when the SEALDD server sends out the DL monitoring packet.

NOTE 2:	When the SEALDD server sends the dummy UL packet as monitoring response to the SEALDD client depends on SEALDD server implementation.

For crossflow RTT measurement, when the 1st DL packet matching the received DL flow information is to be sent, the SEALDD server encapsulates the DL monitoring packet with previously received T1 and sends the DL monitoring packet to the SEALDD client.

10.	The SEALDD client records the local time T4 when the SEALDD client receives the DL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD client can also calculate the bitrate and jitter over a certain period over a specific SEALDD connection by recording the status of the SEALDD monitoring packets. The SEALDD client also evaluates the reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report.

For crossflow RTT measurement, if T1 is received in the DL packet, the SEALDD client records local time T2 and calculates RTT based on T1 and T2.

Depending on which entity triggers the data transmission quality measurement, step 11 and step 12 corresponds to step 2 to step 5, step 13 corresponds to step 6.

For the Non-3GPP access measurement, if the Non-3GPP access measurement information includes list of WLAN SSIDs, then the SEALDD client performs the signal strength measurement for the requested WLAN SSID. If the request contains location-based measurement then the SEALDD client measures the signal strength of nearby WLAN SSIDs based on its location.

11-12.	The SEALDD client reports the data transmission quality measurement results (e.g. latency, jitter, bitrate) to the VAL server via the SEALDD server. The report also includes the list of WLAN SSIDs and their signal strength measurements (like RSRP, RSRQ, SINR, RSSI).

13.	The SEALDD client reports the data transmission quality measurement results to the VAL client.

NOTE 3:	The crossflow RTT measurement calculated by the SEALDD client in step 10 can be used as an estimation of the lower bound for the Motion-to-Photon latency in the XR scenarios.

When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 11 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server collects and aggregates the needed report for the VAL server.

#### 9.7.3.6	Transmission quality measurement subscription request

Table 9.7.3.6-1 describes the information flow from the SEALDD server to the SEALDD client for data transmission measurement subscription.

Table 9.7.3.6-1: Transmission quality measurement subscription request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD-UU flow ID | M | Identifier of the SEALDD-UU flow. |
| Measurement conditions | O | Indicates the temporal and/or spatial conditions. |
| Transmission quality measurement requirements list | M | The measurement requirement information |
| > Measurement ID | M | Measurement identifiers, e.g. latency, bitrate, jitter, signal strength (e.g.RSRP, SINR, RSRQ, RSSI) |
| > Reporting frequency | O | The reporting frequency of measurement results (e.g. periodic reporting). If not present, it implies periodic reporting. |
| > Reporting periodicity | O | If the reporting frequency is periodic, the reporting periodicity shall be provided. |
| > Measurement period window | O | Indicates the measurement period window for transmission quality measurements |
| > Measurement expiration time | O | Indicates the measurement expiration time |
| > Reporting criteria | O | Indicates the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criteria of more than one criteria is specified. |
| > SEALDD policy | O | Specifies quality guarantee policies associated with the SEALDD connection |
| >> Quality guarantee policy | M | Indicates the event (e.g. measurement threshold) to be measured for, the quality guarantee. |
| >> Non-3GPP access measurement information | O | Indicates the non-3GPP access(like WLAN) measurement policy (e.g. WLAN SSIDs, BSSID, location-based measurement) |

#### 9.7.3.8	Transmission quality measurement notification

Table 9.7.3.8-1 describes the information flow from the SEALDD client to the SEALDD server for notifying the transmission quality measurement reports.

Table 9.7.3.8-1: Transmission quality measurement notification

| Information element | Status | Description |
| --- | --- | --- |
| Transmission quality measurement reports list | M | The generated transmission quality results in SEALDD server |
| > Measurement ID | M | Measurement identifiers, e.g. latency, bitrate, jitter, signal strength(RSRP, SINR, RSSI, RSRQ) |
| > Average measurement value | O | The average measurement value of measurement results |
| > Minimum measurement value | O | The minimum measurement value of measurement results |
| > maximum measurement value | O | The maximum measurement value of measurement results |
| > Standard deviation measurement value | O | Standard deviation measurement value of measurement results |
| > kPercentile measurement value | O | Indicates the kpercentile measurement value of measurement results |
| > Measurement period | O | Indicates the measurement period |
| > Timestamp | O | Indicates the timestamp of measurement results |
| > Non-3GPP access measurement information | O | Indicates the measured WLAN access(e.g. SSID, BSSID) and signal strength values |

#### 9.10.3.1	SEALDD policy configuration request

Table 9.10.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration.

Table 9.10.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| -Identity | O | Identifier of the VAL UE or VAL user for which SEALDD policy applies |
| SEALDD policy | O (See NOTE 4) | The SEALDD policy associated with application traffic identifiers, VAL UE identity |
| > Quality guarantee policy | O (See NOTE 1) | Indicates the event (e.g. measurement threshold) to be measured for the quality guarantee |
| > Quality optimization policy | O (See NOTE 1) | Indicates the data transmission adjustment policy (e.g., adjustment need) to be performed in SEALDD layer. |
| > Bandwidth control policy | O (See NOTE 2) | Indicate the bandwidth control preference, e.g. re-allocating the bandwidth limit between different VAL users, including UL/DL |
| > Geofence policy | O (See NOTE 3) | Indicates the geofence policy per VAL service |
| >> Geofence location | O (See NOTE 3) | Indicates the geofence location information for the mentioned VAL service |
| >>Geofence policy action | O (See NOTE 3) | Indicates the policy action like allowed or blocked |
| > temporal policy | O (See NOTE 3) | Indicates the time period for which the sealdd traffic is allowed |
| > policy expiration time | O | Indicates the validity of the policy |
| Multi-modal SEALDD policy | O (See NOTE 4) | Multi-modal SEALDD policy associated with set of individual SEALDD flows. |
| > Synchronization policy | O | Indicates the synchronization threshold for multi-modal application, as specified in 3GPP TS 22.261 [2] |
| > Policy expiration time | O | Indicates the validity period of the policy |
| > Non-3GPP access measurement policy | O | Indicates the non-3GPP access(like WLAN) measurement policy (e.g. WLAN SSIDs/BSSID, Signal strength thresholds(high/low), time based threshold for offload to WLAN, |
| NOTE 1: This IE is used for the SEALDD enabled transmission quality guarantee, as specified in clause 9.9. NOTE 2: This IE is used for the SEALDD enabled bandwidth control, as specified in clause 9.8. NOTE 3: This IE is used for the SEALDD connection establishment and data delivery, as specified in clause 9.2 NOTE 4: At least one of these IEs shall be present. |  |  |

#### 9.2.3.3	SEALDD regular transmission connection establishment request

Table 9.2.3.3-1 describes the information flow from the SEALDD client to the SEALDD server or from the SEALDD server to the SEALDD client for requesting the regular SEALDD connection establishment.

Table 9.2.3.3-1: SEALDD regular transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD flow ID | M (See NOTE 1) | Identity of the SEALDD flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side (for client side initiated request) or the SEALDD server side (for server side initiated request) used to establish SEALDD connection. |
| VAL UE/user identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime, applicable for SEALDD server side initiated request. |
| Capability for BAT and periodicity adaptation | O (See NOTE 2) | Indicates BAT and periodicity adaptation capability for SEALDD client in client side initiated request. |
| Transmission assistance info | O (See NOTE 2) | Indicates transmission assistance information for uplink SEALDD traffic in client side initiated request. It includes BAT, BAT window, periodicity, and periodicity range. |
| L4S feedback capability | O (See NOTE 3) | Identifies the L4S feedback capability (i.e. ECN identification, L4S feedback) for client side initiated request |
| VAL UE client access capability | O | Indicates the access technologies supported by VAL UE client. E.g.,3GPP, Non-3GPP(WLAN) |
| NOTE 1: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. NOTE 2: Only one of these IEs may be present in the message. NOTE 3: This IE is used for the SEALDD enabled congestion control for VAL applications, as specified in clause 9.8.2.2. |  |  |

#### 9.2.2.6	SEALDD connection status procedure

Figure 9.2.2.6-1 illustrates the procedure for SEALDD connection status from the VAL server to the SEALDD server.

Figure 9.2.2.6-1: SEALDD connection status procedure

1.	The VAL server sends the SEALDD connection status subscribe request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE identity, and the SEALDD client connection status check periodicity under "SEALDD client connection status" event as described in Table 9.2.3.7-1. The VAL server can also include the Non-3GPP access measurement information(like list of WLAN SSIDs, location-based measurement) in the request message.

2.	The SEALDD server subscribes to the NEF UE reachability, Application Detection and Loss of connectivity events using the procedure defined in clause 4.15.3.2.3b TS 23.502 [6]. It can also use the NRM Event monitoring procedure defined in clause 14.3.6.2.2 TS 23.434 [4].

3.	The SEALDD server also sends a SEALDD client connection status reporting configuration request to the SEALDD client. The request message consists of reporting interval, SEALDD-UU flow ID, and method of reporting. The SEALDD-UU flow ID identifies the application traffic flow for which the reporting notification is configured. The SEALDD client monitors the application using SEALDD-UU flow ID. The method of reporting is defined as regular or irregular. The regular reporting method uses the reporting interval to send the notification. The irregular reporting method sends the notification if the current connection status is different from the previous connection status or if the application state changes (like crash, close, stop).

The SEALDD server can include the Non-3GPP access measurement in the SEALDD client connection status reporting configuration request. The SEALDD server can use the Non-3GPP access measurement information received in step 1 or Non-3GPP access measurement information configured in the SEALDD server as policy by VAL server or configured as internal policy.

4.	The SEALDD client configures the reporting configuration and provides the response to the SEALDD server.If the Non-3GPP access measurement includes list of WLAN SSIDs, then the SEALDD client performs the signal strength measurement for the requested WLAN SSID. If the request contains location-based measurement, then the SEALDD client measures the signal strength of nearby WLAN SSIDs based on its location. The response also includes the list of WLAN SSIDs and their signal strength measurements.

5.	The SEALDD server sends the connection status subscription response to the VAL server.

6a-6b. The SEALDD server receives the notification for the UE connection status from the subscribed NEF, NRM. It can also receive a notification from the SEALDD client regarding the connection status. If the UE is in power saving mode, then the SEALDD client sends the connection status reporting notification with the status as sleeping to the SEALDD server and suspends the connection status periodic reporting.

7.	Based on the NRM, NEF event subscription response and SEALDD client connection status reporting notification message with "SEALDD client connection status" event as described in Table 9.2.3.9-1, the SEALDD server processes the responses and sends the SEALDD client connection status of unreachable or sleeping status notification to the VAL server.

#### 9.2.3.7	SEALDD connection status subscription request

Table 9.2.3.7-1 describes the information flow from the VAL server to SEALDD server to subscribe to SEALDD connection status information.

Table 9.2.3.7-1: SEALDD connection status subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| Event ID list | M | Identifies a list of events such as establishment, release, congestion report, SEALDD client connection status. |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of VAL UE or VAL user. |
| SEALDD-S Data transmission connection information | M | Address/port of the VAL server to send/receive the application packets to/from the SEALDD server. |
| Immediate reporting flag | O | Indicates the immediate reporting of connection status notification |
| SEALDD client connection status check periodicity | O | Indicates the frequency to perform SEALDD client connection status check |
| Non-3GPP access measurement information | O | Indicates the Non-3GPP access measurement information like list of WLAN SSIDs, location-based measurement |

#### 9.2.3.10	SEALDD client connection status reporting configuration request

Table 9.2.3.10-1 describes the information flow from the SEALDD server to SEALDD client to configure the SEALDD client for connection status reporting.

Table 9.2.3.10-1: SEALDD client connection status reporting configuration request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD-UU Flow ID | M | Indicates the SEALDD-UU flow ID of the application for which the reporting is required |
| Mode of reporting | O | Indicates the mode of reporting. e.g. regular or irregular |
| > Reporting interval | O (See NOTE) | Indicates the reporting interval to report the notification |
| SEALDD client connection status reporting priority | O | Indicates the priority of SEALDD client connection status for the requested SEALDD-UU flow ID |
| Non-3GPP access measurement information | O | Indicates the Non-3GPP access measurement information for the SEALDD client to perform measurement. It contains list of WLAN SSIDs or location-based reporting and signal strengths (e.g., RSRP, RSSI, RSRQ, SINR) values for measurement |
| NOTE: This IE is only present if the mode of reporting is regular |  |  |

#### 9.2.3.12	SEALDD client connection status reporting notification

Table 9.2.3.12-1 describes the information flow from the SEALDD client to the SEALD server for sending the the SEALDD client connection status reporting configuration response.

Table 9.2.3.12-1: SEALDD client connection status reporting notification

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client connection status | M | Indicates the status of VAL UEs/users like reachable, unreachable, or sleeping |
| Non-3GPP access measurement information reporting | O | Indicates the Non-3GPP access measurement information report. It contains measured list of WLAN SSIDs and signal strengths(e.g., RSRP, RSRQ, RSSI, SINR) values |
| SEALDD data tranmssion connection access usage | M | Indicates which access(3GPP or Non-3GPP) is used for the SEALDD-UU data transmission |