3GPP TSG-SA Meeting #106	SP-241934

Madrid, Spain, 10-13 December 2024

Title:	Reply LS on clarifications on consent management.

Response to:	LS on clarifications on consent management from GSMA OPG (SP-241427)

Release:	Rel-18

Source:	3GPP TSG SA

To:	GSMA OPG

Cc:	3GPP SA WG2, 3GPP SA WG3, 3GPP SA WG6, 3GPP CT WG4, 3GPP CT WG3

Contact person:	Alessio Casati

alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

3GPP TSG-SA thanks GSMA OPG for their LS on clarifications on consent management.

3GPP TSG SA would like to clarify that 3GPP TSG SA WG6 has specified the support of RNAA (Resource owner-aware Northbound API Access), as part of CAPIF in Rel.18 in TS 23.222, and the detailed security specification for CAPIF RNAA has been specified by SA3 in TS 33.122 (CAPIF). 3GPP TSG SA WG SA6 completed a Rel.19 study, which includes in its scope further enhancements to CAPIF for resource owner authorisation management (RNAA). GSMA OPG is invited to provide any feedback to the conclusions of the studies captured in TR 23.700-22 (SA6) and TR 33.700-22 (SA3).

In addition to above comments, the following answers are provided to the received questions.

Q1. When NEF or EES (as trusted AF) is exposing APIs using CAPIF RNAA, how is NEF or EES utilizing the UDM’s user consent information for processing authorization for API consumer/invoker?

3GPP TSG SA Answer: see answers in Q2.

3GPP user consent framework, Annex V of TS 33.501: User Consent Subscription Data (UcSubscriptionData) is specified in 3GPP TS 29.503 as subscription data stored in the UDR. This data can be retrieved from UDR by UDM and from UDM by any other authorized NF (e.g., NWDAF, NEF). Permanent subscription data within this framework can be modified only by provisioning/administration means. User Consent Subscription Data is used by the NF which is deemed an enforcement point for user consent. The user consent parameters, which are bound to a SUPI/GPSI and purpose of data processing, include whether the user consent is granted or not. Annex V provides technical means to be used when it is required by regional regulations or operator’s local policy, not otherwise. The user consent framework as defined in TS 33.501 relates to user consent data to be used within the 5G Core.

The CAPIF RNAA mechanism is about getting authorization from the resource owner for accessing protected resources per user (resource owner identifier), per application (API invoker identity), per API, and per scope and purpose (of data processing) in support of the framework. This could be used to authorize accessing privacy-related (sensitive) information. CAPIF can serve both 3GPP and non-3GPP (CAPIF) API Exposing Functions and the resource owner can be a user or a subscription owner. Rel-18 CAPIF RNAA specification only states that authorization-related information is available in the CCF. There is no further specification about the storage of the information.

Q2. What is the relationship between CAPIF RNAA and UDM’s user consent information? Is there any plan/roadmap for a unified approach?

3GPP TSG SA Answer:

There is currently no relationship. Whether a unified approach related to user consent can be achieved would require additional study in 3GPP. 3GPP SA will keep GSMA OPG informed.

A NOTE in 3GPP TS 23.222 is reported below for information:

NOTE: 	The authorization information from the resource owner used by CCF (described in 3GPP TS 33.122) is independent from the user consent information used from user subscription data at UDM/UDR (described in Annex V of 3GPP TS 33.501). In the current release of 3GPP specifications, synergy between CCF and UDM is not specified.

GSMA OPG is further invited to provide any feedback to the resource owner authorization enhancements specified in 3GPP TS 23.222 for Rel-19.

Q3. For the UDM’s user consent information, are the user consent management aspects (e.g. capturing or revoking user consent from the subscriber) specified? Please illustrate.

3GPP TSG SA Answer:

Please refer to the answer to Q1. In addition, how the user consent record gets created in the first place or changed later on is not part of 3GPP specification since this depends on operator practices, e.g., if there are OAM systems, Customer Care systems, etc.

Examples of user consent management for different services from 3GPP SA2 specifications are:

- The user consent aspect for MSISDN exposure to an authenticated and authorized AF is specified in clause 4.15.10A of TS 23.502.

- The UE LCS Privacy Profiles in UDM is described in clause 5.4 of TS 23.273. The UE LCS Privacy Profiles contains Location Privacy Indication, which can be provided and updated by the UE and/or AF as specified in clause 6.12 of TS 23.273.

Q4. Is there any plan/roadmap for considering other legal bases for processing personal data apart from user consent (e.g. contract, legal obligations, vital interests of the data subject, public interest, and legitimate interest [GDPR]) in a unified way?

3GPP TSG SA Answer:

Data controllers (operators) handling of personal data usually depends on legal jurisdiction ("legal basis for processing personal data"). 3GPP can only provide the technical mechanisms to enable the operators to fulfil legal aspects.

Q5. What are the privacy considerations in 3GPP with respect to exposure of sensitive information (e.g. UE ID, location) to untrusted AFs.

3GPP TSG SA Answer:

3GPP can only provide technical mechanisms to store sensitive information securely and to restrict access to those authorized.

# 2	Actions

To: GSMA OPG

ACTION: 	3GPP TSG SA kindly asks GSMA OPG to consider the above information and provide any feedback.

# 3	Dates of next TSG SA meetings

3GPP SA#107   	March 12th -14th "2025    	Incheon, Korea

3GPP SA#108 	June 10th -13th "2025    	Prague, Czech Republic