3GPP TSG SA WG 1 Meeting #108 	S1-244101

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	Novamint, Viasat, Inmarsat

pCR Title:	Pseudo-CR on update 5.19

Draft Spec:	3GPP TR 22.887 v0.2.0

Agenda item:	7.3

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This contribution proposes to update the use case 5.19  of TR.22887 to remove the FFS and to fix some text issues

1. Introduction

In TR 22.887, the use case 5.19  has still some FFS to be addressed

2. Reason for Change

To replace “satellite system” by “satellite access type” for [PR 5.19.6-001] allowing to clarify the requirment and to remove the FFS for the use case 5.19 of  TR 22.887

+ additional changes for replacing GEO par GSO and replacing handover by mobility plus to update the texts to use the proper terms

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887 v0.2.0.

* * * First Change * * * *

## 5.19	Use case on using GSO for sleep/power management

### 5.19.1	Description

Many UEs are power limited and need to conserve power to extend battery life.  In some cases, UEs receive (or have to send) packets very infrequently.  In these cases, it is wasteful of the UE to continuously maintain and establish sync with a NGSO satellite that needs to be tracked through the sky.  A GSO satellite can be very useful for these UEs.  The UE would only try to connect to a NGSO satellite if the GSO satellite told it to do so.  The GSO satellite would only tell the UEs to connect to the NGSO if the network informed the GSO that packets were waiting for the UE.  In cases where the UE needs to send a packet, it would either send it directly to the GSO, or request (via the GSO) resources from the network on a NGSO satellite.

### 5.19.2	Pre-conditions

- The network supports both GSO and NGSO satellites

- The network supports mobility between GSO satellite(s) access and NGSO satellite(s) access

- A network plan that deconflicts frequency allocations between the GSO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The different satellite orbits employed by the network may be supported by the different satellite access types

- The network should be aware of when to tell the UE to move to a NGSO satellite connection.

### 5.19.3	Service Flows

- UE establishes a connection with a GSO satellite

- UE enters sleep mode

- UE wakes ups periodically and re-establishes connection with a GSO

- GSO informs UE of whether it needs to wake up

- If there is no need to wake up, UE goes back to sleep

- If there is a need to establish a connection, UE exists wakes up, and establishes a connection with NGSO instead of the GSO

### 5.19.4	Post-conditions

UE is in appropriate sleep mode on GSO or monitoring communications mode on NGSO

### 5.19.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 5.19.6	Potential New Requirements needed to support the use case

[PR 5.19.6-001] Subject to regulatory requirement and operator’s policy, a 5G system with multi orbit satellite access shall be able to support a mechanism for the network to inform a UE when coverage is available and through which satellite access type it can be used.