3GPP TSG SA WG 1 Meeting #108 	S1-244297

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Use Case on communication on board of UAM aircrafts

Agenda item:	8.1.6

Source:	Huawei, HiSilicon

Contact:	Alice Li, alice.li1@huawei.com

Abstract: This contribution provides a new use case on communication on board of UAM aircrafts.

1. Introduction

“Further Use Cases on Industry and Verticals” has been identified as one of key areas for 6G study. This contribution proposes a new use case on communication on board of UAM aircrafts.

2. Reason for Change

Urban air mobility (UAM) is a new safe, secure and more sustainable air transportation system for passengers and cargo in urban environments, enabled by new technologies and integrated into multimodal transportation systems. The transportation is performed by electric vertical take-off & landing (eVTOL) aircrafts, remotely piloted or with a pilot onboard. Compared with the common UAVs (Uncrewed Aerial Aircraft), the UAM aircrafts have several major differences.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	https://www.easa.europa.eu/en/what-is-uam.

[x2]	https://www.autoflight.com/en/air/.

[x3]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[x4]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

[x5]	https://rd.hkust-gz.edu.cn/2023/05/沈向洋教授：勇闯无人区，打造智能融合低空系统si/

[x6]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[x7]	https://www.flyeye.io/drone-acronym-daa/.

* * * Second Change (new clause) * * * *

## 10.x	Use case on communication on board of UAM aircrafts

### 10.x.1	Description

Urban air mobility (UAM) is a new safe, secure and more sustainable air transportation system for passengers and cargo in urban environments, enabled by new technologies and integrated into multimodal transportation systems. The transportation is performed by electric vertical take-off & landing (eVTOL) aircrafts, remotely piloted or with a pilot onboard [x1]. In February 2024 an eVTOL named as "PROSPERITY" [x2] that can contain 5 passengers conducted a Shenzhen-Zhuhai test flight, which was a cross-sea and cross-city route. KT showed, at the MWC2024, their UAM Skypath solution, which can provide 5G service for UAM aircrafts flying at 300 – 600 meters altitude.

Compared with the common UAVs (Uncrewed Aerial Aircraft), the UAM aircrafts have the following major differences:

- large size and heavy weight, higher AGL (above ground level)

Low-altitude airspace usually refers to the airspace with a vertical distance of less than 1000m from the ground, and it can be extended to less than 3000m according to the characteristics and actual needs of different regions [x3]. The AGL of UAM aircrafts can be up to 1000m, while the AGL of small UAVs is less than 300m. Communication for the UAM aircrafts with higher AGL need to be considered.

- higher reliability and safety requirement with human beings onboard

UAM aircrafts share some common low altitude airspace. As predicted by Professor Shen, there will be about 100,000 UAVs flying in Shenzhen's sky at the same time in the future [x5]. Considering the area of Shenzhen is 1997 km2, there will be about 50 UAVs flying in 1 km2 airspace. To ensure high reliability and safety, the UAM aircrafts must be aware of the object information that near its flight trajectory.

To guarantee the safety of aircrafts, Detect and Avoid (DAA) technology is widely used in UAVs by using a combination of sensors, cameras, and radar to continuously monitor the UAV’s surroundings [x7]. These sensors detect obstacles, other aircraft, and potential hazards in the flight path. The system then processes this information in real-time and adjusts the UAV’s flight path to avoid collisions. However, the capabilities of the sensors on UAVs are not able to sense the blockage far away. Since the UAM aircrafts carry human beings on board, UAV’s DAA system is not adequate to ensure the safety of passengers onboard. By also utilizing the 3GPP sensing service, the safety of UAM aircrafts flying in the common airspace can be guaranteed, as well as the running efficiency of UAM aircraft can be improved so as to transport more passengers or goods.

Typically, the message size for sensing one object could be 1kbyte [x6], including information of size/position/speed/direction. Around 25 objects (25kbyte) per frame (20ms) need to be sensed for an aircraft. The reliability requirement for UAM aircrafts is about 99.9%.

- in addition, with passengers onboard the human communication on board of UAM has to be considered.

### 10.x.2	Pre-conditions

The UAM aircrafts are connected to the 6G network, which provides the following services:

- sensing services to the UAM aircrafts;

- communication services to the passengers onboard.

### 10.x.3	Service Flows

1.	May and Fei plan visit City B from City A by UAM, and the aircraft can contain up to 4 passengers.

2.	To guarantee the safety of the passengers onboard, the UAM aircraft requests the sensing service from the 6G network. The base station(s) along the flight path will sense the environment information especially other aircrafts within its interesting area, e.g., the interesting area is an airspace with the size of 1 square kilometre and height from 0 to 1000 meters.

3.	As part of the sensing service, the 6G network sends the sensing and/or the warning information to the UAM aircraft.

4.	Upon receipt of the above information, the UAM aircraft further process the information to identify collision threat. If there are some blockages threatening flight safety, the UAM aircraft will perform collision avoidance in advance.

5.	Meanwhile passengers onboard either watch HD video or surf the Internet using their smartphone. The view is very beautiful during the flight, May is very happy to share the scenery with her friends through 4K real-time video on social media by smartphone, e.g., WeChat. Fei is not interested in the scenery during the flight, and he is enjoying a football match live broadcast. The core network sends data of football game with 4K or 8K live broadcast to base station, then Fei can receive the football game data by his smartphone.

### 10.x.4	Post-conditions

Thanks to the sensing services and communication services provided by the 6G, May and Fei had a great onboard experience and safely arrived to the destination via the UAM aircraft.

### 10.x.5	Existing features partly or fully covering the use case functionality

From TS 22.125 [x4] “Uncrewed Aerial System (UAS) support in 3GPP”, KPIs for services provided to the UAV applications are defined, especially Table 7.1-1 KPIs for services provided to the UAV applications and Table 7.2-1 KPIs for command and control of UAV operation.

However, the message size of UAV command and control is less than <10kbyte. Latency and reliability requirement are not sufficient for UAM aircrafts. In addition, the AGL for UAV is not high enough for UAM aircrafts.

### 10.x.6	Potential New Requirements needed to support the use case

[PR 10.x.6-1] The 3GPP shall support the transmission of sensing information to the UAM aircrafts with the following KPI requirements.

|  | Typical transmission interval | Altitude AGL | Typical message Size | End to end Latency | Reliability |
| --- | --- | --- | --- | --- | --- |
| Sensing information to a UAM aircraft | 20 ms | up to 1000 m | 25 kbyte | 20 ms | 99.9% |

[PR 10.x.6-2] The 3GPP shall support services provided to the UAM applications with the following KPI requirements.

| Services | Data rate | End to end Latency | Altitude AGL | service area |
| --- | --- | --- | --- | --- |
| 8K video live broadcast | 100Mbps UAM aircraft originated (note) | 200 ms (note) | up to 1000 m | Urban, scenic area |
|  | 600Kbps UAM aircraft terminated (note) | 20 ms (note) | up to 1000 m |  |
| Video streaming | 4 Mbps for 720p video 9 Mbps for 1080p video UAM aircraft originated (note) | 100 ms (note) | up to 1000 m | Urban, rural area |
|  | 100 Mbps for 8K video UAM aircraft originated (note) | 100 ms (note) | up to 1000 m |  |
| NOTE: These values are aligned with the KPIs for services provided to the UAV applications in TS 22.261, table 7.1-1. |  |  |  |  |