3GPP TSG SA WG 1 Meeting #108 	S1-244409

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244165)

Source:	CATT，CSCN

pCR Title:	Use Case on Interworking with non-3GPP Satellite Networks for Maritime Communication

Draft Spec:	3GPP TR 22.870 v0.0.0

Agenda item:	8.1.3

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes a use case about 3GPP 6G system applied for the maritime communication supporting interworking with non-3GPP satellite system (e.g. VDES-Satellite).

1. Introduction

The support to maritime communication has been discussed in 5G and focused on the assistance in mission critical communication at that moment. As the evolution of dedicated maritime communication system, and rapid increasing need of data communication, 6G is expected to improve the support to maritime communication capable of interworking  with dedicated maritime communication system, especially dedicated satellite network. This contribution proposes a use case about 3GPP 6G system applied for the maritime communication supporting the interworking with maritime satellite network VDES-Satellite.

2. Reason for Change

Introduce a new use case for the interworking between 3GPP system and non-3GPP satellite network for martime communication.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR22.870 v0.0.1

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	IALA Guideline G1117: "VHF DATA EXCHANGE SYSTEM (VDES) OVERVIEW".

* * * Next Change * * * *

7	Ubiquitous Connectivity

7. x	Use case on interworking between multiple satellite networks for maritime communication

7.x.1	Description

As the footprint of human activities and exploration to the ocean extends in recent decades, the importance of maritime communication becomes increasing apparent. Under International Maritime Organization(IMO) regulatory framework, traditional dedicated maritime communication systems such as Global Maritime Distress and Safety System (GMDSS), Automatic Identification System (AIS) are mainly designated to satisfy the basic needs of safety, security and efficient navigation of the shipping while other wireless communication systems such as 4G/5G, WiFi can complement additional needs of the on-board crews’ communication and off-shore communication with coast administration center independently. Driven by the digital transformation and emerging indications of AIS overload, VHF data exchange system (VDES) was introduced for maritime mobile communication services with the combination of terrestrial and satellite components for ship-ship and ship-shore communications[x]. But the digital divide between sea and land communication is still prominent due to the limited broadband capability, insufficient coverage and high cost of satellite application. The introduction of NTN in 5G enables the potential to address broader use cases of ship communication via public IMT system. 6G is expected to improve the cooperation of dedicated maritime communication system and public IMT system to provide ubiquitous connectivity and reliable services in the fields of vessel traffic management, port operation and management, maritime safety and rescue, fisheries and aquiculture, and ocean oil drilling.

Both VDES and legacy 3GPP network support multiple RATs, including terrestrial access and satellite access. The 6G system should have the capability to connect various heterogeneous access networks, including 3GPP terrestrial, 3GPP satellite and non-3GPP satellite access networks, in order to allow the operator and/or authorized 3rd party to make best use of network infrastructures for diverse service requirements of the maritime communication such as Search and Rescue (SAR) services, navigation information exchange, sensor data collection and etc.

As the end user (e.g. vessels, crews, autonomous surface ships) moves along the coast, in port and at sea, 6G system is expected to support mobility between 3GPP network and non-3GPP satellite network with minimum interruption on service continuity, enable the UE to select proper non-3GPP and/or 3GPP accesses to connect to under network control, simultaneous connections to 3GPP and non-3GPP accesses for the same or different services, authorization and authentication for accessing a 3GPP network using non-3GPP credential, etc.

In addition to the extension of the coverage and service availability, 6G system should allow the operator to improve the efficiency of overall network resource and network robustness for better user experience via the close interworking between 3GPP and non-3GPP access networks e.g. offload the traffic from VDES-satellite to 3GPP satellite network.

7.x.2	Pre-conditions

Not applicable

7.x.3	Service Flows

Not applicable

7.x.4	Post-conditions

Not applicable

7.x.5	Existing features partly or fully covering the use case functionality

Regarding TS 22.261,

The 5G system shall support a set of identities for a single user in order to provide a consistent set of policies and a single set of services across 3GPP and non-3GPP access types.

7.x.6	Potential New Requirements needed to support the use case

[PR7.x.6-1] Subject to the agreements, the 6G system shall provide a mechanism to enable an authorized UE of non-3GPP satellite network connecting to 6G network to obtain services.

[PR7.x.6-2] Subject to the agreements, the 6G system shall provide a mechanism for a network operator to limit access to its services for a UE of non-3GPP satellite network.

[PR7.x.6-3] The 6G system shall be able to support mobility between the 3GPP access networks and non-3GPP satellite access network with minimum interruption of service continuity.

[PR7.x.6-4] Subject to operator policy and trusted 3rd party’s request, the 6G system shall be able to provide simultaneous data transmission via 3GPP access network and non-3GPP satellite for one or more 3GPP services.

[PR7.x.6-5] Based on operator policy, the 6G system shall be able to offload the traffic from non-3GPP satellite access network to 3GPP access network considering e.g. the traffic load and the trusted 3rd party’s request.