3GPP TSG- SA1 Meeting # 108 	S1-2444761

Orlando, USA, 18-22 November 2024	(revision of S1-244273)

# Source:	Novamint, vivo, EchoStar, Qualcomm, Spreadtrum, UNISOC, Inmarsat, Viasat, MediaTek Inc., Sateliot, SES, Fraunhofer IIS, Eutelsat Group, SKY Perfect JSAT Corporation

# Title:	New WID: Satellite access - Phase 4

# Document for:	Approval

Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Satellite access - Phase 4

Acronym: 5GSAT_Ph4

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
| x | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_5GSAT_Ph4 | SA1 | 1030042 | Study on satellite access - Phase 4 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

# 3	Justification

In Release 20, SA1 conducted a study phase (FS_5GSAT_Ph4) to enhance current use cases and explore new use cases for 5G systems with satellite access. This study phase identified potential enhancements and capabilities necessary for satellite integration within the 5G system, covering the following areas:

Enhancing of existing use cases:

- IMS Voice Call using GEO Satellite Access Enhancements: Investigated the gaps of IMS voice services using GEO (Geostationary Earth Orbit) satellites, given the increasing prevalence of satellite-enabled mobile devices. The study identified gaps, enhancemes and KPIs related to IMS voice call using GEO satellite access.

- Support for Emergency Communications and Mission-Critical Services Enhancements: Addressed the need to support emergency calls, emergency messaging and mission-critical services via satellite, especially for automotive and IoT applications. Due to broad and moving satellite coverage areas, regulatory requirements, such as location-based broadcasting, were identified as key factors for supporting emergency services over satellite.

New use cases:

- Resilient notification: Addressed user expectations for satellite-reliant communications in areas lacking terrestrial coverage. A notification mechanism was proposed to inform users when incoming calls or messages fail due to poor satellite reception.

- Multi-Orbit Satellite Access for Different Services: Examined potential benefits of using multi-orbit satellite networks (LEO, MEO, GEO) within 5G systems. This capability could enhance service experiences by leveraging different orbital characteristics, though simultaneous UE connections across orbits were deemed out of scope.

NOTE:	The inclusion of objectives related to emergency communications, mission-critical services, and multi-orbit satellite access for different services will be determined based on the outcome of the consolidation process.

The study’s findings are documented in TR 22.887 with two use cases (i.e. IMS voice call using GEO satellite, Resilient notification) objectives consolidated, providing a baseline for establishing normative requirements to standardize satellite access capabilities within the 5G system.

# 4	Objective

Using the consolidated potential requirements captured in TR 22.887 as baseline, the objective of this work item is to introduce normative service requirements related to the 5G system with satellite access for

- IMS voice call using GEO satellite access

- Resilient notification

NOTE: 	Other objectives will be added in a second phase when potential requirements and KPIs of the remaining topics of the study are consolidated.

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 22.261 | Introduce new requirements and KPIs on IMS voice call using GEO satellite access, and Resilient notification | TSG#107 | First step |

# 6	Work item Rapporteur(s)

Rapporteur: Thierry Bérisot, Novamint, tberisot@novamint.com

Co-rapporteur: Xu Xia, China Telecom, xiaxu@chinatelecom.cn

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

Supporting IM name

Novamint

vivo

EchoStar

Qualcomm

Spreadtrum

UNISOC

Inmarsat

Viasat

MediaTek Inc.

Sateliot

SES

Fraunhofer IIS

Eutelsat Group

SKY Perfect JSAT Corporation

Firstnet

Verizon

China Unicom

Google

China Telecom

China Mobile

ZTE corporation

CATT

Skylo

EDF