3GPP TSG-SA WG1 Meeting #108 	S1-244681

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244412)

Title:	New use case on Realtime Digital Twins

Agenda Item:	8.1.6

Source:	Nokia, TNO, KPN, Telefonica, Orange

Contact:	<name and email address>

Abstract: This use case proposes to address Real-time Digital Twins over 6G networks. It is proposed to be added to the “Further Use Cases on Industry and Verticals” clause in the 6G TR.

4681

-	Removed requirements on latency and coverage which are covered by the KPI table.

-	Reorder the proposed requirements to put the 6G NW role of supporting DT to the 1st requirement

-	Add a new note under the 1st requirement to clarify the realtime aspect.

4412

- Added supporting companies

- Added references / existing features

- Remove requirements on sensing (moved to existing features)

- Removed most of “subject to” wording in requirements

- Update KPI table format

- Removed “digital representation” term (not shown as change on change)

---------- Use Case template ----------

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x] 	ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[y]	3GPP TS 22.261: "Service requirements for the 5G system".

[z]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[aa]	3GPP TS 22.156: "Mobile Metaverse Services”.

[ab]	3GPP TS 22.137: " Service requirements for Integrated Sensing and Communication ".

SECOND CHANGE

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

digital twin: A real-time representation of physical assets in a digital world.

NOTE: This definition was taken from ITU-T Recommendation Y.3090 [x].

THIRD CHANGE

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DT	Digital Twin

FOURTH CHANGE (NEW TEXT)

## 10.x	Use case on Real Time Digital Twins

### 10.x.1	Description

A digital twin can represent any combination of processes, products, persons, and functionalities of real-world items such as in industry, smart cities, or construction sectors. This contextual digital equivalent of the real world offers a unified access to users and/or agents and is used for interaction, control, prediction, test, maintenance, and management of processes and components. To do so, it needs network connectivity to ingest data from multiple sources, e.g., databases, sensors, tags, network data, data models, and optionally steer / control the respective systems via feedback loops. A digital twin is aggregated, generated, and visualized by running dedicated software, including specialized AI/ML algorithms.

The real-time aspect, enabled by the low latency capability of the 6G network, allows to extend the digital twin also towards direct control of the actual physical processes.

This use case covers the following applications (non-exhaustive list)

DT in a manufacturing plant

DT for water management and improved traffic management (smart cities)

DT aggregation of sub-DT to cover complex systems (e.g. full smart cities)

DT for 6G network planning and operation itself (Network Digital Twin)

DT in port operations.

Potential sustainability impacts of the use case

|  | (the UN SDGs/GDC matching goals of each aspect within 3GPP context) | Potential benefits of the use case (added value) | Potential areas of attention of the use case (risks to be mitigated) |
| --- | --- | --- | --- |
| Environmental sustainability aspects (UN SDGs 12, 13, 14, 15 and indirectly 6, 7 & 11. UN GDC “Develop principles for environmental sustainability of digital technologies”) | Energy resources (UN SDG 7, 11, 12) |  | Energy consumption required for processing real-time data, DT generation, data centres, IoT devices, and computing resources Increased dependency and risks to energy shortage and/or disruptions _ unless local energy production is used |
|  | Material resources (UN SDG 11, 12) | the optimization of industrial and manufacturing processes can lead to better resource efficiency, resulting in lower raw materials extraction and water usage Reduced waste from the increase of the production quality and efficiency In some industries, DT can help product design/engineering towards improved modular and sustainable design aiming at circularity patterns and improved lifetime | - require manufacturing, and increased material extraction and consumption of resources including water and land to produce sensors and IoT devices - increased material efficiency is likely to reduce costs and consequently create a rebound effect, which will further push for resource depletion, unless sustainable design practices and lifecycle monitoring of each element are put in place, including the usage of eco-friendly materials, etc - Electronic waste resulting from the end of life of sensors and equipment, unless proper circularity patterns are in place |
|  | Emissions (UN SDG 6, 7, 11, 12, 13, 14, 15) | Reduced usage of natural resource (e.g., water management, smart cities) by improved monitoring Reduced GHG emissions from avoided visits to monitored areas, and avoiding operations visits for network planning and operation | - Increased emission to produce additional sensors and IoT devices required to construct DTs, and to operate the DT, unless renewable energy is used |
| Socio-economic sustainability aspects (UN SDGs 2, 3, 4, 5, 8, 9, 10, 11, 16 & 17 and indirectly 12. UN GDC “Closing Digital Divides and Accelerating SDG Progress” & “Expanding Digital Economy Inclusion” & “Fostering an Inclusive, Safe Digital Space”) | Education (UN SDG 4) | Upskilling on the use of digital technologies Upskilling on the use of the physical system (training, testing) |  |
|  | Health (UN SDG 3) | Reduced dependency on humans (e.g., avoid 24/7 monitoring on site, shifts), leading to increased well-being for humans Reduced exposure to physical risks & EMFs due to proximity with hazardous machines |  |
|  | Inclusion & Equality (UN SDGs 11, 10, 4, 5 and indirectly 3, 16 & 17) | Enhanced inclusion/opportunities for particular roles, irrespective of age, gender, disabilities, geographies etc | Creates a new gap related to IT literacy, eg with ageing populations Potential exclusion of worker populations who do not have coverage to interact remotely with the DT |
|  | Trustworthiness (UN SDGs 11 and indirectly 3 & 17) | Increased trustworthiness on data /information availability due to real time monitoring /control Improved resilience and flexibility Improved safety from using digital twin for virtual safety checks (help to predict and reduce the risks in the event of any issue (e.g., plant failure), which can affect the lives of the workers) | Potential risks to the privacy in the event of a cyber-attack attempting to read DT data Potential risks to the safety in the event of a cyber-attack attempting to control the physical world via DT Safety/predictability of operations – increased business risk from using technology instead of human perception |
|  | Food (indirectly UN SDG 2) | Enhanced accessibility to drinking water and food and its management from DT related to water/food supply, agriculture, etc |  |
|  | Work & income (UN SDG 8 and indirectly 12) | Facilitates job filing avoiding specific requirements in physical access to site, including facilitating expert replacement Remote access to the twin can enable employment in other countries, including in other timezones (reducing digital divide) | Potential impact on employability and labour market (replacement of given jobs by digital processes) |
|  | Infrastructure (UN SDG 9) | Enable sustainable urban development (Smart Cities DT): improving smart city infrastructures (eg water, waste, energy, transport, housing etc) |  |
|  | TCO reduction (UN SDGs 8, 9 and 12) | Economic efficiency improvements from optimizing operations via digital twins, eg allowing many stakeholders to meet in a virtual place, testing before production with end users/customers etc | Possible increased costs from complexity of multi-stakeholder ecosystem Increased CAPEX from digital twin model creation, connectivity and physical system equipment investment Increased OPEX from energy, digital twin model training & maintenance, and connectivity |

### 10.x.2	Pre-conditions

As example service scenario for the Real Time (RT) Digital Twin use case, a production plant is used (e.g., Chemical but could be any manufacturing or production plant).

Typically, these plants are spread over a large area (e.g. one or more km2), mostly outdoor area’s but also including indoor facilities. The constructions are complex and build in all three dimensions with potential metal obstructions preventing a line-of-sight coverage. Still, full network coverage is mandatory over the full geographical area, which the Digital Twin is associated to. Besides the Digital Twin traffic itself, also the day-to-day traffic needs to be handled by the same network.

The network provides the connectivity, sensing and computing power while enabling reliable connections in an energy efficient manner and guaranteeing the required low latency (down to the ms level) to control the chemical processes in real-time.

The machines, robots and vehicles are modelled for accurate planning for 24/7 usage and maintenance needs.

### 10.x.3	Service Flows

Connected Sensors extend the 6G network wireless sensing capabilities where necessary, to offer the extra required data from every step of the production process. As the digital twin also needs to include all moving items on premise, also the tracking of movements and locations (via network sensing and optional external sensors) is enabled. This dense grid of sensors is connected to the Digital Twin, potentially incorporating non-3GPP connection technologies, including via gateways with legacy and/or proprietary technologies to allow re-use of already deployed sensors/systems.

The RT Digital Twin is used for testing purposes, to simulate a specific configuration or setting before really applying it, in this way facilitating any decision or to consolidate and to prevent dangerous situations and/or environmental challenges and come to a continuously adapting control of the installation.

To meet these challenges, the DT (a virtual model of the full petrochemical plant, a complex physical entity, including the necessary AI /ML capabilities) is concurrently accessed via multiple users and devices.

In some cases, the DT is rendered via immersive visualization capabilities for a seamless user interaction.

Figure 10.x.3-1: Realtime Digital Twins Example Scenario

### 10.x.4	Post-conditions

The RT DT is being used both by the Research, IT and the OT team of the industrial plant for multiple purposes to save costs and smoothen operations, as well as plan the evolution of the plant.

### 10.x.5	Existing features partly or fully covering the use case functionality

In 5G, TS 22.261 [y] has documented use cases around factory automation in annex D, and specific requirements for low latency and high reliability in cl 6.28 and 7.2. TS 22.104 [z] also specifies requirements and KPIs for cyber-physical control applications in vertical domains.

Access to digital twins via mobile metaverse services is also documented in TS 22.156 [aa].

Requirements related to wireless sensing capability have been documented in TS 22.137 [ab]. In particular, requirements exist to provide a wireless sensing service in a target geographical area requested by a trusted 3rd party. Such area could be defined to match with the geographic area covering the physical system being represented by the Digital Twin.

### 10.x.6	Potential New Requirements needed to support the use case

[PR 10.x.6-1] Subject to operator’s policy, regulatory requirements and user consent, the 6G network shall provide means to assist authorized 3rd parties in controlling in real-time the physical system being represented by the Digital Twin, including by concurrent users.

NOTE 1:	The real-time aspect allows to extend a Digital Twin service towards direct control of an actual physical system and its related processes.

[PR 10.x.6-2] The 6G system shall provide a means to associate and coordinate data flows in both directions, both via 3GPP and non-3GPP access technologies, related to one or more UEs e.g., located in the geographical area covering the physical system and associated with the same object in the Digital Twin.

[PR 10.x.6-3] Subject to operator policy and user consent, the 6G system shall be able to expose network performance information (e.g., observed or predicted bitrate, latency or packet loss) related to one or more UEs to an authorized 3rd party.

NOTE 2:	The network performance information can be per UE and can take into account all available access technologies, including 3GPP and non-3GPP.

[PR 10.x.6-4] Subject to user consent, the 6G network shall be able to assist authorized 3rd parties in accessing a Digital Twin via an immersive user experience from 6G UEs, including synchronizing the experience across multiple participants up to an acceptable consistency bound.

[PR 10.x.6-5] Subject to user consent, the 6G system shall provide a mechanism to guarantee the privacy and security (e.g. integrity, confidentiality and availability) of the Digital Twin data collected from or sent to UEs.

[PR 10.x.6-6] The 6G system shall be able to enable a Real Time Digital Twin service with following KPIs.

Table 10.x.6-1:	Performance requirements for Realtime Digital Twins

| Use Cases | Characteristic parameter (KPI) |  |  |  |  | Influence quantity |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency (ms) | Service bit rate: user-experienced data rate (Mb/s) | Reliability (%) | Service Area | Location accuracy (cm) | Connection density [devices/m2] |  | UE Speed |  |  |
| Real-Time Digital Twin (NOTE 1) | [1-10] (NOTE 2) | [<100] (UL) (NOTE 3) | [99.999-99.99999] (NOTE 4) | [up to 1km2] (NOTE 5) | [≤ 10] | [1-10] |  | Up to vehicular speed |  | Communication between physical system and the network (NOTE 6) |
| NOTE 1: Wireless-sensing capability is expected to contribute to enrich the Digital Twin model, with KPIs related to e.g. location accuracy, resolution, range and latency NOTE 2: Very low latency for the Realtime aspect, both in UL (to report up-to-date data) and DL (to send timely controls) NOTE 3: It is expected uplink video will be most demanding. NOTE 4: Very high: The control of Realtime industrial processes requires very high service reliability. Can be lower for non-realtime DT or in other contexts (eg training, engineering, planning). NOTE 5: Service coverage both outdoor & indoor (e.g. industrial plant, city area) NOTE 6: performance requirements related to the communication between the DT and the end-user device accessing the DT (including network to UE communication) are not included in this table and may depend on the actual application (e.g. web, immersive etc) used to access the DT |  |  |  |  |  |  |  |  |

Editor’s Note: the KPIs related to sensing capabilities are FFS.