3GPP TSG-SA WG1 Meeting #108 	S1-244607

Orlando, Florida, USA, 18-22 November 2024	revision of S1-244214

Title:	New use case on low-energy positioning in satellite networks

Agenda Item:	8.1.3	Ubiquitous Connectivity (FS_6G-REQ)

Doc type:	PCR

Source:	Thales, ESA, Novamint, SyncTechno Inc., ETRI, Softil, NICT, Sateliot, Airbus

Contact:	Nicolas Chuberre, nicolas.chuberre@thalesaleniaspace.com
	Florin Grec, florin-catalin.grec@esa.int

Abstract: This PCR proposes a new use case on “Low-energy positioning in satellite networks” in TR 22.870 “Study on 6G Use Cases and Service Requirements”. Low-energy positioning in satellite networks is foreseen as a key 6G capability introduced to fulfil 6G IoT or low-power positioning use cases, as well as to reduce the power consumption of the 3GPP positioning service with no dependence on third-party positioning systems.

* * * First Change * * * *

## 7.1	Use case on “low-energy positioning in satellite networks”

### 7.1.1	Description

This use case corresponds to a capability for 6G with satellite access to provide low-energy positioning for low-power and low-complexity devices (e.g. IoT and low-power devices) when served by a satellite network.

### 7.1.2	Pre-conditions

A battery-powered IoT or low-power device has been deployed in an environment where it obtains its initial position and synchronization from non-3GPP methods (e.g., GNSS) and / or terrestrial networks. A combination of non-3GPP technologies (e.g., GNSS) and 3GPP technologies, terrestrial and non-terrestrial, can be configured if compatible with energy consumption requirements.

### 7.1.3	Service Flows

At some point, the IoT or low-power device may lose its original positioning sources due to one of the following reasons:

- GNSS-originated position: GNSS signal may be lost due to interference, or obstruction, or intentionally disabled to augment its autonomy.

- Terrestrial network-originated position: the IoT or low-power device may have moved beyond terrestrial network coverage, such as in the case of asset tracking sensor installed on a container to be transport by sea, rail, or road.

In this context, the IoT or low-power device, served by the satellite access, is expected to know an approximate position from prior positioning. This requires the IoT or low-power device to refresh a more accurate position.

Therefore, the IoT or low-power UE initiates the 3GPP positioning method over satellite to estimate its current location.

The 6G system broadcasts to the UE via its satellite access at least the following necessary information, including but not limited to:

- Satellite ephemeris and additional assistance data (e.g., atmospheric models, etc.)

- Reference signal for time of arrival measurements.

### 7.1.4	Post-conditions

The IoT or low-power device successfully estimates/updates its position based on information provided by the 6G system via the satellite access.

### 7.1.5	Existing features partly or fully covering the use case functionality

Ranging signals and positioning methods for IoT and low-power applications are defined in TS 36.211 / TS 38.211 and TS 36.305 / TS 38.305, respectively, but only for terrestrial networks. They need to be adapted to satellite access context.

### 7.1.6	Potential New Requirements needed to support the use case

[PR X.Y.6-001]: The 6G system with satellite access shall be able to provide network assistance data for the UE to determine its position whatever its connection state (connected, idle, or inactive) or its mobility conditions (at speed up to [50] km/h), without the need for any additional non 3GPP positioning capability.

[PR X.Y.6-002]: The network assistance data provided by the 6G system via its satellite access shall enable the UE to determine its position with a 2D accuracy better than [100] or [50] meters (selectable target accuracy) and [90%] availability in the area covered by the satellite access

[PR X.Y.6-004]: The IoT or low-power UE shall be able to perform ranging measurements to satellites from multiple satellite networks to determine its position.

* * * END of Changes * * * *