3GPP TSG-SA WG1 Meeting #108 	S1-244607

Orlando, Florida, USA, 18-22 November 2024	revision of S1-244214

Title:	New use case on low-energy positioning in satellite networks

Agenda Item:	8.1.3	Ubiquitous Connectivity (FS_6G-REQ)

Doc type:	PCR

Source:	Thales, ESA, Novamint, SyncTechno Inc., ETRI, Softil, NICT, Sateliot, Airbus

Contact:	Nicolas Chuberre, nicolas.chuberre@thalesaleniaspace.com
	Florin Grec, florin-catalin.grec@esa.int

Abstract: This PCR proposes a new use case on “Low-energy positioning in satellite networks” in TR 22.870 “Study on 6G Use Cases and Service Requirements”. Low-energy positioning in satellite networks is foreseen as a key 6G capability introduced to fulfil 6G IoT or low-power positioning use cases, as well as to reduce the power consumption of the 3GPP positioning service with no dependence on third-party positioning systems.

* * * First Change * * * *

- References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 36.763: “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”.

* * * Second Change * * * *

## 7.1	Use case on “low-energy positioning in satellite networks”

### 7.1.1	Description

This use case corresponds to a capability for 6G with satellite access to provide low-energy positioning for low-power and low-complexity devices (e.g. IoT and low-power devices) when served by a satellite network.

### 7.1.2	Pre-conditions

A battery-powered IoT or low-power device has been deployed in an environment where it obtains its initial position and synchronization from non-3GPP methods (e.g., GNSS) and / or terrestrial networks. A combination of non-3GPP technologies (e.g., GNSS) and 3GPP technologies, terrestrial and non-terrestrial, can be configured if compatible with energy consumption requirements. Integrating GNSS into IoT devices operating over satellite networks significantly impacts battery consumption, especially in battery-powered UEs requiring frequent positioning updates. Mitigating the power consumption due to GNSS would therefore be beneficial. The extent of this impact varies based on GNSS start modes (hot, warm, or cold), power consumption of GNSS modules, reporting intervals, and device mobility. According to the analysis carried out in [2], GNSS position fixes can reduce battery life from approximately 5% up to 45% in cases of frequent position fixes.

### 7.1.3	Service Flows

At some point, the IoT or low-power device may lose its original positioning sources due to one of the following reasons:

- GNSS-originated position: GNSS signal may be lost due to interference, or obstruction, or intentionally disabled to augment its autonomy.

- Terrestrial network-originated position: the IoT or low-power device may have moved beyond terrestrial network coverage, such as in the case of asset tracking sensor installed on a container to be transport by sea, rail, or road.

In this context, the IoT or low-power device, served by the satellite access, is expected to know an approximate position from prior positioning. This requires the IoT or low-power device to refresh a more accurate position.

Therefore, the IoT or low-power UE initiates the 3GPP positioning method over satellite to estimate its current location.

The 6G system broadcasts to the UE via its satellite access at least the following necessary information, including but not limited to:

- Network assistance data: Satellite ephemeris and additional assistance data to improve accuracy (e.g., ionospheric models to correct the atmospheric delay errors, etc.)

- Reference signal for time of arrival measurements.

### 7.1.4	Post-conditions

The IoT or low-power device successfully estimates/updates its position based on information provided by the 6G system via the satellite access.

### 7.1.5	Existing features partly or fully covering the use case functionality

Ranging signals and positioning methods for IoT and low-power applications are defined in TS 36.211 / TS 38.211 and TS 36.305 / TS 38.305, respectively, but only for terrestrial networks. They need to be adapted to satellite access context.

### 7.1.6	Potential New Requirements needed to support the use case

[PR X.Y.6-001]: The 6G system with satellite access shall be able to provide network assistance data for the IoT or low-power UE to determine its position using 3GPP positioning technologies whatever its connection state (connected, idle, or inactive) or its mobility conditions (at speed up to [50] km/h).

Editor’s Note: The minimum necessary network assistance data includes satellite ephemeris and corrections to atmospheric delays such as ionospheric errors.

[PR X.Y.6-002]: The network assistance data provided by the 6G system via its satellite access shall enable the IoT or low-power UE to determine its position with horizontal accuracy better than [100] or [50] meters (selectable target accuracy) and [90%] availability in the area covered by the satellite access, without increasing the UE's energy consumption by more than [5]% upon activation of the positioning method.

Editor’s Note: According to [2], which indicates that in the best-case scenario the impact of GNSS on UE power consumption in IoT-NTN is around 5%, we have set this percentage as a preliminary target.

[PR X.Y.6-004]: The IoT or low-power UE shall be able to perform positioning measurements to satellites from one or multiple satellite networks to determine its position.

* * * END of Changes * * * *