SA WG2 Meeting #166                                                S2-2411357

Orlando, USA, 18 – 22 November 2024	(was S2-24XXXXX)

Source:	vivo

Title:	KI#1#3, Conclusion update on Topology2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: it proposes the conclusion update for Topology2.

1 Proposal

It is proposed to add the following conclusions update for Topology 2 to TR 23.700-13.

* * * * Start of Changes (all text new)* * * *

8.1.3	Architecture to Support Topology 2

8.1.3.1	General

Two options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

Editor's note:	Whether a NAS-based option is needed in addition is FFS.

The following conclusion aspects and principles are applied to UE reader specific:

-	Registration and authorization aspects:

-	The UE supporting to be reader performs an initial registration to AMF, indicating its Capability of acting as a reader.

-	The AMF is responsible for the UE reader authorization during the UE registration as the following:

-	The AMF authorizes UE whether to be reader based on the UE capability and UE subscription data for UE reader in UDM during registration.

-	When the UE is authorized as a reader, the AMF indicates to the UE that the UE is authorized as a reader and provides the UE reader configurations, may including: reader ID, permit area to be a reader (Optional), permit time to be a reader (Optional).

-	The AMF assigns the TA list to the UE based on the service area of AIoTF and AMF.

-	The UE subscription data for UE reader in UDM includes allow the UE to act as a reader, permit area to be a reader (Optional), permit time to be a reader (Optional), client list for which the UE is allowed to be a reader.

-	UE reader selection

-	The UE reader selection supports both given Target area and given Target UE reader.

8.1.3.2	User-plane option

The following principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

Figure 8.1.3.2-1: User-plane architecture for Topology 2

Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT WG1 in coordination with RAN WG3.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA WG3.

NOTE 3:		The Further details are left to normative phase.

NOTE 4:		The terminology, e.g. AIoT RAN is is left to normative phase.

-	UP connection setup:

-	The UE is configured by the network with: FQDN of AIoTF, DNN, S-NSSAI.

-	The PDU session is established using the dedicated DNN and/or S-NSSAI based on the URSP.

-	UE reader selection aspects:

-	The General principles as defined in clause 8.1.3.1.

-	The AIoTF is responsible for the UE reader selection.

-	For a given UE reader, The AIoTF takes the Target UE reader ID or Target area provided by AF to perform UE reader selection for the AIoT operation.

-	UE association with AIoTF

-	The UE requests the AIoTF to setup the UE association with the AIoTF via the UP connection.

-  After the UE association setup, if needed, the AIoT service operation request is delivered to the UE via UP connection.

-	RAN is responsible for resource allocation for UE reader.

NOTE 1:	The information related to resource allocation is up to RAN decision.

-	Charging information collection

- The AIoTF is responsible for the UE reader charging information collection (e.g. duration as the UE reader).

8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC and a protocol between gNB/AIoT RAN and AIoTF. The related protocol stack options are shown in Figure 8.1.3.3-1 and Figure 8.1.3.3-2.

Editor's note:	Which option to use, i.e. whether gNB/AIoT RAN connect to AIoTF directly (Option A in Figure 8.1.3.3-1) or via AMF (Option B in Figure 8.1.3.3-2) is FFS.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details are FFS.

NOTE 1:	For RRC Option, the NAS layer of the UE provides the Key for the Uu AS layer to protect the Uu AS interaction including the AIoT operation. So the AMF and UE connection with the AMF are necessary.

In RRC Option A:

Figure 8.1.3.3-1: Protocol Stack for the RRC option (Option A)

-	Interoperation between RAN and AIoTF

-	The non-UE associated new N2AP is setup between the RAN and the AIoTF.

-	The AIoTF interacts with the RAN for AIoT operation via the new N2AP.

NOTE 1:	The above new N2AP information needs to be confirmed with RAN WG in normative phase.

-	UE reader selection

-	The general principles are as defined in clause 8.1.3.1.

-  Case1: the AF indicates the specific UE reader list and the AIoTF provide the specific reader list to the RAN and the RAN allocates resource the reader.

-  Case2: the AF provides given Target area, the AIoTF is responsible for the 1st round UE reader selection and provides the candidate reader list and the RAN can further select reader from the candidate reader list.

-	UE association and Serving RAN association:

-	For specific UE reader list indicated by the AF or the candidate UE reader list selected by AIoTF, the AIoTF needs to provide a list of UE ID to the RAN via the non-UE associated signalling between the RAN and the AIoTF. However, the RAN cannot understand the UE ID from the AIoTF since the RAN uses the RAN UE NGAP ID of the NGAP setup with the AMF to identify the UE context. And the AIoTF doesn’t know the RAN UE NGAP ID since there is no UE associated connection between the RAN and the AIoTF.

-  Furthermore, the AIoTF needs to be aware of the RAN serving the UE reader if the specific UE reader list are required to be provided to the RAN.

Editor's note:	How to solve the above UE association and Serving RAN association is FFS.

-	RAN is responsible for resource allocation for UE reader.

NOTE 2:	The information related to resource allocation is up to RAN decision.

-	Charging information collection

-	the UE or the RAN is responsible to collect the charging information (e.g. duration as the UE reader) and report them to AIoTF.

In RRC Option B:

Figure 8.1.3.3-2: Protocol Stack for the RRC option (Option B)

-	Interoperation between AMF and AIoTF

-	The AIoTF interacts with RAN via AMF and the AMF provides AIoT Service Operation for the AIoTF.

-	Interoperation between RAN and AMF

-	The NGAP between RAN and AMF is enhanced to support AIoT.

-	AIoTF interacts with RAN via AMF and AMF provide AIoT Service Operation.

NOTE 3:	The above NGAP enhancement needs to be confirmed with RAN WG in normative phase.

-	UE reader selection

-	The general principles are as defined in clause 8.1.3.1.

-  Case1: the AF indicates the specific UE reader list and the AIoTF provide the specific reader list to the RAN and the RAN allocates resource the reader.

-  Case2: the AF provides given Target area, the AIoTF is responsible for the 1st round UE reader selection and provides the candidate reader list and the RAN can further select reader from the candidate reader list.

-	RAN is responsible for resource allocation for UE reader.

NOTE 4:	The information related to resource allocation is up to RAN decision.

-	Charging information collection

-	the UE or the RAN is responsible to collect the charging information (e.g. duration as the UE reader) and report them to AIoTF.

* * * * End of Changes * * * *