SA WG2 Meeting #166	S2-2411398

# Orlando, USA, 18 – 22 November, 2024                        (revision of S2-24xxxxx)

Source: 	NEC

Title: 	AIoT general Conclusion

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes the AIoT general Conclusion for Rel-19.

1	Discussion

In TR 23.700-13 v1.1.0 and the current RAN WGs discussion, there are multiple options for the following items:

- AIoT service:

  - Inventory: Request to perform an inventory operation.

  - Read: Request to read information from an AIoT Device.

  - Write: Request to write information to an AIoT Device.

  - Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

  - Enable: request to enable a temporarily disabled AIoT Device.

- Device types in RAN1:

- The overall objective of the SI is to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

i. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

ii. ≤ a few hundred µW peak power consumption has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

- Topology:

  - Topology 1: BS reader connecting to AIoTF with Direct Path or Indirect Path via AMF.

  - Topology 2: UE reader connecting to AIoTF with User-plane option or RRC options with Option A/B.

It is important for AIoT to address the key use case and deployment as the first step in the start stage, where Inventory via the dedicated Ambient IoT core network can be the high priority, since it can be widely used in the warehouse, store and factory with simple deployment and minimum requirements to the AIoT device and network. The AIoT device has very low capability including security, it is better to deploy the dedicated Ambient IoT core network at current stage to avoid impacts to the non-AIoT services.

The following principles are suggested in the AIoT general conclusion:

- AIoT Inventory service via the dedicated Ambient IoT core network has the high priority as the key use case and deployment.

- All AIoT Device types defined in RAN1 shall be supported.

- Topology 1 and 2 shall be supported.

2	Proposal

It is proposed to update TR 23.700-13 as following.

# * * * * Start of Change* * * *

8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

8.x	General Conclusions

The following principles are suggested as the AIoT general conclusions:

- AIoT Inventory service via the dedicated Ambient IoT core network has the high priority as the key use case and deployment.

- All AIoT Device types defined in RAN1 shall be supported.

- Topology 1 and 2 shall be supported.

# * * * * End of Changes * * * *