3GPP TSG-SA2 Meeting #166	S2-2411430

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0487 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on multihop ProSe U2N Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For multihop 5G ProSe U2N Relay with model A discovery, link modification procedure is used to include the new Remote UE at the connection to the U2N Relay and the intermediate U2N relay builds its local IP routing table based on the Remote UE info received in link modification request from its child Intermediate Relay. But the IP address of the Remote UE is assigned by the U2N Relay and it cannot be informed at Remote UE info from Intermediate Relay unless U2N relay or Remote UE has informed the Intermediate U2N relay of the Remote UE’s IP address. Therefore, it is needed additional procedure to inform IP address of Remote UE to each Intermediate U2N Relay. - Subclause 6.4.3.8.2 includes IP address/prefix management, but it is not clear whether it also applies to multihop 5G ProSe U2N Relay with model B discovery. - When IM Relay forwards IP address assignment message from U2N relay to Remote UE, it is not clear how it may determine to forward the message to its child IM Relay and how to determint the destination L2 ID (unicast or broadcast) for both model A based mechanism and model B based mechanism and it needs further study. - For multihop 5G ProSe U2N Relay with model B, IP address of the Remote UE needs to be informed to Intermediate Relay so that Intermediate Relay may update the IP routing table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications on IP address handling and path selection with discovery with model B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | U2N multihop relay feature will not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.8.1, 6.4.3.8.2, 6.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of 1st Changes * * *

##### 6.4.3.8.1	Layer-2 Link establishment and management (based on Model A discovery)

Each of the 5G ProSe Intermediate UE-to-Network Relay establishes a Layer-2 Link with its immediate parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, before it can serve the 5G ProSe Remote UE, or a child Intermediate UE-to-Network Relay. RSC based configuration for the 5G ProSe Intermediate UE-to-Network Relay indicates whether it should establish the Layer-2 Link with a parent relay before it can participate in the discover operation.

Layer-2 link management procedures as defined in clause 6.4.3.6 for 5G ProSe UE-to-Network Relay operation are used. When establishing the Layer-2 link with its parent relay (either a 5G ProSe UE-to-Network Relay or another 5G ProSe Intermediate UE-to-Network Relay), the 5G ProSe Intermediate UE-to-Network Relay takes the role of the 5G ProSe Remote UE and its parent relay takes the role of 5G ProSe UE-to-Network Relay. The Layer-2 link is established per RSC.

The 5G ProSe Remote UE uses the procedure defined in clause 6.4.3.6 to establish a Layer-2 Link with the selected 5G ProSe Intermediate UE-to-Network Relay. The Intermediate UE-to-Network Relay takes the role of of 5G ProSe UE-to-Network Relay in the procedure.

NOTE:	In the case of Layer-3 UE-to-Network Relay operation, only the root 5G ProSe UE-to-Network Relay needs to establish the PDU session with the network.

When an Intermediate UE-to-Network Relay has a child Intermediate UE-to-Network Relay or a 5G ProSe Remote UE connected, it updates the Layer-2 link with its parent relay, using the procedure defined in clause 6.4.3.4, with the following enhancements:

-	The link modification procedure is enhanced to inform the parent relay, e.g. another 5G ProSe Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, of the User Info of the child relay(s) or the Remote UE(s). The Link Modification Request message contains a list of 5G ProSe Remote UE (Remote User ID, Remote UE Info) and the corresponding PC5 QoS Flow associated with each 5G ProSe Remote UE. The Remote User ID and Remote UE Info are as defined in clause 6.5.1.1.

-	The Remote UE informs the assigned IP address to its parent Intermediate UE-to-Network Relay.

An Intermediate UE-to-Network Relay triggers a link modification procedure towards its parent relay if a new Remote User ID is received from its child relay during the link modification procedure.

The 5G ProSe UE-to-Network Relay's Remote UE reporting procedure includes all the Remote UE information reported by its child relays.

* * *Start of 2nd  Changes * * *

##### 6.4.3.8.2	IP address/prefix management for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay (based on Model A discovery)

When IP based PDU session types are used by the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay needs to act as an IP Router.

The actual IP address allocation is performed by the 5G ProSe UE-to-Network Relay, as defined in clause 6.5.1.

To support DHCP based IP address configuration, the 5G ProSe Intermediate UE-to-Network Relay acts as a DHCPv4 or DHCPv6 proxy. To support IPv6 SLAAC, the 5G ProSe Intermediate UE-to-Network Relay also relays the Router Solicitation and Router Announcement messages between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay.

Editor’s Note: It is FFS how Intermediate UE-to-Network Relay determines the destination L2 address when forwarding IP address allocation/configuration message to its child Intermediate UE-to-Network Relay or Remote UE.

The 5G ProSe Intermediate UE-to-Network Relay builds its local IP routing table based on the Remote UE info from the link modification procedures as described in clause 6.4.3.8.1. The local IP routing table is per RSC and used based on RSC of the link the packet is received.

* * *Start of 3rd  Changes * * *

#### 6.5.1.4	5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF (based on Model B Discovery)

Figure 6.5.1.4-1: Connection establishment via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support

Connection establishment via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support as described in clause 6.5.1.1 is used for multi-hop UE-to-Network Relay after Model B Discovery with the following differences and clarifications:

1.	Service authorization and provisioning are performed for the 5G ProSe Layer-3 UE-to-Network Relay, Intermediate Relay(s) and 5G ProSe Layer-3 Remote UE.

2.	The 5G ProSe Layer-3 UE-to-Network Relay may establish a PDU Session for relaying.

3.	The 5G ProSe Layer-3 Remote UE performs Model B discovery of a 5G ProSe Layer-3 UE-to-Network Relay. The Remote UE obtains the path information to the UE-to-Network Relay(s) from the discovery procedure.

4.	The 5G ProSe Layer-3 Remote UE selects a path and establishes a connection for unicast mode communication as described in clause 6.4.3.9.

5.	For IP PDU Session Type and IP traffic over PC5 reference point, the UE-to-Network Relay assigns IPv6 prefix or IPv4 address to Intermediate Relay(s) and the Remote UE. The Intermediate Relay act as DHCP proxy or relay the IP allocation message (e.g., Router Solicitation, Advertisement).

Editor’s Note: It is FFS how Intermediate UE-to-Network Relay determines the destination L2 address when forwarding IP address allocation/configuration message to its child Intermediate UE-to-Network Relay or Remote UE.

The UE-to-Network Relay informs assigned IP address of Remote UE to its child Intermediate UE-to-Network Relay(s).

6.	The 5G ProSe Layer-3 Remote UE may provide PC5 QoS Info and PC5 QoS rule(s) to the 5G ProSe Layer-3 UE-to-Network Relay using Layer-2 link modification procedure via Intermediate Relay(s). Intermediate Relays may split the PC5 QoS hop-by-hop as described in clause 5.6.2.

7.	For the Remote UE Report, the Remote User ID is an identity of the 5G ProSe Layer-3 Remote UE.

For IP PDU Session Type, the Remote UE info is IP info related to Remote UE.

-	for IPv4, the 5G ProSe Layer-3 UE-to-Network Relay shall report IP address and TCP/UDP port ranges that are assigned to 5G ProSe Layer-3 Remote UE(s) by the UE-to-Network Relay;

-	for IPv6, the 5G ProSe Layer-3 UE-to-Network Relay shall report IPv6 prefix(es) assigned to individual 5G ProSe Layer-3 Remote UE(s).

For Ethernet PDU Session Type, the Remote UE info is Remote UE MAC address which is detected by the 5G ProSe Layer-3 UE-to-Network Relay.

NOTE:	For IP type of PDU Sessions, the Intermediate Relays forward the Downlink packet to the correct PC5 links based on destination IP address of the packet. For Ethernet type of PDU Sessions, the Intermediate Relay forwards the Downlink packets according to the destination Ethernet MAC address.

* * *End of Changes * * *