SA WG2 Meeting #166	S2-2411536

18-22 November 2024, Orlando, USA

Source:	Xiaomi

Title:	Discussion on supporting MBS broadcast service for NR NTN

Document for:	Approval

Agenda Item:	19.50

Work Item / Release:	/ Rel-19

Abstract of the contribution: This contribution discusses topics related to NR NTN supporting MBS broadcast service.

Introduction

A liaison (S2-2411305/R3-245844) was received from RAN3 indicates that RAN3 has started to work on NGAP impact to support NTN MBS in Rel-19, as the following:

More specifically, how the AMF indicates to the gNB when the intended service area is smaller than the satellite footprint. RAN3 agreed that the intended service area provided to the gNB can be Mapped Cell ID(s), TAI(s), or geographical service area information. The name and encoding of the geographical service area information will align with RAN2 decision. RAN3 has started to work on the appropriate signalling solution (see attached agreed TPs – IE name and encoding are pending RAN2 discussion).

This may impact SA2.

This paper discusses the issue and its probable solutions

Discussion

- Existing Solution

An MBS service area, as described in TS 23.247, is the area over which the MBS session data is distributed. An MBS service area can be a Cell ID list, a TAI list, geographical area information or civic address information.

The MBS service area indicated by the AF can be Cell ID list and TAI list if the AF resides in trust domain and can be aware of such information. Otherwise, if MBS service area provided by the AF is geographical area information or civic address information, NEF/MBSF translates the MBS service area to Cell ID list and TAI list.

The MBS service area (i.e. Cell ID list and TAI list) is sent to the MB-SMF and further to all NG-RANs which support MBS in the MBS service area to indicate the involved Cell and TA.

- Geographical Service area required for NTN NG-RAN

As described in the liaison, in addition to the existing MBS service area (i.e. Cell ID list and TAI list), the intended service area as geographical service area information is required to be provided to the gNB for supporting MBS broadcast service through NTN NG-RAN. The following are the options on how to provide such information to the gNB:

2.1	Options for geographical service area information provided by AF

Option 1

The MBS service area provided to the MBSF by AF is used as geographical area information or civic address information. The MBSF translates the MBS service area to the Cell ID list and TAI list. If the MBSF is aware that NTN NG-RAN is deployed in the MBS service area, the geographical area information needs to be provided to the involved NTN NG-RAN along with Cell ID list and TAI list information. The awareness of NTN NG-RAN can be based on the configuration information of the Cell ID or TAI.

In this option, the geographical area information delivered to the NG-RAN by 5GC is optional. It depends on whether MBSF is aware of NTN NG-RAN deployment in the MBS service area.

Observation 1: For the case geographical service area information provided by AF, the MBSF provides the information only to the involved NTN NG-RAN based on the awareness of access technology.

Option 2

The MBS service area provided to the MBSF by AF is used as geographical area information or civic address information. The MBSF translates the MBS service area to the Cell ID list and TAI list. Then both Cell ID list, TAI list and the geographical area information are provided to the involved AMF(s) via the involved MB-SMF. The AMF(s) further delivers these information to the involved NTN NG-RAN.

Compared with Option1, the MBSF does not care what access technology is deployed within the MBS service area, i.e. MBSF always provides the geographical area information to the involved AMF(s). The AMF(s) decides to send the geographical area information area to the NTN NG-RAN or each involved NG-RAN based on the existing information of the access type of each involved NG-RAN, if needed.

This option can cover TN case without any additional enhancement.

Observation 2: For the case geographical service area information provided by AF, the MBSF always provides the information to the NG-RAN with no awareness of access technology. It is up to AMF’s decision on whether sending the geographical service area in each involved NG-RAN or only in NTN NG-RAN based on the existing information of the access type of each involved NG-RAN.

Option 3

The MBS service area provided to the MBSF by AF is used as geographical area information or civic address information. The MBSF translates the MBS service area to the Cell ID list and TAI list. Then both Cell ID list, TAI list and the geographical area information area provided to the AMF(s) via the MB-SMF. The AMF(s) further delivers these information to the involved NG-RAN. It’s up to NG-RAN’s decision on whether to send the geographical area information to the UE.

Option 3 is similar with Option 2. The only difference between them is that whether sending the geographical area within the NTN NG-RAN or within each involved NG-RAN is up to the AMF decision (Option 2) or NG-RAN decision (Option 3).

This option can cover TN case without any additional enhancement.

Observation 3: For the case geographical service area information provided by AF, the MBSF always provides the information to the NG-RAN with no awareness of access technology. It is up to the NG-RAN’s decision on whether sending the geographical service area in the NTN NG-RAN or in each involved NG-RAN.

2.2	Option for Cell(s) or TA(s) provided by AF

Option 4

The MBS service area provided to the MBSF by AF is used as Cell ID list and TAI list. It implies the actual MBS service area is the Cell(s) or TA(s) even in the NTN NG-RAN case. In this case, the geographical area information is not required from the originating MBS service request, and considering the use case to be address it is not needed to translate the Cell(s) or TA(s) into the geographical area information. Therefore the MBS service area provided by MBSF is used as Cell ID list and TAI list as required from AF.

Observation4: For the case Cell(s) or TA(s) provided by AF, there is no additional requirement in the system.

Conclusion

Observation 1: For the case geographical service area information provided by AF, the MBSF provides the information only to the involved NTN NG-RAN based on the awareness of access technology.

Observation 2: For the case geographical service area information provided by AF, the MBSF always provides the information to the NG-RAN with no awareness of access technology. It is up to AMF’s decision on whether sending the geographical service area in each involved NG-RAN or only in NTN NG-RAN based on the existing information of the access type of each involved NG-RAN.

Observation 3: For the case geographical service area information provided by AF, the MBSF always provides the information to the NG-RAN with no awareness of access technology. It is up to the NG-RAN’s decision on whether sending the geographical service area in the NTN NG-RAN or in each involved NG-RAN.

Observation4: For the case Cell(s) or TA(s) provided by AF, there is no additional requirement in the system.

Proposal

It is preferable to apply 2nd option, as it simplifies the MBSF operation/implementation and it can also be applicable to the TN case. It is captured in S2-2411537.