3GPP TSG-WG SA2 Meeting #166	 S2-2411664

Orlando, USA, November 18 - 22, 2024	(revision of S2-240xxxx)

Source:	Ericsson

Title:	KI#1 Conclusion Update on Architecture to Support Topology 1

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Discuss and update the interim conclusions for KI#1 on Architecture to support Topology 1.

1. Introduction

In SA2#165, in interim conclusion for topology 1, Direct Path and Indirect Path are included in clause 8.1.2.

In the interim conclusions, both AIoTF and AIOTF are used. In TS 23.501, the abbreviation names of the NFs are all capitalized,

[Proposal-1] It is proposed to update AIoTF to AIOTF as the abbreviation name of the AIoT NF.

Within the interim conclusion, there is a NOTE and an EN left to further discuss the relationship between AIoT RAN and gNB:

NOTE 1:	There is no assumption about whether the AIoT RAN (i.e. BS Reader) also has gNB functionality for NR-Uu or not.

Editor's note:	The relationship between the AIoT RAN and (other) gNB functionality is FFS.

In RAN3 discussion, in case of topology 1, “AIoT RAN” may serve one or several readers (“BS readers” in current SA2 terminology), in case of topology 2, “AIoT RAN” is an “AIoT enabled gNB” serves one or several “UE Readers”.

[Observation-1] In topology 1, “AIoT RAN” in RAN3 refers to “BS reader” in current SA2 terminology. The readers served in “AIoT RAN” refers to the actual BS reader.

[Proposal-2] Align with RAN3 terminology: AIoT RAN is the entity which communicate with CN, BS reader is the readers within AIoT RAN for topology 1.

In RAN3’s CR (R3-245846), there is a statement:

An AIoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter.

Based on such statement, A-IoT-enabled gNB can support topology 2 only, or both topology 1 and topology 2. When supporting both topologies, AIoT RAN supports A-IoT gNB functionalities which also includes legacy gNB functionalities.

[Proposal-3] Resolve the EN and update the NOTE to clarify it does not preclude A-IoT-enabled gNB can be used in AIoT RAN.

For the protocol stack for topology 1 in RAN3, XXAP is used between AIoT RAN and AIoT CN. However, considering the indirect path, The AIoT RAN connects to the AMF. To reuse the existing interface implementation, XXAP should be NGAP.

Over NGAP, the information to be consumed by the AIoT devices (i.e., AIoT device NAS message) needs to be included. The assistant information is regarded beneficial for readers to perform the AIoT service operation. Such assistant information to be consumed by the readers needs to be included. The information to be consumed by the AIoT RAN (i.e., the AIoT session resource setup information which triggers AIoT radio resource allocation for the readers) also needs to be included.

[Proposal-4] Align with RAN3, update “AIoT Reader Protocol” to NGAP, and further clarify the information to be transmitted over NGAP. Add an EN about the alignment with RAN3 is needed.

In the protocol stack for Direction Path and Indirect Path, there is an “AIoTF Functionality” at the top of the protocol stack of the AIOTF as below. There are no NFs to communicate with AIOTF on this layer. From function perspective, the AIOTF always supports AIoT functionalities. Such support does not need to be reflected in the protocol stack.

[Proposal-5] Remove the AIoTF Functionality from the protocol stack.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.1.0:

* * * Start of Change * * * *

### 8.1.2	Architecture to Support Topology 1

The principles and aspects in this clause are agreed to support Topology 1:

-	The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1:

-	Communicate with AIoT RAN, either directly or via an AMF, to trigger e.g. inventory request and command request).

NOTE 1:	There is no assumption about whether the AIoT RAN  also has gNB functionality for NR-Uu or not. It does not preclude A-IoT-enabled gNB can be used in AIoT RAN.

NOTE 2:	It is assumed Ambient IoT services can be deployed independently from existing deployments.

NOTE 3:	It is not expected a deployment will use both direct communication between an AIoT RAN and an AIOTF and indirect communication between an AIOT RAN and an AIOTF via an AMF.

When an AIOT RAN and the AIOTF communicate directly:

-	The AIOTF communicates with an AIoT RAN via a direct interface Nx.

-	Figure 8.1.2-1 below shows the aspects related to Topology 1 (direct path) reference architecture with other NFs removed.

Figure 8.1.2-1: Non-Roaming 5G System Architecture (Direct Path)

-	Figure 8.1.2-2 below shows the aspects related to Topology 1 (direct path) in reference point representation with other NFs removed.

Figure 8.1.2-2: Non-Roaming 5G System Architecture in reference point representation (Direct Path)

NOTE 4:	The protocol used over Nx will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

NOTE 5:	The protocol stack used between the AIOTF and the AIoT RAN will be concluded by RAN WG3.

-	Figure 8.1.2-3 below shows the aspects related to Topology 1 (direct path) protocol stack between the AIOT RAN and AIOTF. Over NGAP, the AIoT device NAS message, the assistant information, and the AIoT session resource control information are transferred.

Figure 8.1.2-3: Example Protocol Stack between AIOTF and AIoT Device for Topology 1 (Direct Path)

Editor's note:	The alignment with RAN3 is needed for NGAP and the information transferred over NGAP.

When an AIoT RAN and the AIOTF communicate indirectly via an AMF:

-	The AIOTF connects with an AIoT RAN via an AMF. The N2 interface between the AIoT RAN and AMF supports Ambient IoT services including delivery of inventory/command messages.

NOTE 6:	If network isolation is required an AMF instance is deployed for supporting the AIOTF communication with the BS Reader.

-	NGAP between the BS Reader and the AMF is enhanced to support Ambient IoT Services.

NOTE 7:	The details of the enhancements will be concluded by RAN WG3.

-	The AMF shall be enhanced to support Services which are used by an AIOTF for Ambient IoT Operations. New SBI based services on the AMF (to be used by the AIOTF) are introduced in 5GC.

-	The AMF passes information between AIoT RAN (e.g. operation requests and responses) and the AIOTF.

Editor's note:	The AMF enhancements are FFS.

Editor's note:	Details of the Service (e.g. whether it is a new service, whether the existing Namf service is enhanced is FFS).

-	Figure 8.1.2-4 below shows the aspects related to Topology 1 (indirect path via AMF) reference architecture with other NFs removed.

Figure 8.1.2-4: Non-Roaming 5G System Architecture (indirect Path via AMF)

-	Figure 8.1.2-5 below shows the aspects related to Topology 1 (indirect Path via AMF) in reference point representation with other NFs removed.

Figure 8.1.2-5: Non-Roaming 5G System Architecture in reference point representation (indirect Path via AMF)

-	Figure 8.1.2-6 below shows the aspects related to Topology 1 (indirect path via AMF) protocol stack between the AIoT RAN and AIOTF.

Figure 8.1.2-6: Example Protocol Between AIOTF and AIoT Device for Topology 1 (indirect Path via AMF)

Editor's note:	The details of the protocol stack are FFS.

Editor's note:	How addressing works for UL traffic (i.e. how the AIoT RAN identifies the appropriate AMF to which to forward UL messages) in the indirect path via AMF is FFS.

* * * End of Change * * * *