SA WG2 Meeting #S2-166	S2- 2411670

18 - 22 November, 2024, Orlando, US

# Title:	[Draft] Reply LS on UE's user plane availability status for LCS-UPP

# Response to:	(S2-2411289/C1-246023) LS on UE's user plane availability status for LCS-UPP

# Release:	Rel-19

# Work Item:	TEI19, 5G_eLCS_Ph3

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Richard Batorfi

richard.batorfi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 thanks CT1 for the LS UE's user plane availability status for LCS-UPP.

Questions:

In 3GPP TS 24.572, the cause value "user plane not available" was introduced for UE to reject the network initiated user plane connection establishment procedure when the user plane is not available or release the established user plane connection when the user plane availability is changed to “not available”, e.g. due to the activation of the PS data off for the UE.

However, currently there is no dedicated procedure specified to inform the LMF when the UE's user plane availability changes from "unavailable" to "available". What currently is supported:

-	The UE can initiate UE requested user plane connection establishment procedure, which may not be appropriate unless the UE has UL positioning signalling, i.e. either such notification is delayed or user plane positioning resources are unnecessarily established;

-	The LMF can attempt reactivation of user plane positioning using the network initiated user plane connection establishment procedure. This leads to extra unsuccessful user plane connection establishment procedure attempts over the air interface until the user plane is "available".

1.	Would the above issue need to be resolved?

2.	If the answer to the question 1 is yes, CT1 would like to ask for SA2 guidance on resolving the issue.

Answer:

SA2 confirms that it is not appropriate to use UE requested user plane connection establishment procedure to inform LMF about the UE's user plane availability.

Additionally, SA2’s believes there is no need to introduce a procedure to inform LMF when the UE's user plane availability changes from "unavailable" to "available. LMF could implement a retry policy after a certain time and based on this logic LMF could try to initiate or reinitiate user plane connection establishment towards the UE. LMF could retry after certain configured time while a new position request arrives.

2. Actions:

To CT1:

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #166-Ad Hoc-e	20-24 January 2025	Online

TSG-SA2 Meeting #167		14-18 October 2025	Greece, Athens