3GPP SA WG2 Meeting #166	S2-2411722

Orlando, FL, USA, 18-22 November 2024

# Title:	[Draft] Reply LS on UE's user plane availability status for LCS-UPP

# Response to:	LS on UE's user plane availability status for LCS-UPP (C1-246023/ S2-2411289)

# Release:	Release 18

# Work Item:	5G_eLCS_Ph3

Source:	SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Hong Cheng

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 would like to thank CT1 for the LS on UE's user plane availability status for LCS-UPP (C1-246023/S2-2411289).

For the questions from CT1, SA2 would like to offer the answers as follows:

Q1: Would the above issue need to be resolved?

SA2’s answer: After the UE informed the LMF regarding the user plane status change to “not available”, it is not reasonable to expect UE attempting the connection by itself, as the UE is unaware of the LMF future needs or decision on the use of user plane for positioning signalling, The UE initiated attempts for user plane connection establishment could cause more wasteful signalling or failures, if the user plane status changes relatively frequently. Therefore, any recovery of the user plane connection for LCS operation should be triggered by LMF.

Q2: If the answer to the question 1 is yes, CT1 would like to ask for SA2 guidance on resolving the issue.

SA2’s answer: Please refer to the answers to Q1. The establishment of the LCS user plane connection was assumed to be under the control of the LMF, including the security management. Therefore, it is preferrable for the LMF to be in control of the re-establishment of the user plane connection after the status change. SA2 did not identify any disadvantage for LMF to mange the reactivation attempts for the user plane positioning.

2. Actions:

To CT1:

ACTION: 	SA2 would like to ask CT1 to take the above into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #AH		24-28 January 2025	Electronic meeting

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece