3GPP TSG-SA2 Meeting #166	S2-2411730

# Orlando, US, November 18 - 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1279 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL training procedure when untrusted AF is acting as VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - This paper adds training procedure of Vertical Federated Learning when untrusted AF is acting as VFL server - For VFL training, existing MLModelTraining and MLModelTrainingInfro can be resused with VFL correlation ID which indicate this service operation is for VFL training. To support the VFL training betweent the AF server and NEF, new NEF service operation is needed. - The AF can indicate to the NEF that it will not use the VFL Correlation ID (tempoary IDs are no longer required). If the NEF received the indication, NEF can remove the stored NWDAF ID(s) allocated to the AF associated with the VFL correlation ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding new clause for training procedure of Vertical Federated Learning when AF is acting as VFL server. - Reuse existing ML Model training service operation for training NWDAF as VFL clients, and new NEF service operation is introduced between AF/NEF. - AF may indicate to remove VFL Correlation ID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing documentation for VFL procedures with untrusted AF as server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.2H.2.3.2	Training Procedure for Vertical Federated Learning when untrusted AF is acting as VFL Server

Figure 6.2H.2.3.2-1: Training Procedure for Vertical Federated Learning when untrusted AF is acting as VFL Server

- The untrusted AF acting as VFL server determines the VFL clients that participate in VFL procedure from the discovery and preparation phase as described in the clause 6.2H.2.1.2 and clause 6.2H.2.2.2. The untrusted AF identifies the VFL client using the temporary NWDAF ID assigned during the discovery procedure.

Steps 2 to 8 are repeated until the training termination condition is reached.

2.	Same as the step 2 defined in clause 6.2H.2.3.1 with the following differences:

The VFL server sends a Nnef_MLModelTraining_Subscribe or Nnef_MLModelTrainingInfo_Request to start the VFL training to each of the selected NWDAF client(s). The request includes VFL correlation ID, Application ID, and VFL capability type (VFL client).

3.	The NEF looks up information about VFL client stored in the discovery procedure (see Clause 6.2H.2.1.2). It forwards the request to the VFL client. If the message in step 2 contains external identifiers, the NEF maps them to internal identifiers.

4.	Same as the step 3 defined in clause 6.2H.2.3.1.

5.	Same as the step 4 defined in clause 6.2H.2.3.1.

6	For each NWDAF as VFL training client, the NEF forwards the intermediate results to the AF by invoking Nnef_MLModelTraining_Notify or Nnef_MLModelTrainingInfo_Request response.

7.	Same as the step 5 defined in clause 6.2H.2.3.1.

8.	Same as the step 6 defined in clause 6.2H.2.3.1.

9.	The AF as VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process by invoking Nnwdaf_MLModelTraining_Unsubscribe service or Nnwdaf_MLModelTrainingInfo_Request service operation with Termination Request, optionally indicating the reason.

10.	The NEF forwards the request to the VFL client. The VFL Server and each VFL clients, stores the VFL correlation ID and latest information related to their locally trained Models.

* * * * End of Changes * * * *