3GPP TSG-WG SA2 Meeting #166	S2-2411802

# Orlando, USA, 18th Oct – 22nd Nov, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5688 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE served by a MWAB: authorization aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia, LG Electronics,] Huawei, [ZTE, Samsung, Ericsson, Qualcomm Inc.] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The approved VMR_Ph2 work item (SP-240632) is set to specify the architecture enhancements, functionalities and procedures to support MWAB based on conclusions of TR 23.700-06 (clause 8). This CR introduces the descriptions on how the MWAB is authorized to operate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause added to describe how the MWAB is auuthorized to operate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MWAB feature cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev4: Additional clarifications on authorization aspects to align with agreed document S2-2411119 for TS 23.502. |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.49.3	Authorization aspects

5.49.3.1	General

There are two aspects related to the authorization of an MWAB. One aspect is the authorization of the MWAB-UE to establish a BH PDU Session. The other aspect is related to the authorization of the MWAB-gNB to operate as a gNB logically belonging to a MWAB Broadcasted PLMN/SNPN which the MWAB-gNB announces in the SIB.

5.49.3.2	MWAB-UE authorization

The MWAB-UE is authorized based on its subscription information which includes dedicated S-NSSAI(s) and DNN for MWAB operation. Such dedicated S-NSSAI(s) and DNN(s) shall be part of the subscription information for the MWAB-UE as specified in clause 5.2.3.3.1 of TS 23.502 [3].

The authorization of the MWAB-UE to establish connectivity in a BH PLMN/SNPN is based on whether dedicated S-NSSAI(s) for MWAB operations are received as part of Allowed NSSAI in BH PLMN/SNPN following existing procedures as described in clause 5.15, and whether it can establish BH PDU session(s) in the BH PLMN/SNPN using the dedicated S-NSSAI(s) and DNN(s).

The MWAB-UE authorization supports time-based or location-based control.

-	For time-based control, existing time-based network slice subscription control can be reused (including the enhancements in clause 5.15.16).  If the dedicated S-NSSAI(s) for MWAB operation is provided as part of the Allowed NSSAI, and when its validity time is not available, the serving AMF of BH PLMN may send UE Configuration Update message to remove the dedicated S-NSSAI(s) for MWAB operation from the Allowed NSSAI as specified in clause 4.2.4.1 of TS 23.502 [3] or de-register the MWAB-UE as in clause 4.2.2.3 of TS 23.502 [3].

-	For location-based control, existing mechanism, e.g. service area restriction, LADN based control (see e.g. clauses 5.6.5 and 5.6.5a) can be reused.

NOTE1:	When the MWAB-UE can no longer use S-NSSAI(s) dedicated to MWAB operation,, e.g. the dedicated S-NSSAIs and DNNs for MWAB operation are removed from the subscription (i.e. the MWAB becomes unauthorized) and a UE subscription update (including a UE configuration update) step happens, or the MWAB-UE moves to an area where the S-NSSAI is not available, if the AMF is configured to do so for the dedicated S-NSSAI and DNN for MWAB operation, the AMF delays the corresponding PDU session release based on an operator configuration (e.g. a local configured timer to allow time for the MWAB-gNB to handover the UE(s) it serves to other cells).

NOTE 2:  If the S-NSSAI(s) used by BH PLMN for MWAB operation is not dedicated for MWAB, the behaviours of AMF as in NOTE 1 cannot be limited to MWAB-UEs only. Additionally, it may also not be possible to configure specific treatment for MWAB-UEs (different from other UEs) in the NG-RAN.

The MWAB-UE may be connected to the BH PLMN/SNPN for other services, if allowed by the subscription data, even if the S-NSSAIs and DNNs for Backhaul PDU sessions are no longer authorized.

5.49.3.3	MWAB-gNB authorization

The authorization of the MWAB-gNB is managed by the OAM system of the MWAB-Broadcasted PLMN/SNPN as specified in TS 38.401 [42]. The MWAB-gNB establishes a secure and trusted connection to the OAM server using the IP connectivity provided by the MWAB-UE via the Backhaul PDU Session(s) as specified in clause 5.49.1.

The OAM of the MWAB Broadcasted PLMN/SNPN determines when/where the MWAB-gNB can operate or when/where it needs to shut down. The OAM of the MWAB Broadcasted PLMN/SNPN may pre-configure the MWAB-gNB to turn on/shut down the operations of respective MWAB Broadcasted PLMN/SNPN part. When the MWAB-gNB is no longer authorized to operate by OAM of the MWAB Broadcasted PLMN/SNPN, the MWAB-gNB should handover the UE(s) of the MWAB Broadcasted PLMN/SNPN to other cells.

NOTE: 	It is expected the HPLMN/Subscribed SNPN network ensures that the BH PDU sessions authorization for MWAB UE does not interfere with the ability of the OAM of the MWAB Broadcasted PLMN/SNPN to manage the MWAB-gNB, e.g. the S-NSSAI(s) for the BH PDU session(s) of the MWAB-UE is(are) maintained long enough in the MWAB-UE subscription and are available for the MWAB-UE for a location and time that is sufficient for the MWAB-UE, so that the BH PDU sessions are available for the MWAB-gNB to be able to perform OAM-controlled shut down, which may include handing over the UEs it serves.

When the MWAB-gNB stops operating under OAM control, the MWAB-gNB may trigger the MWAB-UE to release the BH PDU sessions.

End of CHANGES