3GPP TSG-WG SA2 Meeting #166	S2-2411811

# Orlando, USA, 18th Nov – 22th Nov, 2024                  (revision of S2-2411194)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1134 | rev | 16 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General training procedure for Vertical Federated Learning between NWDAF(s) and AF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile], [CATT], [ZTE], [OPPO], [China Telecom], [ETRI], Huawei, HiSilicon, [Nokia], [Futurewei], [Samsung] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper adding procedure of Vertical Federated Learning when NWDAF is acting as VFL server Rev 16: The “Editor’s Note: It is FFS if the term “backward local ML model training information” needs to be changed into “intermediate results”.” Is removed. Including the parameters at the start of the training that allow the alignment among the VFL participants. The “Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS.” Is removed. Including the information required to be stored at the end of the VFL training to allow the determination in the VFL inference of the VFL correlation ID for a given analytics subscription, the proper local ML model associated with the VFL correlation ID and the same entities that performed the VFL Training as VFL Server and VFL Client as indicated by the agreed text in Clause 6.2H.1 in S2-2411192. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause for procedure of Vertical Federated Learning when NWDAF is acting as VFL server |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training procedure not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3 (new), 6.2H.2.3.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CRs shall be implemented in the following order: CR 1198, CR 1246, CR1235, CR1208, CR1134 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 16: The term “backward local ML model training information” in Step 2 (information from VFL Server to VFL Client) cannot be changed to the term “intermediate results” because the term “intermediary results” is already used in the Step 4 to indicate the information that is sent from the VFL Client to the VFL Server. Therefore, the same term cannot, if changed it generates ambiguity because there is no way to make a differentiation on the direction of the information. However, the wording used in the sentence is also not clear and some clarifications were provided, maintaining the original wording “backward local ML model training information”. It is proposed to include at the start of the training (if the preparation phase is embedded in the start of the training or at the preparation phase,) the following parameters: - Analytics ID: In the preparation phase (if separated from training) or at the start of the training. Only the VFL correlation ID is not enough to indicate to the VFL Participants what is the target of the VFL training. The Analytics ID is a core information - VFL Group ID: Necessary at the preparation phase or at start of VFL training. Needed to allow VFL Participant to verify whether it is allowed to start training with the VFL Server and with the requested parameters - Sample Alignment Information: Required either during the conclusion of the preparation phase or at the start of the training phase. VFL participants must receive from the VFL Server the final list of sample IDs to be used for a specific training to start, as each VFL server makes the final decision on the list based on the input from VFL participants. - VFL Requirements: Used to standardize some high level parameters from the AI process, such as convergency rate, class of AI process (regression - i.e., model with real output - or classification – i.e., model with integer output) - Data Availability requirement and FL Availability time requirement: Keeps the compatibility with the parameters already defined in the model training services. - VFL Client Information: Information to support the determination during VFL Inference of the same NF that performed the VFL Training as a VFL Client for a given VFL correlation ID. For the removal of the ENs in Step 7 and the closing of the gap between VFL Training and VFL Inference in terms of allowing the participants in VFL Inference to use the same VFL participants during training, this contribution proposes to include the following parameters in the VFL Training Termination message in Step 7: - VFL Correlation ID: identification of the VFL training process that is concluded. - VFL termination flag: indication to the VFL Client that the VFL training is finished and the mapping information that will support the VFL Inference can be stored - VFL Server Information: information that identifies the NF executing the VFL training for the VFL correlation ID |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

6.2H.2.3 Training Procedure for Vertical Federated Learning

6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server.

Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS.

Editor's Note:  How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor’s Note:	Terminology will be aligned in the future.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE 1: VFL Server determines to perform VFL training based on the internal trigger or local configuration.

The steps 2-6 are repeated until the training termination condition is reached.

2.	To start VFL training, the VFL server allocates VFL correlation ID. The VFL server sends a request to start the VFL training to each of the selected VFL clients. The VFL server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to each of the selected NWDAF clients(s) and Naf_VFLTraining_Subscribe or Naf_VFLTrainingInfo_Request to the selected AF clients(s). The request includes VFL correlation ID, VFL capability type (VFL Client), Analytics ID, VFL Group ID, Sample Alignment Information (e.g., Sample IDs), VFL Requirements (e.g., training output type such as integer if classification class of AI technique required or real number if regression class of AI techniques required). Optionally, the VFL Server includes: Data Availability requirement, FL Availability time requirement (time span needed for the FL process). Data Availability requirement includes the dataset statistical properties, the time window of the data samples and optionally the minimum number of data samples.

NOTE 2: VFL Server determines the parameters Analytics ID, VFL Group ID, Sample Alignment Information, and VFL Requirement based on local configuration comprising the possible values for such parameters agreed among vendors and/or application providers participating in the same VFL Group for specific VFL task(s).

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing VFL correlation ID, and the backward local ML model training information to each of the VFL clients for next round of VFL training, where the backward local ML Model comprises the updated ML model information to be used by VFL Clients for the next round of training, as determined by the VFL server.

Each VFL Client verifies whether they can join the requested VFL training based on the support for the requested parameters and/or based on local configuration. Each VFL Client provides to the VFL server either a confirmation of participation in the requested VFL training, or a rejection. When the VFL Client accepts to participate in the VFL training process, it provides to the VFL server its VFL Client Information, to be used during VFL inference.

NOTE 3:	In this release, the same NF associated with a VFL Server or VFL Client capability during the VFL training for a VFL correlation ID is also the same NF during the VFL inference.

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

Editor’s Note:	It is FFS which service is used between VFL server and VFL clients for VFL training.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available. The data used by each VFL Client is collected as per alignment information.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and when applicable (e.g., after the first round of training) based on possible backward local ML model training information distributed by the VFL server in the previous training iteration. Each VFL Client computes and reports the client intermediate training result of its local ML model to the VFL server.

Editor’s note:	The report may include ID(s) of sample(s) corresponding to the intermediate training result.

Editor´s note: The following is FFS and may depend on the service design: When the clients reports the client intermediate training result, it also includes the corresponding VFL correlation ID.

5.	The VFL server may collect the local data and generates its own local intermediate training result. The NWDAF acting as VFL Server computes the backward local ML model training information (e.g. gradient information or loss information) based on the VFL Client(s) intermediate training result(s) received in step 4, its own local intermediate results and the label. The backward local ML model training information is used for updating the models of VFL clients. Different backward local ML model training information may be computed for different VFL clients and for the VFL Server itself.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

Editor’s Note: Whether weight of the VFL Client is computed by VFL server is FFS.

Editor’s Note: Whether VFL server and VFL clients share feature information is FFS.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, and/or if the pre-set iteration number is reached) whether VFL Training process converged. If the VFL Server evaluates the VFL Training process did not converge, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 2 - 6. If the VFL Server evaluates the VFL training process converged, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include model metric (e.g. ML model accuracy).

Editor’s Note: The content of the VFL status report is FFS.

Editor’s Note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

Editor’s Note: the following is FFS. The VFL server determines with which VFL Client(s) to continue the VFL. The VFL server may provide to the VFL clients any sample ID(s) if changed

7.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process. The VFL server provides to each VFL Client the following parameters: VFL Correlation ID, the VFL Server Information to allow the determination of the VFL Server for a VFL correlation ID during the VFL inference.

The VFL Server and each VFL Client, stores the mapping information of the VFL correlation ID to the following parameters: Analytics ID related to the VFL training process, latest information related to their locally trained Models, the unique ML model identifier of the locally trained model. In case of VFL server, it also stores for each VFL Client the VFL Client Information received in Step 2 (to be used for the determination of the VFL Clients in the VFL Inference. In the case of a VFL Client, it also stores in the mapping information the VFL Server Information received in Step 7.

NOTE 4: The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS

Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS.

NOTE 1:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

# >>>>END OF CHANGES<<<<