3GPP TSG-WG SA2 Meeting #166	S2-2411889

# Orlando, USA, 18th Oct – 22th Nov, 2024		(revision of S2-2410126)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1477 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures of UE-SAT-UE communications when satellite changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AGW reallocation is key to UE-SAT-UE communication - Case#1: If UE-SAT-UE continues when gNB (satellite) changes: the AGW on the satellite change side is reallocated from the source gNB satellite to the target gNB satellite. - Case#2: If UE-SAT-UE terminates when gNB (satellite) changes: the AGW is reallocated from the satellite back to the ground at both sides. This CR outlines the above mentioned cases as procedures for these two scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 2 new clauses regarding procedures: - A new clause to depict AGW reallocation procedure in satellite change with UE-SAT-UE continues - A new clause to depict AGW reallocation procedure in satellite change with UE-SAT-UE terminates |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-SAT-UE communication cannot be supported when satellite changes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AY.5.Y (new), AY.5.Y.1 (new), AY.5.Y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AX.5.Y Mobility support for optimized media routing

### AY.5.Y.1 UE-SAT-UE continues

This procedure is applied when a UE-SAT-UE is active and decided to be continued when one of the UE’s gNB onboarded serving satellite changes, for which the session is reallocated to a new AGW on the satellite at the satellite changing UE side. This can be triggered by both the originating side and the terminating side.

Figure AY.5.Y.1 Procedure of UE-satellite-UE communication continus

Pre-conditions:

0a. UE A is handed over from a source gNB onboarding satellite to a target gNB onboarding satellite. At the UE A side, the uplink and downlink data are transmitted via AGW-1 onboard the source satellite.

0b. PCF to P-CSCF-A: Npcf_Policy Authorization Notify (satellite ID, access info)

When the handover is completed, the PCF updates P-CSCF-A with the new satellite ID and access information to notify P-CSCF-A that UE A’s serving satellite has changed. If there is no user plane fallback indication, it means that on the 5GC side, the UE-SAT-UE can be continued.

Procedures:

1. 	If a user plane fallback indication is not included, it means that on the 5GC side, the UE-SAT-UE can be continued. The P-CSCF-A then verifies the connectivity status between UE-A’s new satellite and UE-B’s satellite. The P-CSCF-A decides to continue UE-SAT-UE communication and selects a new AGW onboarded satellite, i.e. AGW-2.

The P-CSCF-A determines whether the UE-SAT-UE communication can be continued based on UE A’s new satellite ID received from PCF and the stored UE-2’s satellite ID.

2. The P-CSCF-A sends a SIP RE-INVITE without SDP to UE A.

3. UE A sends 200 OK for SIP RE-INVITE with SDP offer.

4a-4b. P-CSCF-A interacts with the new IMS AGW on satellite, i.e., AGW-2, to allocate transport addresses for UE A based on the mechanism defined in TS 23.334 [74].

5. P-CSCF-A modifies the SDP offer with the IMS AGW transport addresses allocated in step 4a-4b.

P-CSCF-A generates a new SIP RE-INVITE which carries the new satellite ID received in step 0b and the modified SDP offer.

6. If a different satellite ID and a different transport addresses are received (i.e. UE-SAT-UE communication continues with satellite change at UE-A side), and the new satellite has ISL with UE-B’s satellite, P-CSCF-B determines to continue UE-SAT-UE communication.

7. P-CSCF-B updates the allocated IMS AGW, i.e., AGW-3, on the satellite for UE B with the transport addresses received in step 5.

8. P-CSCF-B sends the SIP RE-INVITE with SDP offer to UE-B.

9-10. UE-B response with SDP answer in 200 OK response.

11. P-CSCF-A sends ACK to UE-B.

12a-12b. P-CSCF-A interacts with AGW-2 to allocate transport addresses for UE A based on the mechanism defined in TS 23.334 [74].

13. P-CSCF-A instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation and indicates SMF via PCF to insert UL CL/BP and L-PSA UPF for IMS PDU session.

14. P-CSCF-A modifies the SDP offer in the ACK response with the IMS AGW transport addresses allocated in step 12a-12b, and forwards the updated ACK response to UE-A.

15. P-CSCF-A releases the IMS AGW on the source satellite.

# * * * * Second change * * * *

### AY.5.Y.2 UE-SAT-UE terminates (fall back to ground)

This procedure is applied when a UE-SAT-UE is active and decided to terminate the UE-SAT-UE communicaiton when one of the UE’s gNB onboarded serving satellite changes, for which the session is reallocated to a new AGW on the ground. This can be triggered by both the originating side and the terminating side.

Figure AY.5.Y.2 Procedure of UE-satellite-UE communication fall back to the ground

Pre-conditions:

0a. UE A is handed over from a source gNB onboarding satellite to a target gNB onboarding satellite. At the UE A side, the uplink and downlink data are transmitted via AGW-1 onboard the source satellite.

0b. PCF to P-CSCF-A: Npcf_Policy Authorization Notify (satellite ID, access info)

When the handover is completed, the PCF updates P-CSCF-A with the new satellite ID and access information to notify P-CSCF-A that UE A’s serving satellite has changed. A User Plane Fallback indication could also be included.

Procedures:

1.  If a user plane fallback indication is included, it means that on the 5GC side, the UE-SAT-UE cannot be continued. The P-CSCF-A verifies the connectivity status between UE-A’s new satellite and UE-B’s satellite. The P-CSCF-A decides the UE-SAT-UE communication can not be continued, e.g. no ISL between UE-A’s new satellite and UE-B’s satellite, the P-CSCF-A selects a new AGW on the ground, i.e. AGW-2.

The P-CSCF-A determines whether the UE-SAT-UE communication can be continued based on UE A’s new satellite ID received from PCF and the stored UE-2’s satellite ID.

2. The P-CSCF-A sends a SIP RE-INVITE without SDP to UE A.

3. UE A sends 200 OK for SIP RE-INVITE with SDP offer.

4a-4b. P-CSCF-A interacts with the new IMS AGW on the ground, i.e., AGW-2, to allocate transport addresses for UE A based on the mechanism defined in TS 23.334 [74].

5. P-CSCF-A modifies the SDP offer with the IMS AGW transport addresses allocated in step 4a-4b.

P-CSCF-A generates a new SIP RE-INVITE which carries only the modified SDP offer without any satellite ID.

6. As there is no satellite ID included in the SIP RE-INVITE (UE-SAT-UE is fallen back to the ground at UE-A side), the P-CSCF-B determines to terminate UE-SAT-UE communication.

7a-7b. P-CSCF-B selects a new AGW on the ground, i.e., AGW-4, and interacts with AGW-4 to allocate transport addresses for both IMS access and core network sides.

8. P-CSCF-B updates the SDP offer with the IMS AGW transport addresses allocated in step 7a-7b, and sends the SIP RE-INVITE with updated SDP offer to UE-B.

9. UE-B response with SDP answer in 200 OK response.

10. P-CSCF-B modifies the SDP offer with the IMS AGW transport addresses allocated in step 7a-7b, and sends the 200 OK response with updated SDP offer to IMS AS.

11. P-CSCF-B sends ACK to UE-B.

12a-12b. P-CSCF-A interacts with AGW-2 to allocate transport addresses for UE A based on the mechanism defined in TS 23.334 [74].

13. P-CSCF-A instructs the PCF to authorize the resources necessary to establish a QoS flow on the ground for media via Npcf_PolicyAuthorization_Update service operation.

14. P-CSCF-A modifies the SDP answer in the ACK response with the IMS AGW transport addresses allocated in step 12a-12b, and forwards the updated ACK response to UE-A.

15. P-CSCF-A releases the IMS AGW on the source satellite.

16. P-CSCF-B releases the IMS AGW on the satellite.

# * * * * End of change * * * *