SA WG2 Meeting #166	S2-2412014

November 18 – 22, 2024, Orlando, FL, USA

Source:	Charter Communications

Title:	5G-RG support for ECN marking for L4S

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2 / Rel-19

Abstract of the contribution: This contribution discusses the need for 5G-RG to indicate support for ECN marking in rel-19.

# Introduction

R19 TR23700-70 Study on XRM Phase 2 KI#6 concluded that the following normative work was agreed for wireline access:

1.	Dedicated 5G QoS Flow(s) and non-3GPP access resources (e.g. IPsec Child SAs) are used for carrying L4S enabled IP traffic.

2.	For wireline access:

a.	ECN marking for L4S is supported in W-AGF. It is controlled via N2 signalling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS profile(s) and L4S-enabled W-UP resource(s).

b.	ECN marking for L4S is supported in 5G-RG in UL. It is controlled via N1 signalling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS rule(s) and L4S-enabled W-UP resource(s).

CR2129 (S2-2410952) was agreed at SA2#165, which introduces a new clause specifying ECN marking for L4S in wireline access.  TS23.316 clause 4.x.2 (new), describes that a 5G-RG may support ECN marking for L4S in the UL and the SMF may send the Indication of ECN marking for L4S associated with QoS rule(s) to the 5G-RG via N1 signalling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies mapping between L4S-enabled QoS rule(s) and L4S-enabled W-UP resource(s).

# Discussion

In R18 and R19, the SMF may decide to indicate ECN marking for L4S by (R)AN (i.e., 5G-AN) via N1N2 signaling.  This is illustrated in TS23.502 Figure 4.3.2.2.1-1 UE-requested PDU Session Establishment steps 11 and step 14, but if the 5G-AN does not support ECN marking for L4S by NG-RAN it will indicate in N2 PDU Session Response message (step 14) established QoS Flows status.  In the case that the 5G-AN does not support ECN marking for L4S by NG-RAN but the established QoS Flows status is active, the SMF would need to either perform a N4 Session Modification Request where it would indicate ECN marking for L4S by PSA UPF, or perform a PDU Session Modification procedures to delete the QoS flow (TS23.502 Figure 4.3.3.2-1 step 1d) and if needed indicate to the AF that ECN marking for L4S is not supported as described in TS23.501 clause 5.37.3.

Observation: If a capability indication (e.g., support of ECN marking for L4S) was provided by the nodes capable of ECN marking for L4S (i.e., NG-RAN, N3IWF/TNGF, W-AGF, and 5G-RG) the network would be able to reduce signaling request from the SMF.

As described in CR2129R5, ECN marking for L4S is supported in 5G-RG in the UL.  The 5G-RG receives via N1 signaling an Indication of ECN marking for L4S for a corresponding QoS Flow and applies mapping between L4S-enabled QoS rule(s) and L4S-enabled W-UP resources(s).

One option which was not agreed in previous SA2 meetings (i.e., SA2#164 S2-2408823), included an option for the 5G-RG to indicate its support for ECN marking for L4S in the 5GSM Core Network Capability field.  As described above, there is a benefit of having an indication if ECN marking for L4S is supported.  Therefore, an indication from the 5G-RG allows the network to be aware if L4S-enabled QoS rule(s) and L4S-enabled W-UP resource(s) can be supported by the 5G-RG prior to requesting ECN marking for L4S which reduces signaling in the network.

An indication for support of ECN marking for L4S by the 5G-RG can be provided either:

- in the 5GSM Core Network Capability field or

- as a new indication as part of N1 SM container  or

- as a 5G-RG new subscription IE stored in the UDM (e.g., “5G-RG L4S information”).

Proposal: 5G-RG may indicate in the UE 5GSM Core Network Capability whether the 5G-RG supports ECN marking for L4S in UL.

# Proposal

The proposal is implemented in 23.316 CR21295R6 S2-2412029.