3GPP TSG-SA WG2 Meeting #165	S2-2412242

# Orlando, USA, 18 – 22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5869 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Satellite change for UE-Satellite-UE communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of UE-Satellite-UE communication is concluded in clause 8.3 of TR 23.700-29 and support of setup of UE-Satellite-UE communication is agreed in 5.4.x.2. If satellite changes, the IMS at both side can also determines whethere it is possible to continue UE-Satellite-UE communications based on information from the 5GC. Based on existing Application Function influence on traffic routing negotiation with IMS(acting as AF), SMF can relocate UL/CL and L-PSA UPF on new satellite for the media path to IMS user plane components on new satellites, or remove UL/CL and L-PSA UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Add a new clause to describe the support of Satellite change for UE-Satellite-UE communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Satellite change for UE-Satellite-UE communication is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change(all new) * * * *

5.4.x.3 Satellite change for UE-Satellite-UE communication

When the gNB onboard the satellite serving one of the two UEs involved in a UE-Satellite-UE communication changes (e.g. due to the movement of satellite or UE), the UL CL/BP and L-PSA may be relocated to a target satellite or removed from satellite.

The AMF should report the target satellite identification to SMF as part of the handover procedure in clause 4.9.1 of TS 23.502 [3]. The SMF should determine to relocate the source onboard UL CL/BP and L-PSA to the target satellite or remove the onboard UL CL/BP and L-PSA based on satellites connectivity of the source satellite and target satellite, or based on UE handover to terrestrial network. The SMF may derive whether the two satellites are connected using the satellite identifications received from AMF.

NOTE 1:	How SMF uses the satellite identifications to derive whether the two satellites are connected is not standardized in this release and depends on operators’ policy e.g., by querying transport network system or based on node level information (e.g. Remote GTP-U peer information) reported by the UPF deployed on the source satellite and target satellite which presents that the two satellite are connected or not.

The SMF should report user plane management event with early notification (i.e., change of satellite identifer and potiential change of DNAI associated with UPF onboard the target satellite) and late notification (i.e., DNAI already changed to the one assoicated with UPF on ground) to P-CSCF based on the procedure of notification of user plane management event described in clause 4.3.6.3 of TS 23.502 [3]. If the DNAI reported to IMS is associated with UPF on the ground, it means the UE-Satellite-UE communication cannot continue as the UE has to use the IMS user plane components on ground for service continuity. If the DNAI reported to IMS associated with UPF on target satellite, it is up to the IMS to determine whether the UE-Satellite-UE communication can continue with the peer side UE using the target satellite or not and relocate the IMS user plane components to the target satellite or to the ground accordingly.

NOTE 2: How the satellite identifier of the target satellite is used by IMS to detetermine whether the UE-Satellite-UE can continue is specified in TS 23.228 [15].

To coordinate with IMS for IMS user plane components relocation, the following action is performed by 5GC:

-	For the UE that undergoes satellite change, the following coordination is performed for IMS user plane component relocation:

-	To coordinate the IMS user plane component relocation with the UL CL/BP and L-PSA relocation to a target satellite, the simultaneous change of UL CL/BP, L-PSA procedure in clause 4.3.5.6 or 4.3.5.7 of TS 23.502 [3] is performed at step 3 of Figure 4.3.6.3-1 in TS 23.502 [3].

-	To coordinate the IMS user plane component relocation with the UL CL/BP and L-PSA removal, the procedure of UL CL/BP and L-PSA removal in clause 4.3.5.5 of TS 23.502 [3] is performed at step 3 of Figure 4.3.6.3-1 in TS 23.502 [3].

-	For the other party UE which does not change serving satellite, the following coordination is performed if IMS user plane component is relocated to ground:

-	To coordinate the IMS user plane component relocation to ground with the UL CL/BP and L-PSA removal, based on Application Function influence on traffic routing described in clause 5.6.7 and procedures described in clause 4.3.6 of TS 23.502 [3], upon receiving a Npcf_SMPolicyControl_UpdateNotify message from the PCF that contains a DNAI associated with IMS user plane components on ground network, the SMF determines the DNAI is associated to the PSA UPF on the ground and removes the UL CL/BP and local PSA UPF from satellite.

* * * End of Changes * * * *