3GPP SA WG2 Meeting #166	S2-2412292

# Orlando, US, 18-22 November 2024	               										    (was S2-241xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0520 | Rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the PC5 groupcast transmission of MANET messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed by NIST in S2-2411721 it is currently unclear whether MANET messages are transmitted among U2U Relays using PC5 unicast or PC5 groupcast. The conclusion in 23.700-03 seems to point to the use of PC5 unicast: “Subsequent to a Layer-2 link establishment between a pair of 5G ProSe UEs configured to act as U2U Relays, the two U2U Relays initiate the MANET Neighborhood Discovery Protocol (NHDP) [IETF RFC 6130], followed by the OLSRv2 protocol [IETF RFC 7181]. The support for NHDP [IETF RFC 6130] and OLSRv2 protocol [IETF RFC 7181] is thus mandatory for the 5G ProSe UE configured to act as U2U Relay.” In contrast, the relevant IETF RFCs (RFC 7181 and RFC 6130) seem to point to the use of “lower layer” multicast. From RFC 7181 (OLSRv2): ”OLSRv2 control messages are all sent using link-local multicast.” From RFC 6130 (NHDP): “This protocol makes no assumptions about the underlying link layer, other than support of local broadcast or multicast for communication to 1-hop neighbor routers.” It is proposed to clarify that all MANET messages are transmitted in PC5 groupcast mode, in line with IETF RFC 7181 and IETF RFC 6130. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.14.3.2: Clarified that all MANET messages (HELLO, TC, Discovery Info) are transmitted using Groupcast mode 5G ProSe Direct Communication as defined in clause 5.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of normative text on how MANET messages are transmitted (PC5 unicast vs PC5 groupcast) leading to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.14.3.2	IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

The RSC configuration as defined in defined in 5.1.5.1 indicates if IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication is used. The 5G ProSe Multi-hop UE-to-UE Relays supporting the same RSC establishe IP based Layer-2 links with each other and form a relay cloud to provide IP connectivity for 5G ProSe End UEs connected to any of the 5G ProSe Multi-hop UE-to-UE Relays.

NOTE:	If a 5G ProSe Multi-hop UE-to-UE Relay support multiple RSCs, different Layer-2 links will be established, with each associated with a specific RSC. The 5G ProSe UE-to-UE Relay operates as separate logical entities in the separate relay clouds of different RSCs, i.e. no routing table sharing and no traffic forwarding across different relays clouds of different RSCs.

The 5G ProSe End UE may establish multiple Layer-2 links with different 5G ProSe UE-to-UE Relays supporting the same RSC, in order to maximize the reachability (e.g. in some cases, different 5G ProSe UE-to-UE Relays may serve different disjointed IP subnets).

Mobile Ad-hoc Network (MANET) based routing protocols are used over the IP connectivies in the relay cloud to provide multi-hop forwarding functionality between 5G ProSe End UEs. Each of the 5G ProSe Multi-hop UE-to-UE Relay acts as a MANET router. The exact MANET routing protocols to be supported, e.g. Neighborhood Discovery Protocol (NHDP) [36], OLSRv2 [35], are associated with the RSCs based on configuration.

Based on configuration associated with the RSC, the 5G ProSe End UE obtains the IP address/prefix in two options:

-	Each of the 5G ProSe End UE may be configured with a routable IP address/prefix associated with the RSC. This IP address/prefix does not change when the 5G ProSe End UE changes the connections with 5G ProSe Multi-hop UE-to-UE Relays.

-	If the 5G ProSe End UE does not have an IP address/prefix configured for the RSC, it obtains an IP address/prefix from the 5G ProSe UE-to-UE Relay it connects to. In this case, the 5G ProSe End UE may need to change the IP address/prefix when it changes its connection to another 5G ProSe UE-to-UE Relay.

The 5G ProSe End UE obtains the IP address/prefix of the target 5G ProSe End UE via DNS queries. 5G ProSe Multi-hop UE-to-UE Relay needs to support DNS operations.

When MANET protocol determines the route for IP forwarding, layer-2 link QoS and hop counts can be refelected, e.g. with the Link Metrics value as defined in OLSRv2 [35]. The 5G ProSe Multi-hop UE-to-UE Relay can also enforce the configured hop limit based on the link metrics when building the routing tables, e.g. ignore the route that exceeds the threshold derived from hop limit.

If 5G 5G ProSe Multi-hop UE-to-UE Relay has traffic for its own, it can use any IP address it owns for the transmission.

The 5G ProSe Multi-hop UE-to-UE Relay transmits all MANET messages (i.e. HELLO, TC, Discovery Info) using the Groupcast mode 5G ProSe Direct Communication defined in clause 5.3.3.

NOTE:	The use of “local-link multicast” (which corresponds to the Groupcast mode 5G ProSe Direct Communication defined in clause 5.3.3) is mandated by IETF RFC 6130 [36] and IETF RFC 7181 [35].

End of CHANGEs