3GPP TSG SA WG2 #166	            S2-2412307

18 - 22 November 2024, Orlando, USA 	(revision of S2-2409666)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5634 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Energy Saving per UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusion in TR 23.700-66, the network energy saving per UE should be supported, and the corresponding procedures needs to be enhanced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Network Energy saving per UE operation is described. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy saving and energy efficiency cannot be supported completely. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y.1 (new), 5.X.Z.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

5.X.Y.1	Network Energy Saving per UE

The Network Energy Saving Allowed Indication may be included in the Access and Mobility Subscription data of the UE to indicate whether the network energy saving operation is allowed to be applied for the UE.

5.X.Z.1	Policy control through AMF

When the EIF triggers the network energy saving operations (e.g. changing the network energy saving status) for the UEs through AMF, the AMF checks the subscription data and if the Network Energy Saving Allowed Indication is included. The AMF sends the Network Energy Saving Allowed Indication to NG-RAN and PCF for the UEs as follows:

-	In step 21 of Registration procedure in clause 4.2.2.2.2 of TS 23.502 [3]: The AMF sends the Network Energy Saving Allowed Indication within the N2 message carrying the Registration Accept message.

-	In step 12 of UE Triggered Service Request procedure in clause 4.2.3.2 of TS 23.502 [3]: The AMF sends the Network Energy Saving Allowed Indication within N2 Request.

-	In step 2c of UE Configuration Update procedure for access and mobility management related parameters procedure in clause 4.2.4.2 of TS 23.502 [3]: The AMF sends the Network Energy Saving Allowed Indication to NG-RAN.

-	In step 2 of AM Policy Association Establishment with new Selected PCF procedure in clause 4.16.1.2 of TS 23.502 [3]: The AMF sends the Network Energy Saving Allowed Indication to PCF.

The AMF stores the status network energy saving (e.g. network energy saving operation activated, deactivated) for the UEs. The AMF notifies the change of network energy saving status to PCF to trigger corresponding policy control procedures (e.g. Network Slice Replacement, RFSP index re-allocation).

# * * * * End of changes * * * *