SA WG2 Meeting #S2-166	S2-2411312

18 - 22 November, 2024, Orlando, USA

# Liaison Statement

| Liaison Statement Title: | LS to 3GPP SA1 on support of multiple access technologies based on the IMS service type |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| 5G VoWiFi TF#7a | 30/10/2024 | Conference Call |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| 5G VoWiFi #7a_006r1 | October 2024 | Yan Li, China Telecom |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| GSMA 5G VoWiFi TF | See Below | GSMALiaisons@gsma.com |
| Security Classification | Non-confidential |  |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | GSMA Network Group (NG) |
| External Recipients: | # To: 3GPP SA1 Cc: 3GPP SA2, 3GPP CT1 |

# Background

GSMA 5G VoWiFi Task Force is currently working on new scenarios and features of VoWiFi aspect to improve the QoE of Wi-Fi calling service. One of the use cases to be discussed is the support of multiple access technologies based on the IMS service type.

The types of IMS services nowadays are becoming increasingly diverse, with bunches of data services via IMS Data Channel in addition to audio and video, which may have differentiated requirements on QoS and data transmission. When network conditions change, some user traffic related to dedicated IMS services can be offloaded from 3GPP access to non-3GPP access path, while other types of IMS services remain on the 3GPP access path. For instance, traffic of video and IMS data channel services can be offloaded to the Wi-Fi path, leaving audio traffic on 3GPP access network. Another similar case is also applicable to IMS data channel services (e.g. offloading IMS DC file transfer service to the Wi-Fi path with keeping the IMS DC real-time translation service on the 3GPP access path). Moreover, even traffic from the single same IMS DC service may be split into multiple access paths.

# Details

It is noted that there are couple of requirements related to multiple access technologies listed in 3GPP TS 22.261 clause 6.3.2.1 as follow:

Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type.

When a UE is using two or more access technologies simultaneously, the 5G system shall be able to optimally distribute user traffic over the access technologies in use, taking into account e.g. service, traffic characteristics, radio characteristics, and UE's moving speed.

Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type.

When a UE is using two or more access technologies simultaneously, the 5G system shall be able to optimally distribute user traffic over the access technologies in use, taking into account e.g. service, traffic characteristics, radio characteristics, and UE's moving speed.

GSMA 5G VoWiFi Task Force would like to ask 3GPP SA1 the questions below.

Question 1: For the 1st requirement above, is the traffic type mentioned in the requirement sufficient enough to distinguish different types of IMS service (e.g. audio, video, different IMS DC service, etc.)?

Question 2: For the 2nd requirement above, can the granularity of service type mentioned in the requirement be applied to different types of IMS service or furthermore different types of IMS DC services?

# Requested Action

To 3GPP SA1:

GSMA 5G VoWiFi Task Force kindly requests 3GPP SA1 to provide clarifications to the above questions.

# Contacts

In case of any further questions and/or feedback to this Liaison Statement, please contact GSMALiaisons@gsma.com.

NEXT 5G VoWiFi Task Force MEETING:

5G VoWiFi TF#8    	25th November 2024 Duesseldorf, Germany hosted by Vodafone (hybrid meeting)