3GPP TSG-SA2 Meeting #166	S2-2411390

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5677 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: SCP Event Exposure to detect signaling storm and mitigation actions for signalling storm caused by UE signalling triggered by Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - See considerations in S2-2411388 2. Clause 5.19 describes Control Plane Load Control, Congestion and Overload Control procedures that can be triggered by an AMF or SMF. It should be documented that the newly defined Signalling Storm Analytics can be subscribed by AMF or SMF to decide when to apply those procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Define SCP Event Exposure service to enable SCP to help detecting a signaling storm 2. AMF and SMFs can subscribe at the NWDAF for Signalling Storm Analytics to obtain information about signalling storms caused by excessive UE or NF signalling and can decide to take mitigations actions as described in the Clause 5.19 based on the received analytics information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Missing input data may impede an NWDAF from detecting a signalling storm 2. Missing documented that the newly defined Signalling Storm Analytics can be subscribed by AMF or SMF to decide when to apply Control Plane Load Control, Congestion and Overload Control procedures in Clause 5.19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.19.1, 6.2.6.3, 6.2.19, 6.3.16, new 7.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 5.19.1	General

In order to ensure that the network functions within 5G System are operating under nominal capacity for providing connectivity and necessary services to the UE. Thus, it supports various measures to guard itself under various operating conditions (e.g. peak operating hour, extreme situations). It includes support for load (re-)balancing, overload control and NAS level congestion control. A 5GC NF is considered to be in overload when it is operating over its nominal capacity resulting in diminished performance (including impacts to handling of incoming and outgoing traffic).

AMF and SMFs may subscribe at the NWDAF for Signalling Storm Analytics as specified in TS 23.288 [86] to obtain information related to signalling storms caused by excessive UE or NF signalling and may decide to take mitigations actions as described in the present Clause 5.19 based on the received analytics information.

2nd change

#### 6.2.6.3	SCP profile

SCP profile maintained in an NRF includes the following information:

-	SCP ID.

-	FQDN or IP address of SCP.

-	Indication that the profile is of an SCP (e.g. NF type parameter set to type SCP).

-	SCP capacity information.

-	SCP load information.

-	SCP priority.

-	Location information for the SCP (see locality in clause 6.1.6.2.2 of TS 29.510 [58]).

-	Served Location(s) (see servingScope in clause 6.1.6.2.2 of TS 29.510 [58]).

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	Remote PLMNs reachable through SCP.

-	Endpoint addresses accessible via the SCP.

-	NF sets of NFs served by the SCP.

-	SCP Domain the SCP belongs to. If an SCP belongs to more than one SCP Domain, the SCP will be able bridge these domains, i.e. sending messages between these domains.

-	If the SCP acts as service producer: Names of supported services, NF Service Set ID of the NF service instance, Service authorization information

NOTE:	Service definition defines optional and mandatory parameters, see TS 23.502 [3].

3rd change

### 6.2.19	SCP

The Service Communication Proxy (SCP) includes one or more of the following functionalities. Some or all of the SCP functionalities may be supported in a single instance of an SCP:

-	Indirect Communication (see clause 7.1.1 for details).

-	Delegated Discovery (see clauses 7.1.1 and 6.3.1 for details).

-	Message forwarding and routing to destination NF/NF service.

-	Message forwarding and routing to a next hop SCP.

-	Communication security (e.g. authorization of the NF Service Consumer to access the NF Service Producer API), load balancing, monitoring, overload control, etc.

-	Optionally interact with UDR, to resolve the UDM Group ID/UDR Group ID/AUSF Group ID/PCF Group ID/CHF Group ID/HSS Group ID based on UE identity, e.g. SUPI or IMPI/IMPU (see clause 6.3.1 for details).

-	Optinally expose events related to service-agnostic characteristics of messages the SCP passes.

NOTE 1:	Communication security, e.g. authorization of the NF Service Consumer to access the NF Service Producer's API is specified in TS 33.501 [29].

NOTE 2:	Load balancing, monitoring, overload control functionality provided by the SCP is left up to implementation.

The SCP may be deployed in a distributed manner.

NOTE 3:	More than one SCP can be present in the communication path between NF Services.

SCPs can be deployed at PLMN level, shared-slice level and slice-specific level. It is left to operator deployment to ensure that SCPs can communicate with relevant NRFs.

In order to enable SCPs to route messages through several SCPs (i.e. next SCP hop discovery, see clause 6.3.16), an SCP may register its profile in the NRF. Alternatively, local configuration may be used.

4th change

### 6.3.16	SCP discovery and selection

An NF is configured with its serving SCP(s).

In a deployment where several SCPs are deployed, a message may traverse several SCP instances until reaching its final destination. A SCP may discover and select a next hop SCP by querying the Nnrf_NFDiscovery Service of the NRF or it may be configured with next SCP in the message path.

An SCP may use the SCP profile parameters in clause 6.2.6.3 as discovery parameters in Nnrf_NFDiscovery. The parameter(s) to be used depend(s) on network deployment. The NRF returns a list SCP Profiles as per the provided discovery parameters.

If an SCP receives a Routing Binding Indication within a service or notification request and decides to forward that request to a next-hop SCP, it shall include the Routing Binding Indication in the forwarded request.

NOTE:	It is up to SCP implementation, deployment specific configuration and operator policies as to how the SCP will use information retrieved from the NRF to resolve the optimal route to a producer.

Based on SCP configuration, an SCP deciding to address a next-hop SCP for a service request may then delegate the NF (instance) and/or service (instance) selection to subsequent SCPs and provide discovery and selection parameters to the next-hop SCP.

For the discovery of an SCP acting as NF service producer for the services listed in Clause 7.2.X, the procedures in Clause 6.3.1 apply.

5th change

### 7.2.X	SCP Services

The following NF service is specified for SCP.

Table 7.2.29-1: NF Services provided by UPF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nscp_EventExposure | This service exposes SCP related information to the consumer NFs. | 5.2.X.2 |

End of changes