3GPP TSG SA WG2#166	            S2-2411474

Orlando, USA, 18-22 November 2024 	(revision of S2-24XXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5826 | Rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for AF initiated Slice replacement request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduces the ability for an AF to trigger a slice replacement within a set of subscribed network slices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the ability to trigger slice change by an AF for a specific subscriber. The AF can initiate the request directly or via the NEF. In either case the request goes to the PCF for the UE. The CF requests the AMF tto terminate usage of the current slice and use the Alternative slice instead. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of the support of AF-requested slice change in the 5GS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.5.2.2a (new Clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The DUMMY WI code needs to be replaced by the TEI19_XX WI on "New WID on Network Controlled Network Slice Selection", once allocated after the WI approval |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.15.5.2.2a	AF triggered modification of the Set of Network Slice(s) for a UE

In non-roaming cases, a trusted AF may request to replace a certain S-NSSAI (Replaced S-NSSAI) for a UE with another S-NSSAI (Alternative S-NSSAI) which is part of the UE subscription. Alternatively, an untrusted AF via the NEF, may request a change of network service (which is mapped at NEF to an Alternative S-NSSAI which is part of the UE subscription). In either case , the request is sent to the PCF for the UE handling the impacted subscriber. Similarly, a trusted AF or an AF, subject to authorization via the NEF, may request to terminate Network Slice replacement.

Editor’s Note: The exact nature of network service that maps to Alternative S-NSSAI is FFS.

The PCF for the UE triggers the network slice replacement at the AMF as defined in TS 23.503 [45].

NOTE 1: 	How the AF obtains the S-NSSAI to be replaced and the S-NSSAI replacing is out of scope of this specification.

The PCF sends access and mobility related policies to the AMF, as described in TS 23.503 [45], including the Replaced S-NSSAI and an Alternative S-NSSAI to trigger the Network Slice replacement, together with an indication that instructs the AMF to terminate all PDU sessions associated with the Replaced S-NSSAI and start using the Alternative S-NSSAI. To that effect, the AMF performs the following:

-	If the Alternative S-NSSAI is not within the UE subscription, or is not supported in the UE Registration Area, the AMF notifies the PCF that the Network Slice Replacement is not triggered with a cause value. The PCF further informs the AF.

-	If the Alternative S-NSSAI is within the UE subscription and is supported in the UE Registration Area, the AMF includes the Alternative S-NSSAI in the Allowed NSSAI (if not present) and removes the Replaced S-NSSAI from the Allowed NSSAI and the Configured NSSAI. The PCF notifies the AF about the outcome of the network slice replacement.

NOTE 2: 	It is assumed that the URSP rule is preconfigured to ensure that the UE will use the Alternative S-NSSAI to establish a new PDU Session. Otherwise the PCF sends the UE updated URSP rules.

When the trusted AF or a NEF sends request to PCF for the UE to terminate the Network Slice Replacement, the PCF for the UE sends access and mobility related policies to the AMF, as described in TS 23.503 [45], including the Replaced S-NSSAI and an Alternative S-NSSAI reversed from the  initial procedure, together with an indication that instructs the AMF to perform same actions as in the initial procedure on the reversed S-NSSAIs to restore the original S-NSSAI.

End of CHANGES